-----------------------------------------------------------------------

cFos Modem Command Set

-----------------------------------------------------------------------

?       Show a small help screen of AT Commands and important

S-registers.

A       Force Answer

Bn      Select protocol ONLY for outgoing calls

0       X75 without B3 protocoll, 64000bps      ("ISDNC")

s22=2048, s26=2, s28=2048

1       V110 async, 38400bps, 8n1               ("ISDNB")

s22=2048, s26=2, s28=2048

2       V110 async, 19200bps, 8n1               ("ISDNA")

s22=2048, s26=2, s28=2048

3       TELES 128kbps Bundle Protocoll

s22=2048, s26=2, s28=2048

4       X75, Framelen=256, Win.Size=7 (ELINK mode)

s22=framelen, s26=7, s28=256

5       Datex-J (BTX), Framesize=130, Win.Size=7

s22=framelen, s26=7, s28=128

'framelen' refers to the framelen set by -b switch on

commandline; all values are checked not to exceed the

commandline given settings

D..     Dial

L       dial last number

S       dial stored number (hardcoded, 0=Zaphods BBS)

V       flag connection as SPV

0..9    digits

A..D#*  special "digits"

En      Command Echo

0       echo off

1       echo on

Hn      On/Off Hook control

0       on hook, terminate an active connection

1       off hook, reject an incoming call

In      Inquiry

0       product identification and CAPI version and

manufacturer

1       status line output

2       last call statistics

4       last 10 CAPI info dumps

Qn      Quiet mode, result codes display

0       display result codes

1       suppress result code; be Quiet

Sr=n    Set register r to value n (registers are unsigned 16bit ints)

Sr.b=n  Set bit b to n

Sr,b=n  Set bit b to n

Sr?     Query register r

SrH=n   Set high byte of register r to value n

SrL=n   Set low byte of register r to value n

Vn      Verbal mode

0       numeric result codes

1       verbal result codes

Xn      Result code options

0       modem looks like a standard 9600 bps modem

1       enables CONNECT <rate>

2       enables RINGING

3       enables RING <incoming caller's id>

4       enables CONNECT[...]/<protocols..>

5       enables ?

6       enables /<cause> on ERROR, NO CARRIER, NO ANSWER

and NO DIALTONE

Zn      Restore profile from number n (out of the file PROFILE).

This will further reregister the application at the CAPI.

If no number is given, 0 will be default.

Fn      \

Kn       |

Ln       |

Mn       |  ignored for compatibility

Nn       |

O        |

P        |

T       /

-----------------------------------------------------------------------

cFos extended Modem Command Set

-----------------------------------------------------------------------

{+}     &Bn     Number of Channels to use for 'cFos' Channel Bundling, for

next dial command

&Cn     Ignored for compatibility

&Dn     Status line

0       disable status line (default)

1       enable status line (only if port is initialized)

2       enable status line (only if carrier is high)

&En     EAZ: n = '0' ... '9': changes register 18. Default is

EAZ 0.

&F      Reset to Factory defaults and reset (like ATZ)

&H<str> This sets the hangup string to "+++<str><cr>". E.g. this

enables setting the hangup  sequence  to  "+++ATHE1<cr>"

for the pcANYWHERE "Hayes compatible modem". Default  is

<str>   =   "ATH0",   i.e.   the   hangup-sequence    is

"+++ATH0<cr>".

&Lnnn   Listen to what EAZ's; changes register 13; takes multiple

digits or '*' for all. Default is EAZ 1.

&Onnn   Outdial Number. This number will be used as prefix for

all phone numbers dialed with the ATD command. Default is

none.

&Rn     Reregister the application if all ports are idle; use

#R1 to force reregister (and lockout of all active

connections!)

&T      TRANX: set the clock to the actual time/date (gotten from

the last connection attempt).

&V      View profile

&Wn     Write profile to number n (into a file named PROFILE). If

no number is given, it defaults to 0.

-----------------------------------------------------------------------

cFos Modem Register Chart

-----------------------------------------------------------------------

Reg#      Description                                     Default

S0      Auto Answer on (>=1) / off (==0)                0

S1      Ring count                                      0

S2

S3

S4

S5

S6

S7      Number of seconds modem waits for a carrier     60

S8

S9      Bit-mapped register:                            32737

.0      enable any further result code

.1      enable CONNECT <rate>, otherwise

CONNECT 9600 is always returned

.2      enable RINGING

.3      enable RING <number>

.4      enable /X75.. after CONNECT

.5      enable ERROR/<cause>

.6      enable NO CARRIER/<cause>

S10     Bit-mapped register:                            Default: 0

.0      enable CONNECT.../ID=<callerid>

.1      disable full status line, just display the callerid

on the left side

.2      display numerical values fuer service indicator /

add. si instead of names

.3      display the callerid, requested eaz, si/add si on

a RING like RING 1234/EAZ1/07/00, in spite of the

default method: RING<cr>ID=1234/EAZ1/07/00

.4      enable +++ATH0<CR> hangup sequence in BIOS emulator

mode (and disable hangup on baudrate change)

.5      enable CallerID zero padding

.6      disable multiple CallerID's

S11     Status line line number (0 - screen length-1)   0

Note: only displayed if AT&D1 or AT&D2

S12     Nummer of 1/18 sec. to wait before CONNECT      50

message.

S13     Serviced EAZ Mask                               2

Every bit from bit0.. bit9 enables RING on

this EAZ ('0' - '9')

ex:     0000000010b     answer on '1'

0100000000b     answer on '8'

0001111111b     answer on '0' - '6'

1111111111b     answer all calls

This register is also modified by the AT&L... command.

S14     Serviced SI Mask                                0x80

Every bit enables a service 'cFos' listens to

Bit 0    videophone

Bit 1    telephone

Bit 2    a/b-services

Bit 3    X.21-services

Bit 4    telefax (Group 4)

Bit 5    BTX (64 kbits/s)

Bit 7    data transmission (64 kbits/s)

Bit 8    X.25-services

Bit 9    teletex 64

Bit 10   mixed mode

Bit 15   BTX (new standard)

S15     B-Channel Identification (ffs. 1.TR.6,          0x83

3.2.3.4.4.7)

S16     Source Service Indicator                        7

S17     Source Additional Service Indicator             0

S18     Source EAZ (character: '0'=48.. '9'=57)         '0'

Sent with connect_req message; maybe this has

to be changed on PBX systems.

This register is also modified by an AT&En

command.

S19     Inactivity timer: modem disconnects if          0

no traffic in N minutes; 0==disabled

S20     B-Channel, level 2 protocol                     1

1       X.75 SLP Basis Operation Mode,

mit Implementation Rules nach T.90

2       Transparent-HDLC mit Bit-Stuffing,

Frame-Erkennung und CRC-Check

3       Bittransparent

4       SNA-SDLC

5       X.75 Btx

8       V110 mit transparentem B2 protokoll

9       V110 SNA-SDLC

10      V110 X75 SLP nach T.90

S21     B-Channel, level 3 protocol                     4

1       T70 NL fr Leitungsvermittlung (CSPDN)

2       ISO 8208 (DTE/DTE)

3       Level 3 according to T.90, Appendix II

4       transparent

S22     SELECT_B2_PROTOCOL_REQ: Data-length             2048

S23     SELECT_B2_PROTOCOL_REQ: Link-Address-A          3

S24     SELECT_B2_PROTOCOL_REQ: Link-Address-B          1

S25     SELECT_B2_PROTOCOL_REQ: Modulo-Mode             8

S26     SELECT_B2_PROTOCOL_REQ: Window-Size             2

S27     SELECT_B2_PROTOCOL_REQ: V110 user rate          64

S28     Maximum SEND B2-Framesize                       2048

S29     B3-Protocol-Overhead in Bytes                   0

b3 == 1 (t70nl): 2

b3 == 2 (iso8208): 3

b3 == 4 (transparent): 0

S30     SELECT_B3_PROTOCOL_REQ: LIC                     0

S31     SELECT_B3_PROTOCOL_REQ: HIC                     0

S32     SELECT_B3_PROTOCOL_REQ: LTC                     0

S33     SELECT_B3_PROTOCOL_REQ: HTC                     0

S34     SELECT_B3_PROTOCOL_REQ: LOC                     0

S35     SELECT_B3_PROTOCOL_REQ: HOC                     0

S36     SELECT_B3_PROTOCOL_REQ: Modulo-Mode             8

S37

S38

S39     Numbering Type/Plan                             0x80

S40     Controller                                      0

S41     Info-Mask-low                                   63

S42     Info-Mask-high                                  0

S43     Default V110 user rate & Protocol               64

S44

S45

S46

S47

S48

{+}     S49   number of digits of caller id to ignore           0

(from right to left) when called with CCB

S50    \

S51     |

S52     |   Default Additional Service Indicator and B2-Protocol

S53     |   for every EAZ:

S54     |   If the incoming AddSI is 0 then we get the AddSI

S55     |   from the low byte of register 50+EAZ; the B2-Protocol

S56     |   is got from the high byte.

S57     |

S58     |

S59    /

S60   charge limit, main port     \

S61   charge limit, aux. port      |

S62   time limit, main port (secs) |

S63   time limit, aux. port (secs) |

S64   lower cps limit              | see CFOS.DOC, chapter 12

S65   upper cps limit              |

S66   charge simulation timer      |

S67   pre-off disconnect time      |

S68   connect tries                |

S69   delay between dyn.CCB tries /

-----------------------------------------------------------------------

cFos Numerical Result Codes

-----------------------------------------------------------------------

0     OK

1     CONNECT

2     RING

3     NO CARRIER

4     ERROR

5     ( not used )

6     NO DIALTONE

7     BUSY

8     NO ANSWER

9     ( not used )

10     ( not used )

11     RINGING