Functions reference manual for the X00 developer

X.00 A Low Level Serial I/O Communications Driver

for MS DOS and like Operating Systems.

Copyright (C) 1994 by Raymond L. Gwinn

12469 Cavalier Drive

Woodbridge, Virginia 22192

All Rights Reserved

Januar 01, 1994

Fidonet address 1:265/1

Data 703-494-8331

i

INTRODUCTION

Previously,  the documentation  for  X00 was  little more  than a

collection of notes.  The notes were added to as  the development

process continued and enhancements were made.  I honestly did not

suspect that  X00 would  become as  widely used as  it is.   This

manual is an attempt at some decent documentation for X00.

But who do  I document for?   The user that  is setting up  a BBS

just wants  to get X00 installed and to move on to more important

things.  The user that is having problems needs information about

serial  I/O communications  in general  and the  PC specifically.

The application developer (programmer) needs detailed information

about each individual function and the information generated.

This manual is  intended as  a reference for  use by  application

programmers who desires to use X00.   A separate Users manual  is

included in the distribution file(s).

This manual and the software distributed with it is provided with

no guarantees.  Use at your own risk.

ii

WHAT IS X00 . . . . . . . . . . . . . . . . . . . . . . . . .   1

Warranty  . . . . . . . . . . . . . . . . . . . . . . . . . .   1

Licensing Information . . . . . . . . . . . . . . . . . . . .   2

Non-Commercial License . . . . . . . . . . . . . . . . .   2

Commercial License . . . . . . . . . . . . . . . . . . .   2

Functions . . . . . . . . . . . . . . . . . . . . . . . . . .   4

Function 00h - Set communications parameters . . . . . .   5

Function 01h - Transmit character and wait . . . . . . .   6

Function 02h - Get received character with wait  . . . .   6

Function 03h - Return Serial Port Status . . . . . . . .   7

Function 04h - Activate Port . . . . . . . . . . . . . .   8

Function 05h - Deactivate Port . . . . . . . . . . . . .   9

Function 06h - Raise/lower DTR . . . . . . . . . . . . .   9

Function 07h - Return timer tick information . . . . .    10

Function 08h - Flush output buffer . . . . . . . . . .    10

Function 09h - Purge output buffer . . . . . . . . . .    10

Function 0Ah - Purge input buffer  . . . . . . . . . .    11

Function 0Bh - Transmit no wait  . . . . . . . . . . .    11

Function 0Ch - Non-destructive read-ahead (Peek) . . .    11

Function 0Dh - Keyboard read without wait  . . . . . .    12

Function 0Eh - Keyboard read with wait . . . . . . . .    12

Function 0Fh - Flow Control for serial I/O . . . . . .    13

Xon/Xoff flow control . . . . . . . . . . . . . .    13

RTS/CTS flow control  . . . . . . . . . . . . . .    13

Function 10h - Control-C / Control-K checking  . . . .    14

Function 11h - Set current cursor location . . . . . .    14

Function 12h - Read current cursor location  . . . . .    15

Function 13h - Single character ANSI write to screen .    15

Function 14h - Enable or disable the DCD watchdog  . .    15

Function 15h - Write character to screen using BIOS  .    16

Function 16h - Add  or delete a routine from  the timer

tick  . . . . . . . . . . . . . . . . . . . . . .    16

Function 17h - Reboot system . . . . . . . . . . . . .    17

Function 18h - Block Read  . . . . . . . . . . . . . .    17

Function 19h - Block Write . . . . . . . . . . . . . .    17

Function 1Ah - Break begin or end  . . . . . . . . . .    18

Function 1Bh - Return information about X00  . . . . .    19

Superset Functions  . . . . . . . . . . . . . . . . . . . .    20

Function 1Ch - Activate Port . . . . . . . . . . . . .    21

Function 1Dh - Deactivate Port . . . . . . . . . . . .    22

Function 1Eh - Extended line control initialization  .    23

Function 1Fh - Extended serial port status/control . .    24

Function 20h - Read with no wait (destructive) . . . .    25

Function 21h - Stuff/Poke the receive buffer . . . . .    25

iii

"Layered Application" services  . . . . . . . . . . . . . .    26

Function  7Eh  -   Install  an  "external  application"

function  . . . . . . . . . . . . . . . . . . . .    26

Function  7Fh  -   Remove  an  "external   application"

function  . . . . . . . . . . . . . . . . . . . .    27

Application Notes . . . . . . . . . . . . . . . . . . . . .    28

Which Activate/Deactivate functions to use . . . . . .    28

Watch out for disk I/O . . . . . . . . . . . . . . . .    28

Successive timer tick calls  . . . . . . . . . . . . .    29

1

WHAT IS X00

X00 is  a Serial  Input/Output (SIO) communications  driver.   It

provides  an interface  between  an application  program and  the

serial communications hardware.

X00 was originally  designed as a  FOSSIL driver.   FOSSIL is  an

acronym  containing the first character of several communications

programs.  The FOSSIL specification  is basically an extension of

the PC's BIOS services for serial communications.

Except as  noted in the this Manual, X00 meets the FOSSIL level 5

specification.   X00 provides  additional functions that  are not

contained in the FOSSIL level 5 specification.

X00 only works with 8250 (TYPE) serial I/O devices.  Such devices

include the  8250A, 16450, 16550, 16550A  and the 82510.   If you

have a PC that  is an IBM or near  compatible which has a  serial

communication  port, it is likely  that it contains  one of these

devices.  XU, a  utility distributed with X00, will  identify the

existence of compatible SIO devices.

Like  other Device  Drivers,  X00 will  do  very little  standing

alone.  There must be an application(s) program to use X00 before

you will gain benefit.

Since  the introduction  of the  FOSSILs, a  great number  of new

communications  programs  have come  into existence.   I  like to

think  that the FOSSILs are  largely responsible for  this.  Many

programs that contain their own communications drivers have added

FOSSIL support.  One notable example is RBBS.

Many  programs  written  to   use  BIOS's  serial  communications

services will also work with X00.   This gives those programs the

benefits of buffered interrupt driven I/O.

Warranty

There is none.  Use X00 your own risk.

2

Licensing Information

Non-Commercial License:

A  non-commercial license  for  use of  X00  is included  in  the

distribution file called LICENSE.TXT.

I  have received  several unsolicited  checks from users  of X00.

All  of the checks  have been returned  or uncashed.   To date, I

have not accepted any payments for X00.  If you feel that  X00 is

useful  enough that some sort  of payment is  justified then read

on.

Our  local (public)  High School  desperately needs  funds.   The

school officials work at keeping the students off the streets and

away from  drugs.   The local School  Board is unable  to provide

needed  funds for  non-academic activities.   I suggest  a $10.00

contribution be sent to:

Woodbridge Senior High School

3001 Old Bridge Road

Woodbridge, Virginia, 22192

You  can make the  check payable to  "WSHS, Ballfield Improvement

Fund".   Please make a  note "X00 contribution"  somewhere on the

check.

Many of you have used X00 for years.  I believe  $10.00 is easily

affordable to most X00 users.   Please take the time to sit down,

make out the check, address the envelope and mail a contribution.

Shareware authors that wish to  distribute X00 with their product

should contact me.  Permission will usually be granted.

Commercial License:

Commercial Licenses will  be liberally  granted in  return for  a

donation (usually tax deductible).   Each commercial license will

be individually granted.  That is, there is no single policy that

can be stated  here.  Some  generally uniform commercial  license

policies follow.

1 - X00 is usually licensed by product.

2 - All  commercial licensees of X00 will receive  a special copy

of X00 that will identify, at boot time, the commercial product.

3 -  I may require a fully licensed, fully featured, copy of your

product for personal use on multiple computers.

3

4 - The required  contribution, which is never to  me personally,

will usually be a one time contribution of $100.00.

5 -  Proof of licensing belongs to you.   That is, I am not going

to be responsible for keeping records.  I suggest that you keep a

copy of the donation  check so that you can  send me a copy  if a

question  arises.    Otherwise  you  may  have  to  make  another

contribution.

6  - You may  only distribute versions  of X00 that  contain your

product name in the X00 banner (at boot time).

7 - If you wish  to have the latest version of  X00 automatically

sent to you with your product name in the X00 banner, then do the

following:  Send me a check  for $10.00, in advance, along with a

pre-addressed mailing label.  If you require a 3.5 inch diskette,

then write "3.5 inch" on the pre-addressed mailing label and send

$15.00 (instead of  $10.00).   For updates, the  check should  be

made  out to Raymond  L. Gwinn.   The checks will  be cashed when

received.  Individuals that wish to receive a mailed  copy of the

latest X00 version may  also use this  procedure.  Once you  have

received an  update, you will need  to resend the  update fee and

pre-addressed mailing  label if  you desire  to receive the  next

update.

8  -   Your  contribution  check  should   state  "X00  Licensing

contribution" in the Memo or comment field.

If  you write  me about  a commercial  license, please  include a

voice phone number.  You may  not receive a response if the voice

phone number is omitted.  Correspondence should be sent to:

Raymond L. Gwinn

12469 Cavalier Dr.

Woodbridge, Va. 22192

Prior to making the special version of X00 available, I  may wait

for  the check to clear the bank.   If the check bounces, I will,

most  likely, not give any further  consideration to a commercial

license.

4

Functions

X00  functions are  designed to  be  called by  assembly language

programs.    Some  rudimentary  High Level  access  routines  are

provided  in  the  X00  distribution files.    These  rudimentary

routines are the so called HLLAPI files.

Parameters for all  X00 functions  are passed in  the basic  808x

registers.    In  most cases  the  results  of  the function  are

returned  in registers.    If  no  parameter  is  returned  in  a

register,  X00 will return the register  unchanged except for the

AX register.   If no information is returned in  the AX register,

then its contents upon return from a function is undefined.

Many  (most) of the X00 functions require a port number be passed

in  the DX register.  Port numbers start at 0 (zero).  Port 0 and

COM1  are normally the same.  However, this relationship must not

be assumed.  It is possible for the X00 user to re-map the  ports

on their  systems.  Please  refer to  the Users  Manual for  more

information.

I  have  noted that  some  application programs  are  calling X00

functions from  the timer tick.   This should be done  with care.

If  the X00 DV option is enabled  at load time, X00 could issue a

DESQview Pause command as a result  of your call.  I am  not sure

what  the results of this would be, but it points out the type of

consideration that  should be done before  issuing function calls

from the timer tick.

5

Function 00h - Set communications parameters, baud, parity etc.

Input:    AH = 00h

AL = Communications parameters (defined below)

DX = Port number

Output:   AX = Port status (see function 03h)

This  function is identical to  the IBM PC  BIOS call except that

110 baud and 150 baud have been replaced by 19200  baud and 38400

baud respectively.  The high order  3 bits of AL specify the baud

rate  as  follows:   See functions  1Eh and  1Fh for  an enhanced

version of this function.

010 =   300 baud

011 =   600  ''

100 =  1200  ''

101 =  2400  ''

110 =  4800  ''

111 =  9600  ''

000 = 19200  '' (Replaces IBM 110 baud setting)

001 = 38400  '' (Replaces IBM 150 baud setting)

The low order 5 of AL specify the following:

Bits 4-3 define parity:       0 0  no parity

1 0  no parity

0 1  odd parity

1 1  even parity

Bit 2 defines stop bits:      0    1 stop bit;

1    1.5 bits for 5-bit codes,

2 for others

Bits 1-0 character length:    0 0  5 bits

0 1  6 bits

1 0  7 bits

1 1  8 bits

6

Function 01h - Transmit character and wait.

Input:    AH = 01h

AL = Character

DX = Port number

Output:   AX = Port status (see function 03h)

Upon  entry to this function, AL must contain a character that is

to be transmitted.   If there is room in the transmit buffer, the

character  buffered.   Otherwise, this  function will  loop until

their is  room in the buffer or until  a timeout occurs.  If set,

the high  order bit of  the returned status indicates  a time out

occurred.   See function 03h.   At the time of  this writing, the

timeout is set to 30 seconds.

Note:  X00 executes functions with  the interrupt mask  set as it

was  when entered.   That is,  if X00  is called  will interrupts

masked, interrupts will  remain masked during  the processing  of

the function.  If this function is called with interrupts masked,

it may never return.

-----------------------------------------------------------------

Function 02h - Get received character with wait.

Receive character with wait

Input:    AH = 02h

DX = Port number

Output:   AH = Line status (same AH returned by function 3)

AL = Input character

If a  character available  in the  receive buffer, this  function

returns the next character from the  receive buffer in AL.  If no

receive character is available, this  function will wait until  a

character is received or  until a timeout occurs.   If a  timeout

occurs,  the high order bit of  AH will be set  upon return.  See

function 03h.   At the time of this writing, the timeout value is

set to 30 seconds.

Note:  X00 executes functions with  the interrupt mask  set as it

was when  entered.   That is,  if X00  is called  will interrupts

masked, interrupts will  remain masked during  the processing  of

the function.  If this function is called with interrupts masked,

it may never return.

Function 20h is a get character with no wait.

7

Function 03h - Return Serial Port Status.

Input:    AH = 03h

DX = Port number

Output:   AX = Status bits as follows:

AH = Line Status, where:

Bit 0 = RDA  - input data is available in buffer

Bit 1 = OVRN - the input buffer has been overrun

Bit 2 = Reserved (Parity error in BIOS INT 14h)

Bit 3 = Reserved (Framing error in BIOS INT 14h)

Bit 4 = Reserved (Break detect in BIOS INT 14h)

Bit 5 = THRE - room is available in output buffer

Bit 6 = TSRE - output buffer is empty

Bit 7 = Timeout (set by functions 1 and 2 only)

Bit 7 of AH IS ALWAYS ZERO WHEN RETURNED BY FUNCTION 3.  Bit 7 of

AH may be  set to one by other functions (like 1 and 2) that also

return status.  When bit 7 of AH is set to one, none of the other

15 bits returned in AX are valid.

AL = Modem status, where:

Bit 0 = Delta clear to send (not reliable)

Bit 1 = Delta data set ready (not reliable)

Bit 2 = Delta data carrier detect (not reliable)

Bit 3 = Always set to 1 upon return (DUMMY DCD)

Bit 4 = Clear to send (CTS)

Bit 5 = Data set ready (DSR)

Bit 6 = Ring indicator (RI)

Bit 7 = Data carrier detect (DCD)

Bit  3  of AL  is  always  returned  set  to  1.    This  enables

application programs  to use  bit 3  as a  carrier detect bit  on

hardwired (null modem) links.

8

Function 04h - Activate Port.

Input:    AH = 04h

DX = port number

Output:   AX = 1954h if successful

BL   =  maximum  function  number  supported  (not

counting functions 7Eh and above)

BH = rev of FOSSIL specification supported

This function instructs X00  to use the FOSSIL specification  for

the specified port.  Prior to issuing this function, any function

that  requires a port number  in DX be will  passed on to BIOS or

X00's  INT 14h BIOS emulator.  When this function  is called, all

interrupts   involved  in  supporting   the  communications  port

(specified in DX) are  set up for support by X00.   DTR is turned

on by this function.  The baud rate is NOT set or changed by this

function.

If an additional call  to this function occurs (2  Inits or Init,

Function nn, Init, etc.)  X00 will reset (clear) all  buffers and

all flow control is turned off.  Values returned in the registers

upon a re-init are the same as an initial init (defined above).

NOTICE: At a future date (July 1991 or later), the author intends

to  make  this  function  identical to  the  PS/2's  function  4.

Function 1Ch is a  duplicate of this function.   New or  modified

application programs should discontinue  use of this function and

use the duplicate (1Ch) function.

A section is the  application notes describes a simple  method of

determining  which Activate/Deactivate  function numbers  to use.

The described method should work on all FOSSIL implementations.

9

Function 05h - Deactivate Port.

Input:    AH = 05h

DX = Port number

Output:   Nothing

This function  instructs X00  that it  should  no longer  process

calls for specified port (in  DX) using the FOSSIL specification.

If the port was never activated by function 4, then this function

is ignored.  Any  subsequent function calls, that require  a port

number  in  DX, will  be  passed  to BIOS  or  the  BIOS INT  14h

emulator.

NOTICE: At a future date (July 1991 or later), the author intends

to  make  this  function  identical  to  the  PS/2's  function 5.

Function 1Dh is  a duplicate of this  function.  New  or modified

application programs should discontinue use of this function  and

use the duplicate (1Dh) function.

A section is the  application notes describes a simple  method of

determining  which Activate/Deactivate  function numbers  to use.

The described method should work on all FOSSIL implementations.

-----------------------------------------------------------------

Function 06h - Raise/lower DTR.

Input:    AH = 06h

AL = DTR state to set

DX = Port number

Output:   Nothing

This function is used to control the DTR signal.   AL = 00h means

lower DTR (turn it off), and AL = 01h means to raise DTR (turn it

on).  No other function (except functions 04h and 1Ch) will alter

DTR.

10

Function 07h - Return timer tick information.

Input:    AH = 07h

Output:   AL = Timer tick interrupt number

AH = Approximate interrupts per second (18 on IBM)

DX = Approximate number of milliseconds per tick

This  function  is used  to determine  the characteristic  of the

timer tick for the computer.

Application  programs  can  use  information  returned  by   this

function to dynamically  set up  code that is  executed on  every

timer tick.

-----------------------------------------------------------------

Function 08h - Flush output buffer.

Input:    AH = 08h

DX = Port number

Output:   Nothing

This  function  is  used to  wait  until  any  pending output  is

transmitted.   It does not  return until all  buffered output has

been sent.  This function should be used with care.  Flow control

(documented  below) can  cause your  system hang  in a  tight un-

interruptable loop given the right circumstances.

Note:  X00 executes functions with  the interrupt mask  set as it

was when  entered.   That is,  if X00  is called will  interrupts

masked, interrupts  will remain  masked during the  processing of

the function.  If this function is called with interrupts masked,

it may never return.

-----------------------------------------------------------------

Function 09h - Purge output buffer.

Input:    AH = 09h

DX = Port number

Output:   Nothing

This function  is used to  remove any buffered  output (transmit)

data.   Any  output  data remaining  in  the output  buffer  (not

transmitted yet) is discarded.

11

Function 0Ah - Purge input buffer.

Input:    AH = 0Ah

DX = Port number

Output:   Nothing

This function is  used to  remove any  buffered input  (received)

data.    Any  input data  which  is  in  the  receive  buffer  is

discarded.

-----------------------------------------------------------------

Function 0Bh - Transmit no wait.

Input:    AH = 0Bh

AL = Character

DX = Port number

Output:   AX = 0001h - Character was buffered for xmit

AX = 0000h - Character was not buffered

This is exactly  the same as  function 01h, except  that it  will

always  return immediately.    If X00  is  unable to  buffer  the

character  (the transmit  buffer is  full), a  value of  0000h is

returned in AX. If X00 accepts the character (room is available),

0001h is returned in AX.

-----------------------------------------------------------------

Function 0Ch - Non-destructive read-ahead (Peek).

Input:    AH = 0Ch

DX = Port number

Output:   AH = 00h if character is available

AL = Next character if available

AX = 0FFFFh = If no character is available

This function returns the next character from the receive buffer.

If the  receive buffer is empty,  0FFFFh is returned in  AX.  The

character   returned  remains   in  the   receive   buffer.  Some

programmers call this function "peek".

Function 20h is a destructive read with no wait.

12

Function 0Dh - Keyboard read without wait.

Input:    AH = 0Dh

Output:   AX = IBM-style scan code if available

= 0FFFFh if no character not available

This  function  returns  the  keyboard  scan  code  of  the  next

character from  the keyboard.   This is a  non-destructive (peek)

read of  the keyboard.  If no keyboard data is available,  0FFFFh

is returned in AX.

-----------------------------------------------------------------

Function 0Eh - Keyboard read with wait.

Input:    AH = 0Eh

Output:   AX = Keyboard scan code

This  function  return  the  next character  available  from  the

keyboard.  If no keyboard input is available, this function waits

until  a keyboard  character  is available.   Returned  character

codes are the same as function 0Dh (IBM style scan codes).

13

Function 0Fh - Flow Control for serial I/O.

Input:    AH = 0Fh

AL = Bit defining the flow control to use

DX = Port number

Output:   Nothing

Two kinds of basic flow control are supported.  One is  software,

called  Xon/Xoff  flow control.   The  other is  hardware, called

RTS/CTS flow  control.  Both  software and hardware  flow control

may be enabled at the same time.

The bits in the parameter passed in AL are as follows:

Bit 0 = 1      Enable receiving of Xon/Xoff

Bit 1 = 1      Enable RTS/CTS flow control

Bit 2          Reserved (should be zero)

Bit 3 = 1      Enable transmitting of Xon/Xoff

Setting  an appropriate bit to  zero will cause  the flow control

associated with that bit to be disabled.

Enabling  the  receiving  of  Xon/Xoff  will  cause  X00  to stop

transmitting   upon  receiving   an  Xoff.     X00   will  resume

transmitting when an Xon is received.

Enabling the sending of  Xon/Xoff will cause X00 to send  an Xoff

when its buffers are 3/4  full.  When the buffers are  emptied to

1/4 full, X00 will send an Xon.

X00  WILL ALWAYS CEASE  TRANSMITTING WHEN CTS  IS LOWERED (TURNED

OFF).  TRANSMISSION WILL RESUME WHEN CTS IS RAISED (TURNED ON).

When  RTS/CTS flow control is  enabled, X00 will  drop (turn off)

RTS when  the receive buffer is  3/4 full.  X00  will raise (turn

on) RTS  when the  receive buffer empties  to 1/4 full.   RTS/CTS

flow control is almost  always used by modems that  operate above

2400  baud.    Some  2400  baud  modems  also  use/allow  RTS/CTS

handshaking

If the  baud rate is locked  in the command line  that loads X00,

RTS/CTS  handshaking is  forced.   In this case,  the application

program can not disable RTS/CTS handshaking.

14

Function 10h - Control-C  /  Control-K  checking and  transmitter

disable/enable.

Input:    AH = 10h

AL = Control bits (defined below)

DX = Port number

Output:   AX = 0001h - A Control-C/K has been received

= 0000h - A Control-C/K has not been received

This is  used for BBS operation, primarily.  A bit mask is passed

in AL with the following flags:

Bit 0     Enable/disable Control-C / Control-K checking

Bit 1     Disable/enable the transmitter

To enable Control C/K checking set bit 0 = 1, to disable set  bit

0  = 0.    When checking  is  enabled,  a received  Control-C  or

Control-K will set  a flag  (internal to X00).   The  Control-C/K

character will not be stored  in the input buffer.  The  state of

the  internal Control-C/K flag will  be returned (then reset) the

next time  this function is  called.   The returned value  may be

used by the BBS software as it wishes.

The  transmitter disable  (bit 1  =  1) and  Enable (bit  1 =  0)

function lets the application program to inhibit the transmitter.

The disable of the transmitter is not absolute.  Several internal

functions can  cause the  transmitter to be  re-enabled, such  as

receiving an Xon when receiving of Xon/Xoff is enabled.

-----------------------------------------------------------------

Function 11h - Set current cursor location.

Input:    AH = 11h

DH = Row (line)

DL = Column

Output:   Nothing

This function  is exactly like INT 10h, subfunction 2, on the IBM

PC.  The  desired cursor location is passed in DX,  row in DH and

column in DL.  The row and column are relative to zero.  That is,

the home position on the screen is row 0, column 0.

15

Function 12h - Read current cursor location.

Input:    AH = 12h

Output:   DH = Row (line)

DL = Column

This function  is exactly like INT 10h, subfunction 3, on the IBM

PC.   The  current  cursor location  (using  the same  coordinate

system as function 16h) is returned in DX.

-----------------------------------------------------------------

Function 13h - Single character ANSI write to screen.

Input:    AH = 13h

AL = Character to display

Output:   Nothing

The character passed in AL is sent displayed on the screen at the

current cursor location.   ANSI.SYS type processing is performed.

This routine should  not be used  in such a  way that DOS  output

(which is not re-entrant)  can not be employed by  X00 to execute

the function.

-----------------------------------------------------------------

Function 14h - Enable or disable the DCD watchdog.

Input:    AH = 14h

AL = 01h - Enable watchdog

= 00h - Disable watchdog

DX = Port number

Output:   Nothing

This  function enables  and  disables the  monitoring of  carrier

detect.   When enabled, the state  of the carrier detect (DCD) is

constantly  monitored  during  timer  tick  processing.    Should

carrier  detect  be  lost  (turned  off),  the  system  will   be

re-booted.  Activate/Deactivate etc  have no effect on the  state

of the DCD watchdog.

16

Function 15h - Write character to screen using BIOS.

Input:    AH = 15h

AL = Character to display

Output:   Nothing

The character passed in AL is sent to the screen using BIOS-level

Input/Output routines.   This differs from  function 13h in  that

DOS I/O will not be used.

-----------------------------------------------------------------

Function 16h - Add or delete a routine from the timer tick chain.

Input:    AH = 16h

AL = 01h - Add a function

= 00h - Delete a function

ES = Segment of function

DX = Offset of function

Output:   AX = 0000h - Operation successful

= 0FFFFh - Operation unsuccessful

This  function is used to allow a central authority to manage the

timer interrupts, so that the integrity of the "timer tick chain"

is  not compromised.  Rather than using the traditional method of

saving  the old contents of the timer vector, storing the address

of your routine there, and calling the "old" routine, instead you

call this function.  X00 manages a list of such  entry points and

calls them on a timer tick (interrupt) using a FAR call.

X00  will allow  up to  4 programs  to hook  into the  timer tick

processing.   Interrupts are enabled when the hooked routines are

called.

CAUTION, when  this FOSSIL function was  implemented, it provided

the  best  means to  hook into  the  timer tick  on multi-tasking

systems at that  time.  However,  with the  386 and virtual  8086

mode  of  operation used  by  DESQview,  Windows V3.0  etc,  this

function can crash the  system.  The  reason is the virtual  8086

application (task/window)  that hooked in, may not  be the active

virtual 8086 (task/window) when the timer tick interrupt occurs.

Due to popularity of the virtual 8086 type multi-tasking systems,

the  author  (who  originally  insisted  that  this  function  be

created) RECOMMENDS THAT THIS FUNCTION NOT BE USED.

17

Function 17h - Reboot system.

Input:    AH = 17h

AL = 00h - "Cold boot"

= 01h - "Warm boot"

Output:   Nothing

This  function is intended primarily  as a security  feature.  It

provides  the  application  with  a  last resort  escape  from  a

situation that could allow a security breach.

-----------------------------------------------------------------

Function 18h - Block Read.

Input:    AH = 18h

CX = Requested number of characters to transfer

DX = Port number

ES = Segment of the caller's buffer

DI = Offset of the caller's buffer

Output:   AX = Number of characters actually transferred

This function will transfer from 0 to 65534 characters from X00's

inbound circular  buffer to  the caller's  buffer. ES:DI  are not

changed by  this function.   The  number of  bytes placed  in the

caller's buffer is  returned in AX.   The  number returned in  AX

will be the lesser of the caller's requested amount or the entire

contents (if any) of X00's receive buffer.

-----------------------------------------------------------------

Function 19h - Block Write.

Input:    AH = 19h

CX = Requested number of characters to transfer

DX = Port number

ES = Segment of the caller's buffer

DI = Offset of the caller's buffer

Output:   AX = Number of characters actually transferred

This function will transfer  from 0 to 65534 characters  to X00's

outbound  circular  buffer.    ES:DI  are  not  changed  by  this

function.  The number of bytes placed in X00's buffer is returned

in AX.   The  number returned  in AX  will be  the lesser  of the

caller's requested amount or the amount (if any) required to fill

X00's transmit buffer.

18

Function 1Ah - Break begin or end.

Input:    AH = 1Ah

AL = 01h - Start sending 'break'

= 00h - Stop sending 'break'

DX = port number

Output:   Nothing

This function will  start or terminate a break signal.  If AL=01h

X00 will start the transmission  of a break.  If AL=00h  X00 will

terminate  any  break  signal.    This  function  is  useful  for

activating command mode on some modems.  The  application program

is responsible  for the timing  of the  BREAK.  If  X00 has  been

restrained by an Xoff received  from the modem, the flag  will be

cleared.   An  Activate or  Deavtivate will  stop an  in-progress

BREAK.

It is the  experience of the author that the  duration of a break

should be less than 0.1 seconds.

19

Function 1Bh - Return information about X00.

Input:    AH = 1Bh

CX = Size of caller's info buffer in bytes

DX = Port number

ES = Segment of caller's info buffer

DI = Offset of caller's info buffer

Output:   AX = Number of bytes actually transferred

CX = '0X'      First 2 bytes of 'X00 ' string

DX = ' 0'      Second 2 bytes of 'X00 ' string

Note  that the  values returned  in CX  and DX  can be  stored in

memory to form a  space terminated string 'X00 '.   Additionally,

the values returned in CX and DX can be used to  determine if X00

is installed.

This function will transfer information about X00 and its current

status  to the  caller.   This  function  is intended  to  assist

"generic" applications to adjust to the current environment.

The data structure  (assembly language style)  currently returned

by X00 is as follows:

INFO      EQU  $              ;DEFINE BEGIN OF STRUCTURE

STRSIZ    DW   INFO_SIZE      ;SIZE OF THIS STRUC IN BYTES

MAJVER    DB   CURR_FOSSIL    ;FOSSIL SPECIFICATION REV

MINVER    DB   CURR_REV       ;REV OF THIS DRIVER.

IDENT     DD   ID_STRING      ;"FAR" POINTER TO ASCIIZ

; DRIVER DESCRIPTION STRING.

IBUFR     DW   IBSIZE         ;BYTE SIZE OF THE INPUT BUFFER

IFREE     DW   ?              ;NUMBER OF BUFFERED RECV BYTES

OBUFR     DW   OBSIZE         ;BYTE SIZE OF THE XMIT BUFFER

OFREE     DW   ?              ;NUMBER OF BUFFERED XMIT BYTES

SWIDTH    DB   SCREEN_WIDTH   ;WIDTH OF DISPLAY SCREEN

SHEIGHT   DB   SCREEN_HEIGHT  ;HEIGHT OF DISPLAY SCREEN

BAUD      DB   ?              ;BAUD RATE, COMPUTER TO MODEM

The IDENT  string  is null-terminated,  and  does not  contain  a

newline.  The baud rate byte contains  the bits that Function 00h

would use to set the port to that speed.

The  fields related to a particular port (buffer size, space left

in the  buffer, baud rate) will  be undefined if port  0FFh or an

invalid port is contained in DX.

In some cases  X00 can  not accurately return  information.   For

example, the user can  lock the baud rate at  115200 when loading

X00.  There is no correct FOSSIL value that X00 can return in the

BAUD  field.  X00 will set the  BAUD field equal to the last baud

rate that the application attempted to set.

20

Superset Functions.

Function  numbers above 1Bh and below 7Eh are X00 only functions.

These functions will only  work with X00.  It is  the hope of the

author that these functions will be added by other FOSSIL authors

in  their  implementations  and  become  a  part  of  the  FOSSIL

specification.

Functions  1Ch and 1Dh are exact duplicated of FOSSIL functions 4

and 5.

Functions  1Eh and  1Fh are  intended to  be exact  duplicates of

functions 4 and 5 of the PS/2's BIOS INT 14h.

Function 20h is a destructive read with no wait.

Function 21h is a Stuff/Poke character into the receive buffer.

21

Function  1Ch is  a  duplicate of  function 4.   The  function is

duplicated so that at a future date, X00 functions 4 and 5 can be

made compatible to the PS/2's function 4 and 5.

Developers  are encouraged by the author to start using functions

1Ch and  1Dh instead of functions  4 and 5.   See the application

notes  section  for  instructions   on  a  compatible  method  of

determining  if functions 1Ch and  1Dh should be  used instead of

functions 4 and 5.

Function 1Ch - Activate Port.

Input:    AH = 1Ch

DX = port number

Output:   AX = 1954h if successful

BL  =  maximum  function  number   supported  (not

counting functions 7Eh and above)

BH = rev of FOSSIL specification supported

This function instructs X00  to use the FOSSIL  specification for

the specified port.  Prior to issuing this function, any function

that requires a port  number in DX will  be passed on to  BIOS or

X00's  INT 14h BIOS emulator.  When this function  is called, all

interrupts  involved   in  supporting  the   communications  port

(specified in DX) are set up for  support by X00.  DTR is  turned

on by this function.  The baud rate is NOT set or changed by this

function.

If an additional call  to this function occurs (2  Inits or Init,

Read, Init, etc.) X00 will reset (clear) all buffers and all flow

control is turned  off.  Values returned in the  registers upon a

re-init are the same as an initial init (defined above).

22

Function  1Dh  is a  duplicate of  function 5.   The  function is

duplicated so that at a future date, X00 functions 4 and 5 can be

made compatible to the PS/2's functions 4 and 5.

Developers  are encouraged by the author to start using functions

1Ch and  1Dh instead of functions  4 and 5.   See the application

notes  section  for  instructions  on  a  compatible  method   of

determining  if functions 1Ch and  1Dh should be  used instead of

functions 4 and 5.

Function 1Dh - Deactivate Port.

Input:    AH = 1Dh

DX = Port number

Output:   Nothing

This function instructs X00  that it should no longer  to process

calls for  specified port (in DX) using the FOSSIL specification.

If the port was never activated by function 4, then this function

is ignored.  Any  subsequent function calls, that require  a port

number  in  DX, will  be  passed  to BIOS  or  the  BIOS INT  14h

emulator.

23

Function 1Eh - Extended line control initialization.

This  function is intended to exactly emulate the PS/2's BIOS INT

14 services, function 4.  Some  or all of the functions  provided

here are duplicated by other X00 functions.

Input:    AH = 1Eh

AL = Break, Where:

= 00h, No break and/or turn off break

= 01h, Start send of break.

BH = Parity, where:

= 00h, No parity

= 01h, Odd parity

= 02h, Even parity

= 03h, Mark parity (always 1)

= 04h, Space parity (always 0)

BL = Stop bits, where:

= 00h, One stop bit

= 01h, Two stop  bits for 6,  7 and 8  bit word

length, 1  and 1/2  stop bits for  5 bit

word length.

CH = Word length, where:

= 00h, 5 bits

= 01h, 6 bits

= 02h, 7 bits

= 03h, 8 bits

CL = Baud rate, where:

= 00h, 110 baud

= 01h, 150 baud

= 02h, 300 baud

= 03h, 600 baud

= 04h, 1200 baud

= 05h, 2400 baud

= 06h, 4800 baud

= 07h, 9600 baud

= 08h, 19200 baud

DX = Port number

Output:   AX = Port status (see function 03h)

If  locked at X00 load  time, the appropriate  parameters of this

function are ignored.

24

Function 1Fh - Extended serial port status/control.

This  function is intended to exactly emulate the PS/2's BIOS INT

14 services, function  5.  Some or all of  the functions provided

here are duplicated by other X00 functions.   This function has 2

subfunctions specified by AL.

Subfunction 00h - Read MCR

Input:    AH = 1Fh

AL = 00h, Read modem control register (MCR)

DX = Port number

Output:   AX = Port status (see function 03h)

BL = Modem control register, where:

Bits 7-5 = 0 (Reserved)

Bit 4    = 1 Loopback mode

Bit 3    = 1 OUT2 (interrupts) enabled

Bit 2    = 1 OUT1 active

Bit 1    = 1 Request to send active

Bit 0    = 1 Data terminal ready (DTR) active

Subfunction 01h - Write MCR

Input:    AH = 1Fh

AL = 01h, Write modem control register (MCR)

BL = Modem control register, where:

Bits 7-5 = 0 (Reserved)

Bit 4    = 1 Set loopback mode

Bit 3    = 1 Set OUT2 enable interrupts

Bit 2    = 1 Set OUT1 active (on)

Bit 1    = 1 Set Request to send active (on)

Bit 0    = 1 Set DTR active (on)

DX = Port number

Output:   AX = Port status (see function 03h)

In subfunction 01h (write  MCR) X00 will force bit 3  to 1.  That

is,  X00  will not  allow  the  communications interrupts  to  be

disabled.

RTS may  be used  as a  flow  control signal  by X00.   When  the

application program writes the MCR, the  RTS bit is treated as an

RTS enable  bit.   This  means  that X00  will always  allow  the

application program to turn RTS off.  However, X00 will  not turn

on RTS unless it is safe to do so.

25

Function 20h - Read with no wait (destructive).

Input:    AH = 20h

DX = Port number

Output:   AH = 00h if character is available

AL = Next character if available

AX = 0FFFFh If no character is available

This function returns the next character from the receive buffer.

If the receive buffer is  empty, 0FFFFh is returned in AX.   This

function  is the same as  function 0Ch except  that any character

returned is removed from the receive buffer.

----------------------------------------------------------------

Function 21h - Stuff/Poke the receive buffer.

Input:    AH = 20h

AL = Character to place in the receive buffer

Output:   Nothing

This function will  insert the passed character  into the receive

buffer (at the end).  Subsequent reading of the serial input will

read the character.  The character is inserted at the  end of the

buffer (as though it were just received).

The  character is inserted into the receive buffer by calling the

receive interrupt  routine.    All  normal  receive  checking  is

preformed on the character.  Some things to note are:

If  receiving  of  Xon/Xoff  is   enabled  and  an  Xoff  is

stuffed/poked  into the  buffer,  the transmitter  will stop

until a an Xon is received or stuffed/poked.

If  Control C/K  checking is  enabled and  a Control  C/K is

stuffed/poked, then the character  is not put in  the buffer

and the internal flags is set.

The above should give you a little to think about when using this

function.

This  function  is intended  to be  fully  re-entrant.   Thus, it

should be callable from timer tick processing etc.

26

"Layered Application" services.

The   functions   below  are   provided   to   meet  the   FOSSIL

specification.   The following  functions  in effect  make X00  a

dispatcher for other programs.

To  operate  properly  on   a  multi-port  system,  any  "Layered

Application" must be re-entrant.   The only "Layered Application"

know  to the  author is  called a  Video FOSSIL.   The  one Video

Fossil known to the author is not re-entrant and can  not be used

on a multi-port system.

Function 7Eh - Install an "external application" function.

Input:    AH = 7Eh

AL = Code assigned to external application

DX = Offset of application entry point

ES = Segment of application entry point

Output:   AX = 1954h

BL = Code given to application (same as input AL)

BH = 01h - Installation was successful

= 00h - Installation failed

This  function may be used by  external application code (special

screen  drivers, modem code, database code, etc) to link into the

INT 14h service  for use  by multiple applications.   The  "error

return"  (BH=0   with  AX=1954h)   should  mean  that     another

application layer  has already been installed  at that particular

code. Codes 80h through 0BFh are be supported.

External   application  codes  80h-83h  are  reserved  by  FOSSIL

developers  for  re-organizing  FOSSIL  services by  type  (comm,

screen, keyboard, system).

Installed  application code will be entered, via a FAR call, from

the  INT 14H  dispatcher  whenever  it  is  entered  (with  AH  =

application code).

If the value returned in AX  from this function is not 1954h, the

service code that  is trying to be installed should  bring up its

own  INT  14h code  that can  service  INT 14h  functions 7h-0BFh

(80h-0BFh are "applications").

27

Function 7Fh - Remove an "external application" function.

Input:    AH = 7Fh

AL = Code assigned to external application

DX = Offset of application entry point

ES = Segment of application entry point

Output:   AX = 1954h

BL = Code given to application (same as input AL)

BH = 01h - Removal was successful

= 00h - Removal failed

This  function removes an application's entry from the table.  If

an error is returned, this means ES:DX did not match any entry at

the  slot described by  AL.  An application  that wants to remove

itself from memory  can issue  the 7F function  to remove  itself

from  the  table,  then,  if it  is  successful,  un-install.  If

installed with an INT  14h dispatcher it may  un-install provided

no other application(s) have  been installed on top of  it (using

its dispatcher).

28

Application Notes.

Which Activate/Deactivate functions to use

It is  the hope of  the author (after  a period of  time) to make

X00's  functions 4  and  5 compatible  with  the PS/2's  INT  14h

functions  4  and 5.    Currently, X00's  functions 4  and  5 are

duplicated at functions 1Ch and 1Dh respectively.  I request that

application  programmers start  using  functions 1Ch  and 1Dh  if

those  functions are available.   That is, use  functions 1Ch and

1Dh instead of functions 4 and 5 respectively.

It is relatively simple to determine if the  functions at 1Ch and

1Dh are available.   When your  program is ready to  activate X00

for a given port, issue function 1Ch.  If 1954h is returned, then

the port  is activated.   If 1954h  is not returned,  reissue the

request to function 4.  Save the function number actually used to

activate the port,  then increment  it by one  to deactivate  the

port.

Watch out for disk I/O

Chris Irwin deserves  credit for discovering and  informing me of

the following:  On  some (if not all) IBM/Clones,  interrupts are

masked  for a long  period of  time during  DOS disk  I/O.   As a

result, data  characters were  being lost  during  receive.   The

problem is most noticeable on slower (4.77 meg) systems with 8250

or 16450 type SIO chips installed using high speed modems.

Remembering  that  all high  speed modems  (that  I know  of) use

RTS/CTS  handshaking, I implemented a back door hook in some Beta

X00s that  allowed Chris to turn  RTS off prior to  any disk I/O.

Turning off  RTS while  a disk  write is  taking place fixed  the

problem.   Also, turning off RTS  during disk I/O had  no adverse

effect  on the transfer speed due to the modems internal buffers.

While RTS is off,  the modem simply buffers the  data internally.

When  RTS is  again  turned on,  the  modems internal  buffer  is

emptied  because the transfer rate from the modem to the computer

is faster than the line receive rate.

X00  now has a front door (documented) function (1Fh) that allows

application  programs to disable RTS.  Note that X00 function 1Fh

is not a FOSSIL function.  I hope that it  will eventually become

a FOSSIL function.  X00s function  1Fh is close to a duplicate of

the PS/2's BIOS INT 14h, function 5.

I strongly suggest that  application programmers consider turning

off (disabling) RTS prior to any disk  I/O.  Even if the modem is

not  using RTS/CTS handshaking, turning  off RTS will  not do any

harm (that I know of).

29

Successive timer tick calls

In some  environments,  the timer  tick chain  is called  several

times in succession with  no delay between the calls.   I believe

that one such environment is DESQview.  If a task has been asleep

for 10  timer ticks, the timer tick chain will be called 10 times

when the task is activated.

X00 checks the DWORD at 46Ch.  If the  value at that location has

not  changed,  X00  ignores  the  timer  tick.    Any  hooked  in

application programs are not called.

This  means  that the  timer tick  is  not reliable  for accurate

timing.   All programs  (that I  know of)  that alter  the normal

timer  tick  processing keeps  the DWORD  at  46Ch (time  of day)

updated.  If not, your  time of day will be off.  If  you need to

know elapsed  time, do not count  timer ticks.  Use  the DWORD at

46Ch to calculate elapsed time.  The value stored in the DWORD at

46Ch  is the number of timer ticks  since the computer was booted

or the number of timer ticks past midnight.

30

Application Notes . . . . . . . . . . . . . . . . . . . . . .  28

FOSSIL  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Function 00h  . . . . . . . . . . . . . . . . . . . . . . . . . 5

Function 01h  . . . . . . . . . . . . . . . . . . . . . . . . . 6

Function 02h  . . . . . . . . . . . . . . . . . . . . . . . . . 6

Function 03h  . . . . . . . . . . . . . . . . . . . . . . . . . 7

Function 04h  . . . . . . . . . . . . . . . . . . . . . . . . . 8

Function 05h  . . . . . . . . . . . . . . . . . . . . . . . . . 9

Function 06h  . . . . . . . . . . . . . . . . . . . . . . . . . 9

Function 07h  . . . . . . . . . . . . . . . . . . . . . . . .  10

Function 08h  . . . . . . . . . . . . . . . . . . . . . . . .  10

Function 09h  . . . . . . . . . . . . . . . . . . . . . . . .  10

Function 0Ah  . . . . . . . . . . . . . . . . . . . . . . . .  11

Function 0Bh  . . . . . . . . . . . . . . . . . . . . . . . .  11

Function 0Ch  . . . . . . . . . . . . . . . . . . . . . . . .  11

Function 0Dh  . . . . . . . . . . . . . . . . . . . . . . . .  12

Function 0Eh  . . . . . . . . . . . . . . . . . . . . . . . .  12

Function 0Fh  . . . . . . . . . . . . . . . . . . . . . . . .  13

Function 10h  . . . . . . . . . . . . . . . . . . . . . . . .  14

Function 11h  . . . . . . . . . . . . . . . . . . . . . . . .  14

Function 12h  . . . . . . . . . . . . . . . . . . . . . . . .  15

Function 13h  . . . . . . . . . . . . . . . . . . . . . . . .  15

Function 14h  . . . . . . . . . . . . . . . . . . . . . . . .  15

Function 15h  . . . . . . . . . . . . . . . . . . . . . . . .  16

Function 16h  . . . . . . . . . . . . . . . . . . . . . . . .  16

Function 17h  . . . . . . . . . . . . . . . . . . . . . . . .  17

Function 18h  . . . . . . . . . . . . . . . . . . . . . . . .  17

Function 19h  . . . . . . . . . . . . . . . . . . . . . . . .  17

Function 1Ah  . . . . . . . . . . . . . . . . . . . . . . . .  18

Function 1Bh  . . . . . . . . . . . . . . . . . . . . . . . .  19

Function 1Ch  . . . . . . . . . . . . . . . . . . . . . . . .  21

Function 1Dh  . . . . . . . . . . . . . . . . . . . . . . . .  22

Function 1Eh  . . . . . . . . . . . . . . . . . . . . . . . .  23

Function 1Fh  . . . . . . . . . . . . . . . . . . . . . . . .  24

Function 20h  . . . . . . . . . . . . . . . . . . . . . . . .  25

Function 21h  . . . . . . . . . . . . . . . . . . . . . . . .  25

Function 7Eh  . . . . . . . . . . . . . . . . . . . . . . . .  26

Function 7Fh  . . . . . . . . . . . . . . . . . . . . . . . .  27

Licensing Information . . . . . . . . . . . . . . . . . . . . . 2

Commercial License . . . . . . . . . . . . . . . . . . . . 2

Non-Commercial License . . . . . . . . . . . . . . . . . . 2

RTS/CTS flow control  . . . . . . . . . . . . . . . . . . . .  13

SIO . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Warranty  . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

WHAT IS X00 . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Xon/Xoff flow control . . . . . . . . . . . . . . . . . . . .  13