OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	September 13, 1996 

FROM:	Intel Applications Support (ISA)

SUBJECT:	TC430HX BIOS Release 4.0

About This Release

o	BIOS 1.00.04DH0 (S3 ViRGE and VPlus) Release 4.0 - 2 Meg Flash Device

o	S3 ViRGE video BIOS version 1.00-08-C2-01,518

o	S3 T64V+ video BIOS version 1.03-06-C2-01,518

Features/Errata Fixed In This Release

Release 4.0

o	Added code to properly handle the Parity/ECC error report.

o     Added the capability to detect if an Intel processor is installed, and display “Intel” on the

POST screen along with the board name and the serial number.

Release 3.0

o	Added code to reset the DSP of the Yamaha Sound Blaster™ Compatible device before boot.

o	Added support to display “with MMX™ Technology” to the Advanced Setup Menu.

o	Enabled the Passive Release and Delay Transaction of the 82430HX chip set.

o	Added recovery via Misty card support.

o     Added ECC single-bit and multi-bit error support

Release 2.0

o	Added ability to display VPD data on the screen during bootup. Also added a new DMI                                switchboard interface to enable and disable the displaying of the VPD data.

o	Fixed bug where the INT15 cache services were not returning the "function not supported" message (AH = 86h) when attempting to enable or disable cache under certain defined circumstances.

o	Fixed the long boot time problem.

o	Fixed the incorrect hard-disk predelay values.

o	Added changes for WIN95 suppliment LOGO acceptance tests (16-bit decode).

o 	Fixed problem where motherboard function 0 data was nulled if no mouse installed.

o	Added more boundary checking to INT 13.

o	Added check to make sure that enough room is available in the F000 block to write the DMI Structure Table.  If there is not enough room, the procedure will disable maskable interrupts and Halt the processor.

o	Updated to allow additional LPT and COM addresses per PC95A specifications.

o	Updated Setup nodes such that if a Non-Windows95™ option is chosen, for the Plug and Play Configuration screen, that any Non-Standard Parallel or Serial port choices are invalidated and the Configuration Mode is switched to Auto for the Parallel and Serial ports.

o	Fixed a problem with getting into setup when the setup PROMPT was disabled.

o	Fixed the problem where the MPU401 & Sound Blaster were disabled but the IRQ was left alive in chip.

Release 1.0

o	Fixed the problem where the system hangs up when the mouse is disabled dynamically in Windows95™ Environment.

o	Changed the device node for the Yamaha Audio Chip, where the available DMA was incorrectly reported to PnP operating systems.

o	Fixed the problem where, after configuring the Yamaha audio solution via CMOS, the Plug and Play configuration menu showed that resources assigned to audio were listed as available under the “Use BIOS Setup” option.

o	Enhanced Desktop Management Interface (DMI).

o	Incorporated new S3 T64V+ video BIOS.