OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	July 31, 2026

FROM:	Intel Applications Support - Oregon

SUBJECT:	 Advance/ZE BIOS 6.0

Features/Errata Fixed In This Release

Release 6.0

o	Added fault tolerant flash write method.

o	Fixed problem where some token ring cards can fail.

o	Fixed problem where ESCD said mouse device node disabled but mouse is enabled.

o	Changed POST string "CD-ROM Drive" to "ATAPI Device".

o	Changed to stop using the battery bit in the RTC as an indication of CMOS validity. This bit in the National 306 has been shown to be unreliable.

o	Added code to check for valid day. Added code to set time to 00:00:00 on date/time error.

o	Fixed problem of fields being accessible when Administrative password restrictions are in effect.

o	Fixed problem where setup could disable primary or secondary IDE channel but resources were still allocated.

o	Fixed problem where Setup cannot show correct secondary IDE channel status.

o	Fixed problem in which ESCD was written without valid signature.

o	Added code to clear system RAM just prior to boot.

o	Fixed problem in which ESCD got trashed if a multifunction PCI card could not be configured.

o	Added a mechanism to prevent WIN95ä from enabling a disabled node.

o	Fixed a problem where  ECP wasn't enabled when in the manual mode.

o	Fixed problem of not being able to run a .exe file from a 1.44 floppy after having booted from a 720K floppy.

o	Fixed problem where some PCI video cards would not work behind a bridge.  The fix also fixed video cards with build-in bridges.

o	Fixed problem where mouse node was set to non-configurable when mouse present.

o	Fixed year 2000 date problem.

o	Added new algorithm to detect processor speed.  Now setup shows the correct CPU speed.

o	Added new EPP device node to fix issue with WIN95 BIOSTEST failure which says EPP fails due to one DMA channel given not two.

o	Fixed display of 2E8 serial port options when certain CMOS picks chosen.

o	Fixed display variable pick such that if new options just added to setup are chosen that the other Serial port will not display that I/O address.

o	Allow for PCI cards with ROMs greater than 64K, we will shadow and execute properly.

o	Fixed problem where a PCI card without an ESCD entry was disabled if the previous card/function was disabled.

o	Fixed problem of system hanging when in virtual mode and a jump to the restart vector is executed.

o	Move POST scratch buffer above 1MB so add-in cards that search for free memory can find some.

o	Change dynamic mouse enable/disable to meet new Microsoft requirements.

o	Changes Serial Port 2 IR mode to half-duplex.

o	Corrected error where last byte of logo block was overwritten with a zero whenever the jumper to clear CMOS (and ESCD) is set and ESCD has invalid header offset length.

o	Changed to prevent incorrect operation of IRQs selected/deselected.

o	Changed IRQ assignment priority, so mouse IRQ not given away prematurely.

o	Fixed disabling/enabling of cache during POST.

o	Fixed an issue where some CD-ROMs were not detected.

Release 5.0

o	Disabling either the primary or secondary IDE channel (via the Manual setting of the configuration Mode field of the Peripheral Configuration screen) will force Interrupt 14 and/or 15 to be released  (interrupts are available for use by other devices).  These interrupts are added to the list of available interrupts when the Use Setup Utility option of the Plug and Play screen is used.  Includes renaming Peripheral Configuration screen’s primary and secondary IDE field names.  Format is “Primary PCI IDE Interface” and “Secondary PCI IDE Interface”.

o	Reinstated the Hard Disk Pre-Delay feature.

o	Fixed problem where CPU Auto HALT features were incorrectly being disabled.  The fix lowers power consumption about 5 watts during CPU inactivity even when APM is disabled in setup (as long as a power driver is installed).

o	Added call to detect serial mouse if a PS/2 mouse is not installed in the system.  Will now wake the system if a serial mouse is installed and a PS/2 mouse is not installed.

o	Fixed problem in which not all resources were freed following the failure of allocating resources for a PCI device.

o	Fixed timing loop in keyboard controller related code that was causing problems with network remote diskless boot.

o	Set Input attribute in Keyboard device node.

o	Set IPL attribute in Floppy and IDE device nodes.

o	Corrected IRQ programmed for 3F8,IRQ 3;  2F8,IRQ 4;  3E8,IRQ 3; & 2E8,IRQ 4. Programming IRQ 3 & 4 reversed.  3F8,IRQ 4;  2F8,IRQ 3;  3E8,IRQ 4;   & 2E8,IRQ 3 were programmed correctly.

o	Corrected problems and potential problems possibly causing CMOS corruption.  Battery failure status was not being preserved through POST.

o	Re-enabled palette snooping for those PCI BIOSs that disable it during initialization and request palette snooping be enabled via setup.

o	Corrected floppy function 08, Return Disk Parameters.   It returned wrong parameters if the drive number was illegal.

o	Fixed bug in which BIOS text string could be displayed incorrectly.

o	Disabled Hotkey 6 in the keyboard controller. It's default is enabled and set to the Break Key.  This caused problems with some software.

o	Added an additional test on the boot record (test for 0000 at the start of the record) to avoid trying to boot from a non-system disk.

o	Updated the ATA/ATAPI detection code to be more robust.  Handles faster CPUs, more reliably detects CD-ROM drives, and handles IDE drives which act as master and slave when no slave exists.

o	Fixed Int. 15 Function C7h where bogus amount of memory could be returned.

o	Fixed password problem in which the password is entered followed by a backspace is considered valid.

Release 4.0

o	Updated device node ID for the parallel port when programmed for ECP mode.  Was 400h now is 401h.

o	Updated code so function 40h(Get PnP ISA config structure) doesn't use NVRAM.

o     Will now support Turbo/Deturbo in System Management Mode.

o	Changed APM function 53, subfunction 0AH that indicates that the system does not have a battery (Not a portable PC).

o	Added a more robust APM check that helps prevent the system from powering down in the middle of a disk operation.

o	Adjusted the way PS/2 mouse state (installed/deinstalled) was stored to prevent incompatibility with some UNIX OS’s.

o	Fixed Int. 15h function DAh subfunction 12h cx=02h (read cache status) in CX upon return to indicate Valid Cache size of zero (no cache installed), previously returned invalid cache size.

o	Change to Int. 15 Function DAh Subfunction 92h

-DX now returns the CPU internal speed in BCD for all supported processors.

o	Support for single language only.

o	Adjusted for recovery to function properly.

o	Adjusted for new National Semiconductor 87306-B support.

o	Fixed IRQ 5 and 7 display problems with ISA PNP screen of Setup.

o    Fixed problem with APM where it would loop going into and out	of System Management Mode when trying to come out of a powered down state, and video was disabled (not power managed) in setup.

Release 3.0

o	Fixed security hotkey.

o	Fixed bug with shared memory being programmed at C000.

Release 2.0

o 	Fixed problems with S3 Trio 32 Video failures via reduction in Video BIOS Clock rate reduction from 60Mhz to 57Mhz.  Also corrected some font issues.

o	Fixed problem with ISA Shared Memory Setup options. Failure was characterized by a failure of PCI video (with video error beep) when one would set the ISA Shared Memory option to start at C8000h.

o	Fixed problem which would cause Protected Mode applications/Operating Systems to have a General Protection Fault when modifying the Device Node  configurations for onboard Parallel Port or onboard PCI IDE.

o	Fixed problem revolving around BIOS Data Area offset 6Ch (dword - 32 bit value) a.k.a. Timer Count value.  Symptom of problem fixed:  “incorrect updating of Timer Count value in BIOS Data Area occurs during Sleep/Power-Down when running Windows 95”.  Fixed by utilizing Real Time Clock AND IRQ 0 (was previously only IRQ 0) as conditions for updating Timer Count value in BIOS Data Area.

o	Added enhancement to APM code to allow audio feedback (beeping) which indicates APM code detection of:

a)	Sleep/Resume/Soft Power Off button press (single beep),

b)	Soft Power Off via 2 second hold of Sleep/Resume/Soft Power Off button (two descending beeps).

o	Fixed PCI code dealing with devices Disabled.

o	Fixed PCI Bridge I/O Space allocation issues.

o	Adjusted code to always dispatch APM code regardless of Enable or Disabled via SETUP option.

o	Adjusted Motherboard Device Node to change the Serial Port Device Identifier from a Non-FIFO to a FIFO type device.

o	Fixed BIOS Error Display code to Not Wait for User Input if no Keyboard was detected.

o	Adjusted SETUP to allow IRQ 5 or 7 to be chosen for I/O 378h for parallel port.

o	Adjusted parallel port mode choices to support Compatible, Bi-directional, ECP, & EPP.

o	Fixed BIOS Error Display code to Not Display Resource Conflict Message when no PS/2 Mouse and add-in card using IRQ 12.

o	Fixed OS/2ä trap problem related to APM being Enabled/Disabled in SETUP.

o	Adjusted Resource Auto-configuration order for Windows 95ä so that motherboard resources will be allocated before PCI resources.  This has been implemented via a SETUP option.

o	Adjusted PCI Resource allocation related to Video IRQ assignment.  We can now, on a product by product basis, decide to not assign IRQ’s to PCI Video devices.

o	Fixed the programming of the Cache Line Size and Latency Timer for Bridges.

o	Fixed Motherboard Device Node Attributes to indicate “device can only be configured at runtime” from “device can be configured at runtime”.

o	Fixed SETUP code to make sure one interrupt was preserved for PCI usage.

o	Fixed IDE timing programming so that when have two different mode drives on a single channel the slower mode will get programmed for that channel.

o	Fixed SETUP code for display of IRQ 5 and IRQ 7 in Plug and Play screen.

o	Fixed APM code issues related to missing interrupts.

o	Fixed APM code for alarm events.

o	Adjusted APM code so front panel switch being held for 3 seconds will be interpreted as a soft power off.

o	Fixed PIIX IDE Master Latency Timer programming per SETUP.

o	Fixed Motherboard Device Node for IDE reporting it is always enabled even if not.

o    	Added new Crystal Audio code.  Fixes problems of hangs at boot time.

o	Added Product Information Storage and Retrieval features.

o	Fixed PCI and ISA PnP auto-configuration code so that a Configuration Error is flagged and the device is not disabled.

o	Adjusted ISA PnP auto-configuration code so that back-to-back Isolation is not performed.

o	Fixed IDE auto-configuration code so that an empty IDE channel (an IDE channel without any drives connected to it) would not hang the System BIOS at POST-Time.

o	Fixed jump to wrong label in ISA PnP auto-configuration code.

o	Fixed Int. 13h, Function 0Ch which was returning invalid value in AH.

o	Adjusted Message Server to handle Languages greater than 64k in size.

o	Adjusted ATA code to issue a second reset after ATA initialization to fix problem seen in some IDE CD-ROM drives not being accessible after boot.

o	Fixed code which handles freeform fields in SETUP to not update a field if invalid entry received.

o	Adjusted Motherboard Device Nodes to ignore the Checksum (per the “Plug and Play BIOS Specification” Version 1.0A) for Set System Node.

o    	Added new Crystal Audio code.