OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	July 30, 2026
 
FROM:	Intel Application Support-Oregon

SUBJECT:	Advanced /RH  1.00.07.CV2 BIOS Release 7.0

About This Release

o	BIOS 1.00.07.CV2 (ATI VT-A3/A4) Release 7.0 - 2 Meg Flash Device

Features/Errata Fixed In This Release

Release 7.0

o	Include both A3/A4 video BIOS’s and autodetect which revision is present and load the appropriate video BIOS. A3 - Revision 2.118, A4 - Revision 2.221

o	Included DMI fixes in structures (corrected values).

o	Included DMI CPU speed fix.

o	Fixed problem with dynamic disabling of PS/2 mouse in Win95.

o	Fixed APM 1.2 issues with Microsoft.

o	Fixed manufacturing defaults to comply with 4.20 spec.

o	Fixed issue with certain PCI SCSI cards. During device (PCI) initialization, the DIM buffer is now above 1MB and not below 640KB. So PCI ROMs which use this area for setup vector don’t conflict with the system BIOS.

o	Fixed ACFGTEST XREF failures with PXII3 device.

o	New Creative Vibra 16S/C audio detection and initialization code 	to better handle

onboard Vibra 16C/S and the use an offboard ISA Legacy Sound Blaster cards as

the sole audio solution.

If customer wishes to use an offboard Sound Blaster compatible card in

addition to onboard Vibra 16S or Vibra 16C, then Offboard Sound Blaster

compatible card MUST be an ISA PNP or PCI card (NOT ISA Legacy) which can be

dynamically configured to not interfere with the onboard sound device.

o  	BIOS 7.CV2 is a secure BIOS, which means that if the board was manufactured with BIOS 7, then the board can not be flashed down to a lower revision of BIOS.  If the

board was manufactured with BIOS 5.CV2 (a non -secure BIOS), then it can be updated to version 7.CV2, and at a later time be recovered using the Flash Recovery procedure back to a lower revision (non-secure version) of BIOS.

Release 6.0

o	Fixed an issue with the snoop ahead during PCI read cycles.

o	Reporting of 200mhz CPU speed was corrected.

o	Included in the latest DMI fix. Strings passed to DMI BIOS calls are now checked for null terminators.

o	Fixed an issue on the secondary controller which was not correctly identifying

some CD ROMs.

o	The memory detection fix was modified again. The cause of the problem was write buffering and the new routine flushes the buffer on writes.

o	70ns Memory Timing values were corrected.

o	ECC Memory Timings were corrected.

o	Fixed 512KB Cache not flushing properly.

o	Added APM 1.2 support

o	Added ECC support and new setup option.

o	Included new memory detection fix to eliminate the system finding memory in empty banks.

Release 5.0

o	Fixed a problem where phantom SIMMs were being detected.

o	Added backoff timings for Fast Page SIMMS

Release 4.0

o	Fixed a CMOS date bug.

Release 3.0

o	Fixed intermittent problem with Keyboard Controller not initializing.

Release 2.0

o	Fixed DMI issue with DMI slot information structures.

o	Fixed PnP issue with PS/2 mouse (Added back PS/2 Mouse Device Node)

o	Fixed an APM issue which now allows movement of the serial mouse to also wake up a system.

o	Included last Vibra 16S fixes related to disabling the device node.

Release 1.0

o	Updated copyright notice to the year 1996.

o	Fixed an ESCD update bug and related PCI post test 0Dh failure.

o	Fixed bug related to Unix Novell Netware. Cleared the BIOS ESCD area from 500h to the ESCD base prior to booting which fixed the Novell issue.