“Advanced ZP”

Expandable Desktop Baseboard

Technical Product Summary

PRELIMINARY

Preliminary Version 0.5

February 1995

Intel Corporation makes no warranty for the use of its products and assumes no responsibility for any errors which may appear in this document nor does it make a commitment to update the information contained herein.

Intel Corporation retains the right to make changes to these specifications at any time, without notice.

Contact your local sales office to obtain the latest specifications before placing your order.

The following are trademarks of Intel Corporation and may only be used to identify Intel products:

AboveÔ

ActionMediaâ

DVIâ

EtherExpressÔ

ExCAÔ

FaxBACKÔ

FlashFileÔ

i287Ô

i386Ô

i387Ô

i486Ô

i487Ô

i750â

i860ä

i960â

ICEä

iCOMPä

Indeoä

Intel287Ô

Intel386Ô

Intel387Ô

Intel486Ô

Intel487Ô

Intel Insideâ

iPSCâ

LANDeskÔ

MCSâ

NetPortÔ

OverDriveÔ

OverDriveä Ready

ParagonÔ

PentiumÔ

RapdiCADä

SatisFAXtionâ

StorageExpressä

TokenExpressÔ

The Indeo, OverDrive and PentiumTM logos are trademarks and the Intel and Intel Inside logos are registered trademarks of Intel Corporation.

Intel Corporation and Intel’s FASTPATH are not affiliated with Kinetics, a division of Excelan, Inc. or its FASTPATH trademark or products.

MDS is an ordering code only and is not used as a product name or trademark. MDS is a trademark or registered trademark of its respective owner.

Additional copies of Intel literature may be obtained from:

Intel Corporation

Literature Sales

P.O. Box 7641

Mt. Prospect, IL 60056-7641

TRADEMARK ACKNOWLEDGMENTS

Centronics is a trademark of Centronics Data Computer Corp.

IBM, MDA, MCGA, EGA, VGA, Personal System/2, PS/2, PC/XT and PC/AT are trademarks of International Business Machines.

PolySwitch is a registered trademark of Raychem Corporation.

All other trademarks are the property of their respective owners.

ã INTEL CORPORATION, 1994

Advanced ZP Expandable Desktop Baseboard

Preliminary Technical Product Summary

Table of Contents

Introduction

ADVANCED ZP ED FORM FACTOR

Board Level Features

CPU

PERFORMANCE UPGRADE

SECOND LEVEL CACHE

SYSTEM MEMORY

EXPANSION SLOTS

PERIPHERAL COMPONENT INTERCONNECT (PCI) PCISET

NATIONAL SEMICONDUCTOR 87306 SUPER I/O CONTROLLER

SYSTEM BIOS

CONNECTORS

POWER CONSUMPTION

Appendix A - User-Installable Upgrades

SYSTEM MEMORY

RTC BATTERY

Appendix B - Jumpers and Switches

EXTERNAL CPU CLOCK SPEED (50/60 MHZ) - SWITCHES 7 & 8

INTERNAL CPU BUS CLOCK SPEED - SWITCH 6

SETUP DISABLE - SWITCH 5

CLEAR CMOS - SWITCH 4

PASSWORD CLEAR - SWITCH 3

PROCESSOR VOLTAGE REGULATION - SWITCH 2

ISA BUS SPEED - SWITCH 1

RECOVERY BOOT ENABLE - J1K1

Appendix C - I/O Map

Appendix D - Memory Map

Appendix E - PCI Configuration Space Map

Appendix F - Interrupts & DMA Channels

Appendix F - Connectors

POWER SUPPLY CONNECTORS

PRIMARY POWER (J9J1)

AUXILIARY (3.3V) PCI POWER (J6G1)

Front Panel Connectors -(J2A1, J1B1)

SLEEP/RESUME

INFRA-RED

AUXILIARY 12V FAN POWER

SPEAKER CONNECTOR

TURBO LED

HARD DRIVE LED (DISK)

KEY LOCK/POWER LED

RESET CONNECTOR

I/O Connectors

PS/2 KEYBOARD & MOUSE PORTS (J8K2, J8K3)

SERIAL PORTS (J7K1, J7K2)

IDE CONNECTORS (J6H1, J6H2)

PARALLEL PORT CONNECTOR (J9E1)

FLOPPY CONNECTOR (J9C1)

ISA CONNECTORS (J1G1, J2G1, J2G2, J3G1)

PCI CONNECTORS (J4G1, J5G1, J5G2)

Appendix G - INTEL/AMIBIOS Setup

Setup Overview

INTRODUCTION

COMMAND SCREEN NAVIGATION

MAIN MENU NAVIGATION

Menus

MAIN MENU

MAIN MENU CONTINUED

MAIN MENU CONTINUED

MAIN MENU CONTINUED

ADVANCED MENU

SECURITY MENU

EXIT MENU

Appendix H - INTEL/AMIBIOS Error messages and Beep Codes

Appendix I - BIOS Recovery

USING THE UPGRADE UTILITY

RECOVERY MODE

Appendix J - Environmental Standards

Appendix K - Reliability Data

Appendix L - Customer Support

IPAN

IPUB

IPALS

FAXBACK

# Introduction

The Intel Advanced ZP Expandable Desktop (ED) Baseboard features the Pentium™ processor in a Baby-AT form factor baseboard with  I/O integrated.   Advanced ZP baseboards are focused on providing the best possible performance at the lowest possible price for mainstream desktop Pentium™ processor based computers.

Advanced ZP is a flexible baseboard which is available in Pentium™ processor CPU speeds of 50/75 MHz and 60/90 MHz, The processor is complemented by a standard 256kB asynchronous SRAM second level Write-Back cache and support for up to 128 MB of  Fast Page or EDO DRAM.  A Pentium™ Processor OverDrive™ socket (Socket 5) provides access to future performance enhancements.

The Advanced ZP ED baseboard offers outstanding I/O capabilities starting with the full set of PC I/O including  floppy drive, dual channel PCI local bus IDE interface, two serial ports with FIFO’s, an EPP/ECP capable parallel port, and an Infra-red (IrDA) port.  Three PCI local bus slots (1 full length capable) provide a high bandwidth data path for data-movement intensive functions such as graphics.  Integrated Bus Mastering PCI IDE controller provides two high performance IDE interfaces for hard drives and CD-ROM.  Bus mastering enhances the performance in multi-tasking environments such as Windows™ 95. Four ISA slots (three full length capable) complete the I/O mix.

In addition to superior hardware capabilities, features like bus mastering IDE, Windows™ 95-ready Plug and Play and Advanced Power Management (APM) with application restart are provided by software available from Intel for the Advanced ZP platform.

The Advanced ZP Expandable Desktop baseboard provides the foundation for cost effective, high performance, highly expandable platforms which deliver the latest in  CPU and I/O technology.

## ADVANCED ZP ED FORM FACTOR

The Advanced ZP ED baseboard is designed to fit into a standard Baby-AT form factor chassis.  Figure 1 illustrates the mechanical form factor for the Advanced ZP ED.  The actual dimensions of the Advanced ZP ED baseboard do not strictly adhere to the standard Baby-AT guidelines, exceptions to the standard are listed in the "BaseBoard Design Exceptions" section.

Figure 1. Advanced ZP ED Baseboard dimensions.

### Baseboard Design Exceptions

#### Keyboard connector

The standard Baby-AT form factor specifies an AT style keyboard connector.  The baseboard is designed to accept either a standard AT style keyboard connector or PS/2 style mouse and keyboard connectors.  Boards equipped with the PS/2 style connectors may require chassis modifications.  Boards with PS/2 style connectors will have different product codes than boards with AT style connectors.

#### Baseboard dimensions

The Advanced ZP is 3.0" shorter than the Baby AT standard.  The shorter board length may require some chassis to be modified to add additional mounting holes.

#### Mounting Hole Placement

The mounting holes located in the bottom left and right corners of Figure 1 are pseudo Baby AT standard and are available in many, but not all, Baby AT compatible chassis.

#### Front panel connectors

There is no front panel connector on the motherboard for a Turbo/Deturbo switch. The processor speed can be set either through a parameter in the CMOS Setup Utility, or from the keyboard (<CTL><ALT><+> = Turbo, <CLT><ALT><-> = Deturbo).  Changing processor speed from the keyboard may be prohibited by the operating system, or when the processor is in protected mode.

Setting the processor to deturbo (or slow) only slows the processor to the approximate equivalent of a 25 MHz clock rate, not the standard 8 MHz clock rate.

#### Jumpers/Switches

There is no Color/Mono jumper/switch on the baseboard to specify Monochrome or Color video mode at boot, the BIOS will automatically detect the type of video card installed.

Also, there is no Flash write protect jumper/switch, the BIOS needs to be able to write to FLASH to support the Plug and Play features.

# Board Level Features

A - Two PCI IDE interfaces

B - National 87306 I/O controller

C - Primary power connector

D - Four SIMM sockets (two banks)

E - 82438FX Triton Data Path (TDP)

F - Parallel port connector

G - Floppy drive connector

H - 256KB Secondary cache

I  - 82437FX Triton System Controller (TSC)

J - Socket 5 Pentium™ Processor socket

K- CPU voltage regulator

L - Front Panel I/O connectors

M - Configuration switch block

N - 82371FB PCI ISA/IDE Accelerator (PIIX)

O - Battery for the Real-time clock

P - Four ISA expansion connectors

Q - BIOS recovery boot jumper

R - Flash EEPROM for system BIOS

S - Three PCI expansion connectors

T - Serial port connectors

U - PS/2 Keyboard and Mouse port

(An optional AT style Keyboard connector can

also be ordered)

V - 3.3 volt power connector for PCI

## CPU

The Advanced ZP ED baseboard is designed to operate with 3.3 volt PentiumTM processors.  A patented on-board voltage regulator circuit provides the required 3.3 volts from the 5 volt tap provided by a standard PC power supply.  The baseboard supports the PentiumTM Processors 615/75 and 735/90, which run internally at 75 and 90 MHz, and have  iCOMPTM ratings of 615 and 735 respectively. The PentiumTM Processor is backward-compatible with the 8086, 80286,  i386TM and i486TM CPUs.  It supports both read and write burst mode bus cycles, and includes separate 8K on-chip code and data caches which employ a write-back policy.  Also integrated into the PentiumTM CPU is an advanced numeric coprocessor which significantly increases the speed of floating point operations, while maintaining backward compatibility with i486DX math coprocessor and complying to ANSI/IEEE standard 754-1985.

## Performance Upgrade

A 320 pin Zero Insertion Force socket provides users a "Level-5 OverDrive Ready" CPU performance upgrade path.  An OverDrive processor being developed for use with this socket will provide performance beyond that delivered by the originally installed PentiumTM Processor.

## Second Level Cache

The PentiumTM CPU's internal cache is complemented by a 256 KB direct mapped write-back second level cache implemented with eight 20nS 32kx8, 3.3v asynchronous SRAM devices. A 5v 32Kx8 external Tag SRAM (15nS) provides caching support for up to 64MB of system memory.

## System Memory

The Advanced ZP ED baseboard provides four 72-pin SIMM sites for memory expansion.  The sockets support 1M x 32 (4 MB), 2M x 32 (8 MB), 4M x 32 (16 MB), and 8M x 32 (32 MB) single-sided or double-sided SIMM modules.  Minimum memory size is 8 MB and maximum memory size, using four 8M x 32 SIMM modules, is 128 MB.  Memory timing requires 70 ns fast page devices or, for optimum performance, 70nS EDO DRAM.  Parity generation and checking is not supported.

The four sockets are arranged as Bank A and Bank B, with each bank consisting of two sockets and providing a 64-bit wide data path.  Both SIMMs in a bank must be of the same memory size and type, however Banks A and B may have different types of memory installed.  It is even possible to have Fast Page DRAM in one bank and EDO DRAM in the other, in which case each bank is independently optimized for maximum performance.  Bank A only, Bank B only, or both banks may be populated.  There are no jumper settings required for the memory size or type, which is automatically detected by the system BIOS.  Tin lead SIMMs are required to be used when adding Fast Page or EDO DRAM.

#### EDO DRAM

Extended Data Out (or Hyper Page Mode) DRAM is designed to improve the DRAM read performance.  EDO DRAM holds the memory data valid until the next CAS# falling edge, unlike standard fast page mode DRAM which tri-states the memory data when CAS# negates to precharge for the next cycle.  With EDO, the CAS# precharge overlaps the data valid time, allowing CAS# to negate earlier while still satisfying the memory data valid window time.

## Expansion Slots

Up to six expansion slots may be populated on the Advanced ZP ED baseboard.  There are four ISA bus expansion connectors and three PCI expansion connectors.  One slot is shared by connectors that will accommodate either an ISA or a PCI expansion card, but not both at the same time; thus accounting for the disparity between the number of slots and connectors.  All three PCI expansion slots accept PCI master cards, and fully comply with the PCI 2.0 specification.   A maximum of three ISA slots and one PCI slot can accommodate full length add-in cards.  Interference with the processor heat sink and CPU voltage regulator limits the rest of the ISA and PCI slots to being able to support only 1/2-length add-in cards.

### PCI 3.3 Volt Capabilities

To maintain strict compliance with the PCI specification, the baseboard provides a connector which can be used to route 3.3 volt power to the PCI slots.  The connector may be used with a separate 3.3 volt power supply or with a custom designed voltage converter.

Note: The on-board 3.3 volt regulator provides power for the CPU, PCIset and L2 cache only, not the PCI slots.

## PERIPHERAL COMPONENT INTERCONNECT (PCI) PCIset

The Intel Triton 82430FX PCIset consists of the 82437FX Triton System Controller (TSC), two 82438FX Triton Data Path (TDP) devices, and one 82371FB PCI ISA/IDE Accelerator (PIIX) bridge chip. The Triton PCIset provides the following functions:

· 	CPU interface control

· 	Integrated L2 write-back cache controller

– Pipelined Burst or standard SRAM

– 256kB or 512kB Direct Mapped

·	Integrated DRAM controller

– 64-bit path to Memory

– Support for EDO and Fast Page DRAM

– 4 MB to 128 MB main memory

· 	Fully synchronous PCI bus interface

– 25/30/33 Mhz

– PCI to DRAM > 100 Mbytes/sec

– PCI to DRAM posting of 12 Dwords

– 5 Dword buffers for CPU to PCI write posting

– 4 Dword buffers for PCI to Memory bus master cycles

– Support for up to 5 PCI masters

· 	Interface between the PCI bus and ISA bus

· 	Integrated fast IDE interface

– Support for up to 4 devices

– PIO Mode 4 transfers up to 16MB/sec

– Integrated 8 x 32-bit buffer for PCI IDE burst transfers

· 	Enhanced Fast DMA controller

· 	Interrupt controller and steering

· 	Counters/Timers

·	SMI interrupt logic and timer with Fast On/Off mode

### 82437FX Triton System Controller (TSC)

The 82437FX provides all control signals necessary to drive a second level cache and the DRAM array, including multiplexed address signals.  It also controls system access to memory and generates snoop controls to maintain cache coherency.  The TSC comes in a 208 pin QFP package.

### 82438FX Triton Data Path (TDP)

There are two 82438FX components which provide data bus buffering and dual port buffering to the memory array.  Controlled by the 82437FX, the 82438FX devices add one load each to the PCI bus and perform all the necessary byte and word swapping required.  Memory and I/O write buffers are included in these devices.  The TDP devices are 100 pin QFP packages.

### 82371FB PCI ISA/IDE Accelerator (PIIX)

The 82371FB provides the interface between the PCI and ISA buses and integrates a dual channel fast IDE interface capable of supporting up to 4 devices,  seven type F DMA channels, five 16-bit timer/counters, two eight-channel interrupt controllers, PCI-to-AT interrupt mapping circuitry, NMI logic, ISA refresh address generation, and PCI/ISA bus arbitration circuitry together onto the same device.  The PIIX comes in a 208 pin QFP package.

Detailed information on the PCIset is available in the Intel 82430 PCISet data sheet.

### IDE SUPPORT

The Advanced ZP ED baseboard provides two independent high performance bus-mastering PCI IDE interfaces capable of supporting PIO Mode 3 and Mode 4 devices for up to 16 MB/sec transfers.  Support for Type F DMA and AT API devices are provided in the system BIOS.  The system BIOS also supports Logical Block Addressing (LBA) and ECHS on both IDE interfaces.  When used in conjunction with a driver provided by Intel for the Windows95™ environment, the IDE interface operates as a PCI bus master for optimum performance in a multi-tasking environment.

## National Semiconductor 87306 Super I/O Controller

Control for the integrated serial ports, parallel port, floppy drive, RTC and keyboard controller is incorporated into a single component, the National Semiconductor 87306.  This component provides:

·	Two NS16C550-compatible UARTs with send/receive 16 byte FIFO

- Support for an IrDA compliant Infra Red interface

·	Multi-mode bi-directional parallel port

- Standard mode; IBM and Centronics compatible

- Enhanced Parallel Port (EPP) with BIOS/Driver support

- High Speed mode; Enhanced Capabilities Port (ECP) compatible

·	Industry standard floppy controller with 16 byte data FIFO (2.88 MB floppy support)

·	Integrated Real Time Clock accurate within +/- 13 minutes/yr

·	Integrated 8042 compatible keyboard controller

Configuration of these interfaces is possible via the CMOS Setup program that can be invoked during boot-up.  The serial ports can be enabled as COM1, COM2, IrDA, or disabled.  The parallel port can be configured as normal, extended , or disabled.   The floppy interface can be configured for 1.2 MB 5¼” media or for 720 kB, 1.2 MB, 1.44 MB, or 2.88 MB 3½” media.  Header pins located near the back of the board allow cabling to use these interfaces

### Keyboard/mouse  Interface

PS/2 keyboard/mouse connectors are located on the back panel side of the baseboard (AT keyboard is optional).  The 5V lines to these connectors are protected with a PolySwitch* circuit which acts much like a self-healing fuse, re-establishing the connection after an over-current condition is removed.  While this device eliminates the possibility of having to replace a fuse, care should be taken to turn off the system power before installing or removing a keyboard or mouse.  The system BIOS can detect, and correct mis-plugged keyboards and mice.

The integrated 8042 microcontroller contains the AMI Megakey keyboard/mouse controller code which, besides providing traditional keyboard and mouse control functions, supports Power-On/Reset (POR) password protection.  The POR password can be defined by the user via the Setup program.  The keyboard controller also provides for the following "hot key" sequences:

	CTRL-ALT-DEL:  System software reset.  This sequence performs a software reset of the system by jumping to the beginning of the BIOS code and running the POST operation.

	CTRL-ALT+ and CTRL-ALT-:  Turbo mode selection.  CTRL-ALT- sets the system for de-turbo mode, emulating  an 8Mhz AT,  and CTRL-ALT+ sets the system for turbo mode.  Changing the Turbo mode may be prohibited by an operating system, or when the CPU is in Protected mode or  virtual 86 mode under DOS.

·	CTRL-ALT-<defined in setup>:  Power down and coffee-break key sequences take advantage of the SMM features of the PentiumTM Processor to greatly reduce the system’s power consumption while maintaining the responsiveness necessary to service external interrupts.

### Real Time Clock, CMOS RAM and Battery

The integrated Real Time Clock (RTC) is DS1287 and MC146818 compatible and provides a time of day clock, 100-year calendar with alarm features and is accurate to within 13 minutes/year.  The RTC can be set via the BIOS SETUP program.  The RTC also supports 242-byte battery-backed CMOS RAM in two banks which is reserved for BIOS use.  The CMOS RAM can be set to specific values or cleared to the system default values using the BIOS SETUP program.  Also, the CMOS RAM values can be cleared to the system defaults by using a configuration switch on the baseboard.

An external coin-cell style battery provides power to the RTC and CMOS memory.  The battery has an estimated lifetime of seven years and is socketed for easy replacement.  Refer to Appendix A for battery replacement details.

### IrDA (Infra-Red) Support

Serial port 2 can be configured to support an IrDA module via a 5 pin header connector.  Once configured for IrDA, the user can transfer files to/from portable devices such as laptops, PDA’s and printers using application software such as LapLink.  The IrDA specification provides for data transfers at 115kbps from a distance of 1 meter.

A 5-pin header is provided to allow connection to a Hewlett Packard HSDSL-1000 compatible Infra-red transmitter/receiver.

## System BIOS

The Advanced ZP ED baseboard uses an American Megatrends Incorporated (AMI) Pentium ROM BIOS, which is stored in Flash EEPROM and easily upgraded using a floppy disk-based program.  BIOS upgrades will be downloadable from iPAN, the electronic bulletin board service of IntelTechDirectä.  In addition to the AMIBIOS, the Flash EEPROM also contains the Setup utility, Power-On Self Tests (POST), update recovery code, and the PCI auto-configuration utility.  This baseboard supports system BIOS shadowing, allowing the BIOS to execute from 32-bit on-board write-protected DRAM.

The BIOS displays a sign-on message during POST identifying the type of BIOS and a five-digit revision code.  As an example the Beta BIOS for the Advanced ZP will be 0.06.01.BS0. As BIOS updates occur the revision number will increase to 0.07.01.BS0, and so on.

Information on BIOS functions can be found in the IBM PS/2 and Personal Computer BIOS Technical Reference published by IBM, and the ISA and EISA Hi-Flex AMIBIOS Technical Reference published by AMI.  Both manuals are available at most technical bookstores.

### FLASH Implementation

The Intel 28F001BXT 1 Mb FLASH component is organized as 128K x 8 (128 KB). The Flash device is divided into five areas, as described in Table 1.

| System Address |  | FLASH Memory Area |
| --- | --- | --- |
| F0000H | FFFFFH | 64 KB Main BIOS |
| EE000H | EFFFFH | 8 KB Boot Block (Not FLASH erasable) |
| ED000H | EDFFFH | 4 KB Plug and Play ESCD Storage Area |
| EC000H | EDFFFH | 4 KB OEM LOGO Area |
| E0000H | EBFFFH | 48 KB System BIOS Reserved |

Table 1. Flash Memory Organization

The FLASH device resides in system memory in two 64 KB segments starting at E0000H, and can be mapped two different ways, depending on the mode of operation.  In Normal Mode, address line A16 is inverted, setting the E000H and F000H segments so that the BIOS is organized as shown in the system address column above.  Recovery mode removes the inversion on address line A16, swapping the E000H and F000H segments so that the 8 KB boot block resides at FE000H where the CPU expects the bootstrap loader to exist.  This mode is only necessary in the unlikely event that a BIOS upgrade procedure is interrupted, causing the BIOS area to be left in an unusable state.  For information on recovering the BIOS in the event of a catastrophic failure, refer to Appendix E.

### BIOS Upgrades

FLASH memory makes distributing BIOS upgrades easy.  A new version of the BIOS can be installed from a diskette.  BIOS upgrades will be available as downloadable files on the iPAN bulletin board.

The disk-based Flash upgrade utility, FMUP.EXE, has three options for BIOS upgrades:

· 	The Flash BIOS can be updated from a file on a disk;

· 	The current BIOS code can be copied from the Flash EEPROM to a disk file as a backup in the event that an upgrade cannot be successfully completed; or

· 	The BIOS in the Flash device can be compared with a disk file to ensure the system has the correct BIOS version.

The upgrade utility ensures the upgrade BIOS extension matches the target system to prevent accidentally installing a BIOS for a different type of system. The default setting is to allow BIOS upgrades.  A recovery jumper is provided to allow recovery in the unlikely event of an unsuccessful BIOS upgrade.  The jumper forces the ROM decode to access a 32 KB block of write protected recovery code in the Flash device.

### setup Utility

The ROM-based Setup utility allows the configuration to be modified without opening the system for most basic changes.  The Advanced ZP ED baseboard also incorporates many new capabilities into the Flash EEPROM, including:

·	Auto configuration of IDE hard disks (manual selection of the drive type is no longer required).

·	Support for two IDE disk interfaces (primary and secondary) providing access to 4 ATAPI IDE devices including CD-ROM readers.

·	An option allowing the user to assign a block of addresses below the 1 MB boundary as unshadowed to allow for expansion card ROM which can cause timing problems when shadowed and cached.  The default setting assigns this area as shadowed .

·	Interrupts IRQ9, IRQ10 and IRQ11 (typically used for PCI) may be assigned to add-in card ISA adapters, thereby informing the PCI configuration utility which interrupts not to use.

The Setup utility is accessible only during the Power-On Self Test, POST, by pressing the <DEL> or <F1> key after the POST memory test has begun and before boot begins.  A prompt may be enabled that informs users to press the <F1> key to access Setup.  A baseboard jumper can be set to prevent user access to Setup for security purposes.  Setup options are detailed in Appendix C.

### PCI Auto-configuration

The PCI auto-configuration utility operates in conjunction with the system Setup utility to allow the insertion and removal of PCI cards to the system without user intervention.  When the system is turned on after adding a PCI add-in card, the BIOS automatically configures interrupts, DMA channels, I/O space, and other parameters.  The user does not have to configure jumpers or worry about potential resource conflicts.  Because PCI cards use the same interrupt resources as ISA cards, the user must specify the interrupts used by ISA add-in cards in the Setup utility.  The PCI Auto-Configuration function complies with version 2.0G of the PCI BIOS specification.

### ISA Plug & Play

The BIOS incorporates ISA Plug and Play capabilities conforming to version 1.0a of the Plug-n-Play specification.  This will allow auto-configuration of Plug and Play ISA cards, and resource management for legacy ISA cards, when used in conjunction with the ICU, ISA Configuration Utility.  Copies of the ISA Plug and Play specification may be obtained via FaxBack (800) 628-2283, or via CompuServe  by typing Go PlugPlay.

### Shadow Memory

Memory from C8000-DFFFF is non-shadowed.  This is a change from previous Intel products using AMI based BIOS.  This may have a slight adverse affect on the performance of some ISA legacy (non Plug and Play) cards.

### Power Management

The Advanced/ZP BIOS supports power management via System Management Mode (SMM) interrupts to the CPU and Advanced Power Management (APM Ver. 1.1).  The APM capabilities will allow the system to be put into a power managed Stand By state by:

1.  Pressing a sleep/resume button on the front of the chassis

2.  Entering an user configurable hot-key sequence on the keyboard

3. Expiration of the user configurable system inactivity timer

Stand By mode reduces power consumption to Energy Star levels by utilizing the power saving capabilities of the Pentium™ processor and turning off DPMS compliant monitors and hard drives.  Stand By mode also allows the system to be responsive to external interrupts such as in-coming FAX’s or network messages.  Add-in cards supplied with APM-aware (v1.1) drivers can also be put into a power managed state for further energy savings.

The Advanced/ZP also is supplied with a power management utility for Windows called Powerman.  In addition to the features mentioned above, Powerman provides support for the system to wake up at scheduled times (such as delayed faxes), and support for a user prompted password upon wake up.  Powerman is supplied on the foundation software CD that is shipped with the Advanced/ZP.

### Flash LOGO Area

Advanced ZP ED supports a 4 KB programmable FLASH user area located at EC000-ECFFF.  An OEM may use this area to display a custom logo.  The Advanced ZP ED BIOS accesses the user area just after completing POST.

### Security features

#### Administrative Password

The administrative password protects several sensitive Setup options from being viewable to a user unless the password is entered.  These sensitive fields are viewable unless the administrative password is set.

#### BIOS Password

A BIOS password feature provides security during the boot process.  A password can be set using the Setup utility, and must be entered prior to peripheral boot or keyboard/mouse operation. To enable, disable, or change the password, refer to the Setup program options in Appendix C.

If the password is forgotten, it can be cleared by turning off the system and setting the "password clear" jumper to the clear position.

#### Setup Enable Switch

A baseboard configuration switch controls access to the BIOS Setup utility.  By setting switch SW5 to the ON position, the user is prevented from accessing the Setup utility during the Power-On Self Test or at any other time.  The message prompting the user to press <F1> to enter setup is also disabled

## Connectors

### Front panel connections

The Advanced ZP ED baseboard provides header connectors to support functions typically located on the chassis bezel:

- System Reset

- Power LED

- Keyboard Lock

- Hard Drive activity LED

- Turbo LED

- System Speaker

- Secondary system Fan

- Infra-Red (IrDA) port

- Sleep/Resume

Figure 3: Front Panel Connectors

#### Sleep/Resume

This two pin header, when connected to a momentary switch, can be used to put the system into a power managed state (Stand By) that will reduce the system’s power consumption.  If the system is in Stand By mode and the switch is closed, the system will instantly “wake up” or Resume full system activity.  When used with a power supply with a high efficiency rating, the Advanced ZP ED is easily capable of reducing the system power to below EPA Energy Star requirements.  The function of the Sleep/Resume button can also be achieved via the keyboard with a hot key sequence programmable in setup.  This connector can  be found in two locations on the Advanced ZP baseboard to minimize system cable requirements.

#### Infra-Red (IrDA) connector

Serial port 2 can be configured to support an IrDA module via a 5 pin header connector.  Once configured for IrDA, the user can transfer files to/from portable devices such as laptops, PDA’s and printers using application software such as LapLink.  The IrDA specification provides for data transfers at 115kbps from from a distance of 1 meter.

#### Speaker

The external speaker provides error beep code information during the Power-On Self Test if the system cannot use the video interface.  See Appendix D - AMIBIOS Error Messages and Beep Codes for more information about error beep codes.

### Back panel connections

The back panel provides external access to PS/2 style keyboard and mouse connectors integrated on the Advanced ZP ED baseboard.  A single AT-style keyboard connector is optional.   Figure 4a shows the general location of the PS/2 keyboard and mouse connectors and Figure 4b shows the general location of the AT style keyboard connector.

Figure 4a. Advanced ZP ED PS/2 Style Back panel

Figure 4b. Advanced ZP ED AT Style Back panel

### I/O connections

The baseboard contains shroud-less stake pin header connections for cabling the serial, parallel,  floppy, and IDE interfaces.  Figure 5 shows the locations of these connectors and the orientation of pin 1 on each.

Figure 5. I/O Connections

## Power Consumption

Table 2 lists the current used by system resources in a configuration which includes 8 MB of DRAM.  Table 3 lists the typical power consumed by the same configuration.  Note that the 3.3 volts used to drive the CPU and core logic is derived from an on-board voltage regulator from the +5 volt source. This information is preliminary and is provided only as a guide for calculating approximate total system power usage with additional resources added.

### Current

| DC Voltage | 75 MHz Typical Current* | 90 MHz Typical Current* |
| --- | --- | --- |
| +5V | 3.3 amps | 3.45 amps |
| -5V | 20 milliamps | 20 milliamps |
| +12V | 75 milliamps | 75 milliamps |
| -12V | 10 milliamps | 10 milliamps |

Table 2. Advanced ZP ED Current Requirements (Preliminary)

*(measured with 8 MB DRAM, VGA controller and Floppy Drive while sitting at DOS prompt)

### Watts

| Resource | Typical Power* | Stand By Power* |
| --- | --- | --- |
| Advanced ZP ED baseboard, 8 MB, 256 KB cache, 3½” floppy drive, 540 MB hard drive | 32 Watts | 21 Watts |

Table 3. Current Use by System Resources (Preliminary)

*(true power measured from the wall with a 65% efficient power supply)

# Appendix A - User-Installable Upgrades

## SYSTEM MEMORY

Table A-1 shows the possible memory combinations.  The Advanced ZP ED will support both Fast Page DRAM or EDO DRAM  SIMMs, but they cannot be mixed within the same memory bank.  If Fast Page DRAM and EDO DRAM  SIMMs are installed in separate banks, each bank will be optimized for maximum performance.   Parity generation and detection is NOT supported.  SIMM requirements are 70ns Fast Page Mode or 60nS EDO DRAM with tin-lead connectors.

| SIMM 1,2 (Bank 0) SIMM Type (Size) | SIMM 3,4 (Bank 1) SIMM Type (Size) | Total System Memory |
| --- | --- | --- |
| Empty | 1M X 32 (4 MB) | 8 MB |
| Empty | 2M X 32 (8 MB) | 16 MB |
| Empty | 4M X 32 (16 MB) | 32 MB |
| Empty | 8M X 32 (32 MB) | 64 MB |
| 1M X 32 (4 MB) | Empty | 8 MB |
| 1M X 32 (4 MB) | 1M X 32 (4 MB) | 16 MB |
| 1M X 32 (4 MB) | 2M X 32 (8 MB) | 24 MB |
| 1M X 32 (4 MB) | 4M X 32 (16 MB) | 40 MB |
| 1M X 32 (4 MB) | 8M X 32 (32 MB) | 72 MB |
| 2M X 32 (8 MB) | Empty | 16 MB |
| 2M X 32 (8 MB) | 1M X 32 (4 MB) | 24 MB |
| 2M X 32 (8 MB) | 2M X 32 (8 MB) | 32 MB |
| 2M X 32 (8 MB) | 4M X 32 (16 MB) | 48 MB |
| 2M X 32 (8 MB) | 8M X 32 (32 MB) | 80 MB |
| 4M X 32 (16 MB) | Empty | 32 MB |
| 4M X 32 (16 MB) | 1M X 32 (4 MB) | 40 MB |
| 4M X 32 (16 MB) | 2M X 32 (8 MB) | 48 MB |
| 4M X 32 (16 MB) | 4M X 32 (16 MB) | 64 MB |
| 4M X 32 (16 MB) | 8M X 32 (32 MB) | 96 MB |
| 8M X 32 (32 MB) | Empty | 64 MB |
| 8M X 32 (32 MB) | 1M X 32 (4 MB) | 72 MB |
| 8M X 32 (32 MB) | 2M X 32 (8 MB) | 80 MB |
| 8M X 32 (32 MB) | 4M X 32 (16 MB) | 96 MB |
| 8M X 32 (32 MB) | 8M X 32 (32 MB) | 128 MB |

Table A-1. Possible SIMM Memory Combinations

## RTC battery

The battery can be replaced with a Sanyo CR2032, or equivalent,  coin cell lithium battery.  This battery has a 220 mAh rating.

# Appendix B - Jumpers and Switches

Figure B-1. Jumper locations and settings (default settings shown)

## External CPU Clock speed (50/60 MHz) - Switches 7 & 8

This jumper sets the CPU's external operating frequency at 50, 60 MHz..  Default setting is 50 MHz.

| Frequency | Switch 7 | Switch 8 |
| --- | --- | --- |
| 50 MHz | OFF | OFF |
| 60 MHz | ON | OFF |

## internal CPU Bus clock speed - switch 6

Sets the internal processor speed to either 3/2 or 2 times the external CPU clock speed.  Switch 6=OFF for 3/2, Switch 6=ON for 2 times. Default setting is 3/2,  (Switch 6 = OFF).  Note that the 2 times (SW6=ON) setting is reserved for possible future processor speed upgrades.

## Setup Disable - switch 5

Allows access to CMOS Setup Utility to be disabled by setting switch 5 to the ON position.  Default is for access to setup to be enabled (switch 5 = OFF)

## Clear CMOS - switch 4

Allows CMOS settings to be reset to default values by moving switch 4 to the ON position and turning the system on.  The system should then be turned off and switch 4 should be returned to the OFF position to restore normal operation.  This procedure should be done whenever the system BIOS is updated.  Default setting is SW4=OFF

## Password clear - switch 3

Allows system password to be cleared by moving switch 3 to the ON position and turning the system on. The system should then be turned off and switch 3 should be returned to the OFF position to restore normal operation.  This procedure should only be done if the user password has been forgotten.  Default setting is SW3=OFF

## Processor Voltage regulation - switch 2

Sets the system board processor voltage regulation to be VR or VRE compliant.  When SW2 should be set to OFF for VR type processors and set to ON for VRE type processors.   The default setting for SW2 is OFF.

## ISA BUS SPEED - switch 1

Sets the ISA bus speed to either 1/4 or 1/3 of the PCI bus speed to best maximize system performance for either 75 or 90 MHz processor speeds.   When SW1=ON the ISA bus speed will be 1/4 of the PCI bus speed  and when SW1=OFF the ISA bus speed will be 1/3 the PCI bus speed.  The ISA bus speed is derived from the PCI bus speed which is one half of  the external processor clock frequency.   The position of SW1 will set the ISA bus speed to either Compatible or Enhanced for 90 Mhz processors and will leave the ISA bus speed Compatible for 75 Mhz processor speed.  For 90 Mhz processor speed, if SW1 is set to Compatible (SW1=ON), the speed will fall within the limits defined by the IBM AT Technical Reference (6-8.33 Mhz). If set to Enhanced (SW1=OFF), the speed will be greater than the maximum defined by the IBM AT Technical Reference manual.  SW1 should always be set to the ON position when a 75 Mhz processor speed is used for maximum ISA bus performance.  Modern ISA cards can operate with the enhanced speeds, however some older cards can experience difficulties.  The actual value of the Bus Clock when set to Compatible or Enhanced is dependent upon the setting of the external processor frequency.  The following table describes the ISA bus speeds for the 75 and 90 Mhz processor speeds.

| Internal Processor Frequency | PCI bus Frequency | SW1=OFF (1/3) | SW1=ON (1/4) |
| --- | --- | --- | --- |
| 75 MHz | 25 MHz | 8.33 MHz | 6.25 MHz |
| 90 MHz | 30 MHz | 10.0 MHz | 7.50 MHz |

## Recovery boot enable - J1K1

Allows the system to boot in the event the system BIOS has been corrupted by moving the jumper from the default position of 1-2 to the 2-3 position.  The procedure for recovery boot is described in detail in Appendix I.

# Appendix C - I/O Map

| Address (hex) | Size | Description |
| --- | --- | --- |
| 0000 - 000F | 16 bytes | PIIX - DMA 1 |
| 0020 - 0021 | 2 bytes | PIIX - Interrupt Controller 1 |
| 0040 - 0043 | 4 bytes | PIIX - Timer 1 |
| 0048 - 004B | 4 bytes | PIIX - Timer 2 |
| 0060 | 1 byte | Keyboard Controller Data Byte |
| 0061 | 1 byte | PIIX - NMI, speaker control |
| 0064 | 1 byte | Kbd Controller, CMD/STAT Byte |
| 0070, bit 7 | 1 bit | PIIX - Enable NMI |
| 0070, bits 6:0 | 7 bits | PIIX - Real Time Clock, Address |
| 0071 | 1 byte | PIIX - Real Time Clock, Data |
| 0078 | 1 byte | Reserved - Brd. Config. |
| 0079 | 1 byte | Reserved - Brd. Config. |
| 0080 - 008F | 16 bytes | PIIX - DMA Page Register |
| 00A0 - 00A1 | 2 bytes | PIIX - Interrupt Controller 2 |
| 00C0 - 00DE | 31 bytes | PIIX - DMA 2 |
| 00F0 | 1 byte | Reset Numeric Error |
| 0170 - 0177 | 8 bytes | Secondary IDE Channel |

| Address (hex) | Size | Description |
| --- | --- | --- |
| 01F0 - 01F7 | 8 bytes | Primary IDE Channel |
| 0278 - 027B | 4 bytes | Parallel Port 2 |
| 02F8 - 02FF | 8 bytes | On-Board Serial Port 2 |
| 0376 | 1 byte | Sec IDE Chan Cmd Port |
| 0377 | 1 byte | Sec IDE Chan Stat Port |
| 0378 - 037F | 8 bytes | Parallel Port 1 |
| 03BC - 03BF | 4 bytes | Parallel Port x |
| 03E8 - 03EF | 8 bytes | Serial Port 3 |
| 03F0 - 03F5 | 6 bytes | Floppy Channel 1 |
| 03F6 | 1 bytes | Pri IDE Chan Cmnd Port |
| 03F7 (Write) | 1 byte | Floppy Chan 1 Cmd |
| 03F7, bit 7 | 1 bit | Floppy Disk Chg Chan 1 |
| 03F7, bits 6:0 | 7 bits | Pri IDE Chan Status Port |
| 03F8 - 03FF | 8 bytes | On-Board Serial Port 1 |
| LPT + 400h | 8 bytes | ECP port, LPT + 400h |
| 0CF8-0CFB* | 4 bytes | PCI Config Address Reg |
| 0CFC-0CFF* | 4 bytes | PCI Config Data Reg |
| FF00-FF07 | 8 bytes | IDE Bus Master Reg. |

Table G-1. Advanced ZP ED I/O Address Map

*  Only accessible after PCI configuration space is enabled.

# Appendix D - Memory Map

| Address Range (Decimal) | Address Range (hex) | Size | Description |
| --- | --- | --- | --- |
| 1024K-131072K | 100000-8000000 | 127M | Extended Memory |
| 960K-1023K | F0000-FFFFF | 64K | AMI System BIOS |
| 952K-959K | EE000-EFFFF | 8K | FLASH Boot Block (Available as UMB) |
| 948K-951K | ED000-EDFFF | 4K | ECSD (Plug and 'Play configuration area) |
| 944-947K | EC000-ECFFF | 4K | OEM LOGO (available as UMB) |
| 896K-943K | E0000-EBFFF | 48K | BIOS RESERVED (Currently available as UMB) |
| 800-895K | C8000-DFFFF | 96K | Available HI DOS memory (open to ISA and PCI bus) |
| 640K-799K | A0000-C7FFF | 160K | Available HI DOS memory (open to ISA and PCI bus) |
| 639K | 9FC00-9FFFF | 1K | Extended BIOS Data (moveable by QEMM, 386MAX) |
| 512K-638K | 80000-9FBFF | 127K | Extended conventional |
| 0K-511K | 00000-7FFFF | 512K | Conventional |

Table F-1. Advanced/ZP Memory Map

The table above details the Advanced/ZP memory map.  The ECSD area from ED000-EDFFF is not available for use as an Upper Memory Block (UMB) by memory managers.  The area from E0000-EBFFF is currently not used by the BIOS and is available for use as UMB by memory managers.  Parts of this area may be used by future versions of the BIOS to add increased functionality.

I/O Port 78 is reserved for BIOS use.  Port 79 is a read only port, the bit definitions are shown below.

| Bit # | Description | Bit = 1 | Bit = 0 |
| --- | --- | --- | --- |
| 0 | Internal CPU Clock Freq. (Switch 6) | 3/2x | 2x |
| 1 | No Connect |  |  |
| 2 | No Connect |  |  |
| 3 | External CPU clock (Switch 7) |  |  |
| 4 | External CPU clock (Switch 8) |  |  |
| 5 | Setup Disable (Switch 5) | Enable access | Disable access |
| 6 | Clear CMOS (Switch 4) | Keep values | Clear values |
| 7 | Password Clear (Switch 3) | Keep password | Clear password |

# Appendix E - PCI Configuration Space Map

The Triton chipset uses Configuration Mechanism 1 to access PCI configuration space.  The PCI Configuration Address register is a 32-bit register located at CF8h, the PCI Configuration Data register is a 32-bit register located at CFCh.  These registers are only accessable by full DWORD accesses.  The table below lists the PCI bus and device numbers used by the baseboard.

| Bus Number (hex) | Dev Number (hex) | Func. Number (hex) | Description |
| --- | --- | --- | --- |
| 00 | 00 | 00 | Intel 82437FX (TSC) |
| 00 | 07 | 00 | Intel 82371FB (PIIX) PCI/ISA bridge |
| 00 | 07 | 01 | Intel 82371FB (PIIX) IDE Bus Master |
| 00 | 0E |  | PCI Expansion Slot 1 |
| 00 | 0F |  | PCI Expansion Slot 2 |
| 00 | 10 |  | PCI Expansion Slot 3 |

Table E-1. Advanced/ZP PCI Config. Space Map

# Appendix F - Interrupts & DMA Channels

| IRQ | System Resource |
| --- | --- |
| NMI | Parity Error |
| 0 | Reserved, Interval Timer |
| 1 | Reserved, Keyboard buffer full |
| 2 | Reserved, Cascade interrupt from slave PIC |
| 3 | Serial Port 2 |
| 4 | Serial Port 1 |
| 5 | Parallel Port 2 |
| 6 | Floppy |
| 7 | Parallel Port 1 |
| 8 | Real Time Clock |
| 9 | User available |
| 10 | User available |
| 11 | User available |
| 12 | On-board Mouse Port |
| 13 | Reserved, Math coprocessor |
| 14 | Primary IDE if enabled, else available to user |
| 15 | Secondary IDE if enabled, else available to user |

Table F-1.Advanced ZP ED Interrupts

| DMA | Data Width | System Resource |
| --- | --- | --- |
| 0 | 8- or 16-bits | Open |
| 1 | 8- or 16-bits | Open - Normally used for LAN |
| 2 | 8- or 16-bits | Floppy |
| 3 | 8- or 16-bits | Parallel Port |
| 4 |  | Reserved - Cascade channel |
| 5 | 16-bits | Open |
| 6 | 16-bits | Open |
| 7 | 16-bits | ISA IDE |

Table H-2. Advanced ZP ED DMA Map

# Appendix F - Connectors

## Power Supply Connectors

## Primary Power (j9j1)

| Pin | Name | Function |
| --- | --- | --- |
| 1 | PWRGD | Power Good |
| 2 | +5 V | + 5 volts Vcc |
| 3 | +12 V | + 12 volts |
| 4 | -12 V | - 12 volts |
| 5 | GND | Ground |
| 6 | GND | Ground |
| 7 | GND | Ground |
| 8 | GND | Ground |
| 9 | -5 V | -5 volts |
| 10 | +5 V | + 5 volts Vcc |
| 11 | +5 V | + 5 volts Vcc |
| 12 | +5 V | + 5 volts Vcc |

## auxiliary (3.3V) PCI Power (j6G1)

| Pin | Name | Function |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | GND | Ground |
| 3 | GND | Ground |
| 4 | +3.3 V | + 3.3 volts |
| 5 | +3.3V | + 3.3 volts |
| 6 | +3.3 V | + 3.3 volts |

# Front Panel Connectors -(J2A1, J1B1)

## sleep/resume

| Pin | Signal Name |
| --- | --- |
| 1 | +5 V |
| 2 | Comatose |

## Infra-red

| Pin | Signal Name |
| --- | --- |
| 1 | +5 V |
| 2 | Key |
| 3 | IR_RX |
| 4 | Ground |
| 5 | IR_TX |

## Auxiliary 12V Fan Power

| Pin | Signal Name |
| --- | --- |
| 1 | Ground |
| 2 | +12 V (fused) |
| 3 | Ground |

## Speaker Connector

| Pin | Signal Name |
| --- | --- |
| 1 | SPKR_DAT |
| 2 | Key |
| 3 | SPKR_DAT onnect |
| 4 | Ground |

## Turbo LED

| Pin | Signal Name |
| --- | --- |
| 1 | PULL_UP_330 |
| 2 | LED_TURBO- |

## Hard Drive LED (Disk)

| Pin | Signal Name |
| --- | --- |
| 1 | PULL_UP_330 |
| 2 | Key |
| 3 | HD ACTIVE |
| 4 | PULL_UP_330 |

## Key lock/Power LED

| Pin | Signal Name |
| --- | --- |
| 1 | LED_PWR |
| 2 | Key |
| 3 | Ground |
| 4 | KEY LOCK |
| 5 | Ground |

## Reset Connector

| Pin | Signal Name |
| --- | --- |
| 1 | Ground |
| 2 | RESET |

# I/O Connectors

## pS/2 Keyboard & Mouse Ports (J8k2, j8k3)

| Pin | Signal Name |
| --- | --- |
| 1 | Clock |
| 2 | Data |
| 3 | No Connect |
| 4 | Ground |
| 5 | Vcc (fused) |

## Serial Ports (j7k1, j7k2)

| Pin | Signal Name |
| --- | --- |
| 1 | DCD |
| 2 | DSR |
| 3 | Serial In - (SIN) |
| 4 | RTS |
| 5 | Serial Out - (SOUT) |
| 6 | CTS |
| 7 | DTR |
| 8 | RI |
| 9 | GND |
| 10 | N.C. |

## iDE Connectors (j6h1, j6h2)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Reset IDE | 1 | 2 | Ground |
| Host Data 7 | 3 | 4 | Host Data 8 |
| Host Data 6 | 5 | 6 | Host Data 9 |
| Host Data 5 | 7 | 8 | Host Data 10 |
| Host Data 4 | 9 | 10 | Host Data 11 |
| Host Data 3 | 11 | 12 | Host Data 12 |
| Host Data 2 | 13 | 14 | Host Data 13 |
| Host Data 1 | 15 | 16 | Host Data 14 |
| Host Data 0 | 17 | 18 | Host Data 15 |
| Ground | 19 | 20 | Key |
| DRQ3 | 21 | 22 | Ground |
| I/O Write- | 23 | 24 | Ground |
| I/O Read- | 25 | 26 | Ground |
| IOCHRDY | 27 | 28 | BALE |
| DACK3- | 29 | 30 | Ground |
| IRQ14 | 31 | 32 | IOCS16- |
| Addr 1 | 33 | 34 | Ground |
| Addr 0 | 35 | 32 | Addr 2 |
| Chip Select 0- | 37 | 38 | Chip Select 1- |
| Activity | 39 | 40 | Ground |

## Parallel Port Connector (j9e1)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| STROBE- | 1 | 2 | AUTO FEED- |
| Data Bit 0 | 3 | 4 | ERROR- |
| Data Bit 1 | 5 | 6 | INIT- |
| Data Bit 2 | 7 | 8 | SLCT IN- |
| Data Bit 3 | 9 | 10 | Ground |
| Data Bit 4 | 11 | 12 | Ground |
| Data Bit 5 | 13 | 14 | Ground |
| Data Bit 6 | 15 | 16 | Ground |
| Data Bit 7 | 17 | 18 | Ground |
| ACJ- | 19 | 20 | Ground |
| BUSY | 21 | 22 | Ground |
| PE (Paper End) | 23 | 24 | Ground |
| SLCT | 25 | 26 | N.C. |

## floppy Connector (j9c1)

| Signal Name | Pin | Pin |  | Signal Name |
| --- | --- | --- | --- | --- |
| Ground | 1 | 2 |  | FDHDIN |
| Ground | 3 | 4 |  | Reserved |
| Key | 5 | 6 |  | FDEDIN |
| Ground | 7 | 8 |  | Index- |
| Ground | 9 | 10 |  | Motor Enable A- |
| Ground | 11 | 12 |  | Drive Select B- |
| Ground | 13 | 14 |  | Drive Select A- |
| Ground | 15 | 16 |  | Motor Enable B- |
| Ground | 17 | 18 | DIR- |  |
| Ground | 19 | 20 |  | STEP- |
| Ground | 21 | 22 |  | Write Data- |
| Ground | 23 | 24 |  | Write Gate- |
| Ground | 25 | 26 |  | Track 00- |
| Ground | 27 | 28 |  | Write Protect- |
| Ground | 29 | 30 |  | Read Data- |
| Ground | 31 | 32 |  | Side 1 Select- |
| Ground | 33 | 34 |  | Diskette Change- |

## ISA Connectors (j1g1, j2g1, j2g2, j3g1)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| GND | B1 | A1 | IOCHK- |
| RSTDRV | B2 | A2 | SD7 |
| Vcc | B3 | A3 | SD6 |
| IRQ9 | B4 | A4 | SD5 |
| -5V | B5 | A5 | SD4 |
| DRQ2 | B6 | A6 | SD3 |
| -12V | B7 | A7 | SD2 |
| 0WS- | B8 | A8 | SD1 |
| +12V | B9 | A9 | SD0 |
| GND | B10 | A10 | IOCHRDY |
| SMEMW- | B11 | A11 | AEN |
| SMEMR- | B12 | A12 | SA19 |
| IOW- | B13 | A13 | SA18 |
| IOR- | B14 | A14 | SA17 |
| DACK3- | B15 | A15 | SA16 |
| DRQ3 | B16 | A16 | SA15 |
| DACK1- | B17 | A17 | SA14 |
| DRQ1 | B18 | A18 | SA13 |
| REFRESH- | B19 | A19 | SA12 |
| SYSCLK | B20 | A20 | SA11 |
| IRQ7 | B21 | A21 | SA10 |
| IRQ6 | B22 | A22 | SA9 |
| IRQ5 | B23 | A23 | SA8 |
| IRQ4 | B24 | A24 | SA7 |
| IRQ3 | B25 | A25 | SA6 |
| DACK2- | B26 | A26 | SA5 |
| TC | B27 | A27 | SA4 |
| BALE | B28 | A28 | SA3 |
| Vcc | B29 | A29 | SA2 |
| OSC | B30 | A30 | SA1 |
| GND | B31 | A31 | SA0 |
|  | KEY | KEY |  |
| MEMCS16- | D1 | C1 | SBHE- |
| IOCS16- | D2 | C2 | LA23 |
| IRQ10 | D3 | C3 | LA22 |
| IRQ11 | D4 | C4 | LA21 |
| IRQ12 | D5 | C5 | LA20 |
| IRQ15 | D6 | C6 | LA19 |
| IRQ14 | D7 | C7 | LA18 |
| DACK0- | D8 | C8 | LA17 |
| DRQ0 | D9 | C9 | MEMR- |
| DACK5- | D10 | C10 | MEMW- |
| DRQ5 | D11 | C11 | SD8 |
| DACK6- | D12 | C12 | SD9 |
| DRQ6 | D13 | C13 | SD10 |
| DACK7- | D14 | C14 | SD11 |
| DRQ7 | D15 | C15 | SD12 |
| Vcc | D16 | C16 | SD13 |
| Master- | D17 | C17 | SD14 |
| GND | D18 | C18 | SD15 |

## PCI Connectors (j4g1, j5g1, j5g2)

| Signal Name | Pin | Pin | Signal Name |  | Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GND | A1 | B1 | -12V |  | AD16 | A32 | B32 | AD17 |
| +12V | A2 | B2 | No Connect |  | 3.3V | A33 | B33 | CBE2- |
| No Connect | A3 | B3 | GND |  | FRAME- | A34 | B34 | GND |
| No Connect | A4 | B4 | No Connect |  | GND | A35 | B35 | IRDY- |
| Vcc | A5 | B5 | Vcc |  | TRDY- | A32 | B32 | 3.3V |
| PCIINT3- | A6 | B6 | Vcc |  | GND | A37 | B37 | DEVSEL- |
| PCIINT1- | A7 | B7 | PCIINT2- |  | STOP- | A38 | B38 | GND |
| Vcc | A8 | B8 | PCIINT4- |  | 3.3V | A39 | B39 | PLOCK- |
| Reserved | A9 | B9 | No Connect |  | SDONE | A40 | B40 | PERR- |
| Vcc | A10 | B10 | Reserved |  | SBO- | A41 | B41 | 3.3V |
| Reserved | A11 | B11 | No Connect |  | GND | A42 | B42 | SERR- |
| GND | A12 | B12 | GND |  | PAR | A43 | B43 | 3.3V |
| GND | A13 | B13 | GND |  | AD15 | A44 | B44 | CBE1- |
| Reserved | A14 | B14 | Reserved |  | 3.3V | A45 | B45 | AD14 |
| SPCIRST- | A15 | B15 | GND |  | AD13 | A46 | B46 | GND |
| Vcc | A16 | B16 | PCLKE |  | AD11 | A47 | B47 | AD12 |
| AGNT- | A17 | B17 | GND |  | GND | A48 | B48 | AD10 |
| GND | A18 | B18 | REQA- |  | AD9 | A49 | B49 | GND |
| Reserved | A19 | B19 | Vcc |  | KEY | A50 | B50 | KEY |
| AD30 | A20 | B20 | AD31 |  | KEY | A51 | B51 | KEY |
| 3.3V | A21 | B21 | AD29 |  | CBEO- | A52 | B52 | AD8 |
| AD28 | A22 | B22 | GND |  | 3.3V | A53 | B53 | AD7 |
| AD26 | A23 | B23 | AD27 |  | AD6 | A54 | B54 | 3.3V |
| GND | A24 | B24 | AD25 |  | AD4 | A55 | B55 | AD5 |
| AD24 | A25 | B25 | 3.3V |  | GND | A56 | B56 | AD3 |
| AD22 (IDSEL) | A26 | B26 | CBE3- |  | AD2 | A57 | B57 | GND |
| 3.3V | A27 | B27 | AD23 |  | AD0 | A58 | B58 | AD1 |
| AD22 | A28 | B28 | GND |  | Vcc | A59 | B59 | Vcc |
| AD20 | A29 | B29 | AD21 |  | SREQ64- | A60 | B60 | SACK64- |
| GND | A30 | B30 | AD19 |  | Vcc | A61 | B61 | Vcc |
| AD18 | A31 | B31 | 3.3V |  | Vcc | A62 | B62 | Vcc |

# Appendix G - INTEL/AMIBIOS Setup

### Setup Overview

#### Introduction

This User’s Guide documents the BIOS Setup Utility.  It explains the overall organization of the utility, how to navigate through the utility, and individual fields and options.  This Setup Guide was written using Beta BIOS Ver. 1.07.BR0 as the basis, other BIOS versions may differ slightly from the descriptions listed below.

The BIOS Setup Utility is divided into 4 major menus:

Main Menu

Advanced Menu

Security Menu

Exit Menu

Each of the major menus occupy the left and center sections of the computer screen.  Selecting certain fields within the major menus will drop the user into sub-menus.  Displayed on the right side of the screen  is a Command Screen which describes the commands used to interact with the Setup Utility.  The following section documents these commands.  The subsequent four sections describe each of the main menus.  Sub-menus are documented within the main menus from which they are invoked.  In each section, the field which is being displayed is bold italic type.  If there are user selectable options for that field they are displayed in italic type.  Explanations and comments about the fields and options follows the menu items.

#### Command Screen Navigation

The right side of the Setup screen contains a list of  commands which are available to the user as they navigate through and interact with the Setup Utility.  These commands are displayed at all times, for each menu and sub-menu.

F1	Help

Help is provided for most of the fields.  It can not be invoked when parameter options are being displayed.

ESC	Back

The ESC key provides a mechanism for backing out of any field.  When the ESC key is entered from the top level of any field, the field options cease to be displayed.  When the ESC key is entered from the top level of any sub-menu, the parent menu is re-entered.  When the ESC key is entered from the top level of any major menu the following screen is displayed:

Exit Discarding Changes?

Press Enter to Continue

Press ESC to Abort

At this point pressing the Enter key will cause the user to leave the Setup Utility without saving any changes they might have made.  Pressing the ESC key will  bring them back to the top level of the current menu.

Enter	Select

The Enter key is used to activate sub-menus, change parameters and select parameter options.

Previous Item

The up arrow is used to move to the menu item above the current item..

¯	Next Item

The down arrow is used to move to the menu item below the current item.

«	Select Menu

The left and right arrow keys are used to move between the 4 major menus.

F5	Setup Defaults

The Setup Defaults key is used to set all Setup Parameters to their default values.  When this key is pressed the following message appears:

Load Setup Defaults?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then all Setup Parameters will be set to their default values.  If the ESC key is selected, then the user is returned to where they were before F5 was pressed without effecting any change.

F6	Previous Values

The Previous Values key is used to set all Setup Parameters to their previous values.  When this key is pressed the following message appears:

Discard Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then all Setup Parameters will be return to the values selected when Setup was entered.  If the ESC key is selected, then the user is returned to where they were before F6 was pressed without effecting any change.

F10	Save & Exit

The Save & Exit key is used to exit the Setup Utility saving the current state of all parameters   When this key is pressed the following message appears:

Exit Saving Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then the Setup Utility will be exited with all parameters set to their current values.  If the ESC key is selected, then the user is returned to where they were before F10 was pressed without effecting any change.

#### Main Menu Navigation

Each menu contains a number of parameters.  Most parameters can be changed by the user, although some are informative only.  Parameters which can be changed by the user are displayed in black, informative only parameters are displayed in blue.  Depending on the security option chosen and in effect (via password), a parameter can be either changeable or non-changeable.  A parameter which is non changeable due to insufficient security privilege or other reasons will be grayed out.  Parameters which have no effect based on the value of other parameters are not displayed.

### Main Menu

#### System Date

Month-Day-Year	Jan 01 1995	United States

Day-Month-Year	01 Jan 1995	Other locations

The System Date field specifies the current date.  When a language other than English is used, the "Other locations" format will be displayed.  Select the month from the pull-down menu provided and revise the day and year as needed.

#### System Time

Hour:Minute:Second

The System Time  field specifies the current time.

#### Floppy Options

When the cursor is placed the Floppy Options field and selected by hitting the Enter Key, the Floppy Options Sub-Menu appears, allowing parameters which affect the boot process to be set.

Floppy Options Sub-Menu

Floppy Detect

Floppy A: Type

Floppy B: Type

Disabled

360 Kb		5¼”

1.2 Mb		5¼”

720 Kb		3½”

1.44/1.25 Mb	3½”

2.88 Mb		3½”

Select the type from the pull-down menu.  The Floppy A: Type and Floppy B: Type fields specify the physical size and maximum supported media size of the floppy drives attached to the floppy interface.

#### Hard Disk 0:, Hard Disk 1: ....

Identification string returned from drive

Not Installed

The Hard Disk 0: through Hard Disk 3: fields report the presence and identification strings reported of to 4 physical PCI IDE drives.  When the cursor is placed on one of these fields and it is selected by hitting the Enter Key, the Hard Disk Sub-Menu appears allowing parameters which affect the drives to be set.

Disk drive assignments

Drive numbers will be assigned to physical drives as follows:

| Primary PCI IDE Interface Only |  |
| --- | --- |
| Master | 0: |
| Subordinate | 1: |
| Secondary PCI IDE Interface Only |  |
| Master | 0: |
| Subordinate | 1: |
| Both PCI IDE Interfaces Used Two Drives Used |  |
| Primary | 0: |
| Secondary | 1: |
| Both PCI IDE Interfaces Used Three Drives Used |  |
| Primary Master | 0: |
| Primary Subordinate | 1: |
| Secondary Master | 2: |
| Primary Master | 0: |
| Secondary Master | 1: |
| Secondary Subordinate | 2: |
| Both PCI IDE Interfaces Used Four Drives Used |  |
| Primary Master | 0: |
| Primary Subordinate | 1: |
| Secondary Master | 2: |
| Secondary Subordinate | 3: |

When drives have multiple partitions on them, partitions beyond the primary partition (logical partitions) will be assigned in ascending order, based on available drive numbers and other installed drives. Primary partitions are assigned first (across multiple drives), and then to logical partitions.  This means that when there are multiple drives containing logical partitions, the drive numbering scheme will result in discontinuous logical partitions on the same physical drive.

For example, if there is a “Drive X” with two partitions which is a Master on the Primary PCI IDE interface, a “Drive Y” with three partitions which is Subordinate on the Primary PCI interface, and a “Drive Z” with two partitions which is a Master on the Secondary PCI IDE interface the drive letters will be assigned as follows:

Drive X

Primary Partition		0:

Second Partition		3:

Drive Y

Primary Partition		1:

Second Partition		4:

Third Partition		5:

Drive Z

Primary Partition		2:

Second Partition		6:

##### Hard Disk Sub-Menu

Hard Disk Type

Auto Detected -- default

User Definable

Number of Cylinders

Number of Heads

Number of Sectors

If	Hard Disk Type = Auto Detected,	Then parameters reported by drive

If	Hard Disk Type = User Definable,	Then parameters entered by user

The Number of Cylinders, Number of Heads, and Number of Sectors  fields display what is reported by the drive if Hard Disk Type is set to Auto Detected.  If Hard Disk Type is set to User Configured then these fields are entered by the user. Hard Disk Type should be set to Auto Detected unless the drive was previously formatted on another computer with hard disk parameters different than those reported by the drive on this system.

Maximum Capacity

Value calculated from Cylinder, Head, and Sector inf. above

Initialization Timeout

Disabled

5 Seconds

10 Seconds -- default for drive 0.  Default  for drives 1-3, Disabled (0 seconds).

31 Seconds

Initialization Time-out selects the number of seconds the BIOS will wait to determine if an IDE drive is present, after initiating a transaction with the drive.  If a drive is present and responds to the BIOS before the Initialization Time-out period, then the BIOS will proceed, i.e. there is no boot-time penalty for a large value when a drive is present.  If a drive is not present, then the BIOS will wait the Initialization Time-out period before proceeding.  If a drive is present, but does not respond to the BIOS within the Initialization Time-out range, then the BIOS will not see the drive and it will not be accessible.  This value should be set to Disabled for any IDE drives not installed.  This parameter works in conjunction with the Hard Disk Pre-Delay parameter contained in the Boot Options sub-menu of the Main menu.

IDE Translation Mode

Standard CHS

Logical Block

Extended CHS

Auto Detected -- default

The IDE Translation Mode parameter controls the way in which the BIOS interacts with the drive in terms of drive geometry.  Proper choice is dependent upon the drive’s size, capabilities, and the operating system (OS) used.

Standard CHS is the translation mode that has been in use for years.  Its use limits IDE capacity to a maximum of 528 Megabytes regardless of the size of the drive used.

Logical Block mode overcomes the 528 MB maximum size limitation imposed by Standard CHS mode. It should be used only when the drive supports LBA (Logical Block Addressing), and the OS supports LBA, or uses the BIOS to access the disk.

Extended CHS mode also overcomes the 528 MB maximum size limitation imposed by Standard CHS mode.   It can be used with drives which are larger than 528 MB, but do not support LBA mode.

Auto Detected allows the BIOS to examine the drive and determine the optimal mode. The BIOS will first attempt Logical Block mode if it is supported by the drive and secondarily use Extended CHS.  If neither of the above two methods are possible, Standard CHS mode will be used.  Important NOTE: When set to Auto Detected the BIOS will detect what the drive is capable of, not the translation mechanism which was used to format the drive (if it had been previously formatted on another system).  If a drive is run in a mode other than the mode in which it was partitioned and formatted, unpredictable results may occur, including data loss.  To avoid this problem, always FDISK and format the drive with the IDE translation mode set to Auto Detected.

Different operating systems have different abilities regarding IDE Translation Modes.  The following information is intended as a guideline only, check with the appropriate Operating System vendor for specific details.

In general, UNIX operating systems (as currently implemented) do not support either LBA or ECHS and must utilize the Standard CHS method.  UNIX can support drives larger than 528 MB, but does so in its own way.

OS/2 2.1 & OS/2 Warp can support LBA, ECHS or Standard CHS methods.  Note that LBA support may require a switch setting on an OS/2 driver in order to operate in that mode. (*)

OS/2 2.0 & Novell Netware can support either ECHS or Standard CHS methods, but not LBA.

DOS & Windows can support LBA, ECHS or Standard CHS methods. If the "Fast 32-bit IDE" option of Windows is utilized, make sure to use the Standard CHS setting only, unless a version of WDCDRV.386 is being used which supports this advanced geometry.  You will get better performance NOT using “Fast 32-bit IDE” if using a drive which employs Mode 3/4 PIO on the PCI interface, unless using a version of WDCDRV.386 which supports these advanced modes.

Multiple Sector Setting

Disabled

4 Sectors/Block

8 Sectors/Block

Auto Detected -- default

The Multiple Sector Setting  field controls the number of sectors which are transferred by an IDE drive per interrupt generated.  If set to Disabled, then an interrupt will be generated for each sector transferred, even if the OS issues a multi-sector transfer.  If set to 4 Sectors/Block or 8 Sectors/Block then 4 or 8 sectors will be transferred per interrupt (for a request that is as large as, or larger than 4 or 8 sectors).  If Auto Detected is chosen, then n sectors will be transferred per interrupt, where n is the maximum value supported by the drive.  If a drive does not support multiple sectors/block then the BIOS will ignore this setting.  If the drive does not support the specific number (4 or 8) of sectors chosen, then the BIOS will default to the largest number of sectors smaller than the chosen value supported by the drive.

Fast Programmed I/O Modes

Auto Detected -- default

Disabled

The Fast Programmed I/O Modes field controls the speed in which Programmed I/O (PIO) transfers occur on the PCI IDE interface.  If set to Disabled, transfers occur at an non-optimized (Mode 0) speed.  If set to Auto Detected, transfers occur at the rate which the drive reports as its maximum (up to and including the maximum mode 4 timing).  This field should only be set to Disabled if a drive is being used which incorrectly reports an ability to function with advanced timings.

#### Language

English (US)

..

..

The Language field controls the language of the text strings used in Setup and the BIOS.  Any installed language will appear as an option in it’s native language.  Only installed languages will appear as options.  If no other languages are installed, this parameter can not be changed.

#### Boot Options

When the cursor is placed the Boot Options field and selected by hitting the Enter Key, the Boot Options Sub-Menu appears allowing parameters which affect the boot process to be set.

Boot Options Sub-Menu

Boot Sequence

C: First, Then A:

A: First, Then C: -- default

C: Only

A: Only

The field controls the order in which the BIOS attempts to boot from the hard and floppy drives.

System Cache

Disabled

Enabled -- default

The System Cache field controls both the processor (L1) and chipset (L2) caches.  Setting System Cache Disabled will hurt performance, but might be required when running software which utilizes software timing loops and needs to be slowed down to execute properly.

Boot Speed

Turbo -- default

Deturbo

The Boot Speed field controls the system CPU cycles.  De-turbo mode slows the CPU by disabling system cache.  It does not reduce CPU speed to 4.77 MHz or 8 MHz AT class machines.

Num Lock

Off -- default

On

The Num Lock field controls whether the keyboard Num Lock function is turned on or off when the system is booted.

Setup Prompt

Disabled

Enabled -- default

The Setup Prompt field controls whether the “Press <F1> Key To Enter Setup” message is displayed during POST (Power On Self Test).  It does not control access, just the message.  Access is controlled via the jumper on the baseboard.  When the jumper is set to disable access to Setup, the Setup Prompt message will not appear.

Hard Disk Pre-Delay

Disabled

1 Second

2 Seconds

3 Seconds -- default

4 Seconds

5 Seconds

6 Seconds

7 Seconds

The Hard Disk Pre-Delay provides a mechanism which delays the BIOS’ attempt to initialize the first IDE drive in the system.  It is necessary because certain drives take longer than others to “spin up” and might hang if they are accessed too soon after being powered up or reset.  The delay is only honored for the first drive accessed.  This parameter works in conjunction with the Initialization Time-out parameter contained in the Hard Disk Sub-menu of the Main menu.  The two are added together.

Typematic Rate Programming

Default - default

Override

Typematic Rate Delay

250 msec -- default

500 msec

750 msec

1000 msec

Typematic Rate

6 char/sec-- default

8 char/sec

10 char/sec

12 char/sec

15 char/sec

20 char/sec

24 char/sec

30 char/sec

The Typematic Rate Delay field controls how long it takes for a key pressed and held on the keyboard to begin repeating.  This affects all character keys as well as cursor and function keys.  The Typematic Rate controls the speed at which characters repeat when you hold down a keyboard key.  The higher the number, the faster the repeat.  This also includes cursor and function keys.

#### Video Mode

The Video Mode  field is informative only.

#### Mouse

Installed

Not Installed

The Mouse  field is informative only.  This message will appear if the system supports a PS/2 style mouse and a PS/2 style mouse is not connected to the PS/2 port.

#### Base Memory

512

640

This Base Memory field is informative only.   There is another Base Memory field which can be changed in the Advanced Chipset Configuration Sub-Menu of the Advanced menu.

#### Extended Memory

The Extended Memory field is informative only and shows the amount of system memory above 1 megabyte.

### Advanced Menu

#### Processor Type, Processor Speed, Cache Size

The Processor Type, Processor Speed, and Cache Size fields are informative only and can not be changed by the user.  Cache Size refers to the amount of L2 secondary cache installed on the system and will be displayed whether or not the cache is disabled in the Boot Options Sub-Menu.  This number does not include the CPU’s L1 (internal) cache.

#### Peripheral Configuration

When the cursor is placed the Peripheral Configuration field and selected by hitting the Enter Key, the Peripheral Configuration Sub-Menu appears, allowing parameters which affect the on-board I/O devices to be set.

Peripheral Configuration Sub-Menu

Configuration Mode

Auto -- default

Manual

The Configuration Mode fields controls whether on board resources are automatically configured, or manually configured by the User.  When set to Auto, on board system peripherals (PCI IDE drives, Floppy Drives, Serial Ports, Parallel Port) are automatically configured during power up.  When set to Manual, system peripherals must be explicitly configured.  The following 6 fields (PCI IDE Interface to Parallel Port Address) are only user accessible when Configuration Mode is set to Manual.

PCI IDE Interface

Enabled -- default

Disabled

Floppy Interface

Enabled -- default

Disabled

The PCI IDE Interface and Floppy Interface  fields allow these drive interfaces to be individually enabled or disabled.

Serial Port 1 Address

Disabled

COM1, 3F8h -- default

COM3, 3E8h

COM4, 2E8h *

Serial Port 2 Address

Disabled

COM2, 2F8h -- default

COM3, 3E8h

COM4, 2E8h *

The Serial Port 1 Address, Serial Port 2 Address fields allow the user to select the I/O ports used by these interfaces.  If a COM port is chosen for one of the Serial Port Addresses, it will not appear in the option list for the other Serial Port Address, i.e. if one of the Serial Port Addresses is set to Disabled, then there will be four * choices for the other serial port address.  Otherwise there will be the three choices shown.

* When an ATI Mach 32 or ATI Mach 64 video controller is active, this port and address are disabled and will not appear.  The ATI video controller uses I/O addresses that conflict with COM4.

Serial Port 2 IR Mode

Enabled

Disabled -- default

The Serial Port 2 IR Mode field allows the User to dedicate Serial Port 2 for Infra-red applications.  Serial Port 2 can also be enabled with software from application programs.  This switch is only available when Configuration Mode is set to Manual.

Parallel Port Address

Disabled

LPT3, 3BCh

LPT1, 378h -- default

LPT2, 278h

The Parallel Port Address field allows the user to select the I/O port used by this interface.

Parallel Port Mode

Compatible -- default

Extended

When set to Compatible, the parallel port will be set up in a mode compatible with the original AT.  When set to Extended the parallel port will be bi-directional.  When set to Extended the parallel port will be set up in the most advanced mode allowed by the chipset and will operate high-speed, bi-directionally.

Serial Port 1 IRQ	IRQ4

Serial Port 2 IRQ	IRQ3

Parallel Port IRQ	IRQ7

The Serial Port Assignment fields display the interrupts used by the Serial and Parallel Ports.  These values may not be changed by the User.  If an interface is disabled via the Serial Port 1 Address, Serial Port 2 Address, or Parallel Port Address fields, then the associated interrupt is not displayed, and is available for use by add in cards.

#### Advanced Chipset Configuration

When the cursor is placed the Advanced Chipset Configuration field and selected by hitting the Enter Key, the Advanced Chipset Configuration Sub-Menu appears, allowing parameters which affect the configuration of the Triton chipset to be set.

Advanced Chipset Configuration Sub-Menu

Base Memory Size

512 KB

640 KB -- default

The Base Memory Size field controls the mapping of addresses between 512K and 640K (80000h-9FFFFh).  If 512 KB is chosen then accesses between 512K and 640K are made available as system resources and can be used by add-in cards.  Unless using an add-in card which requires access to this particular address range, this field should be set to 640 KB.

ISA LFB Size

Disabled -- default

1 MB

The ISA Linear Frame Buffer Size provides a mechanism for creating a hole in the system memory map when the system is configured with 16 MB of RAM or more.  Accesses will be directed to the ISA bus instead of main memory.  This should be set to Disabled unless you are using an ISA card which has a linear frame buffer that needs to be accessed by the CPU, and you are not using the Plug and Play run-time utilities (Configuration Manager & ISA Configuration Utility).

ISA LFB Base Address

15 MB

The ISA LFB Base Address field displays the starting address of the ISA memory hole if 1 MB is chosen for the ISA LFB Size parameter.  If  ISA LFB Size is set to disabled, then this field does not appear.  This field is informative only, i.e. it cannot be changed by the user.

Video Palette Snoop

Disabled-- default

Enabled

The Video Palette Snoop field controls the ability of a primary PCI graphics controller to share a common palette (snoops write cycles) with an ISA add-in video card.  This has been set to Disabled because many Video card manufacturers have not implemented this capability correctly.

It should be set to Enabled if:

An ISA card connects to a PCI graphics card via the VESA connector

AND	That ISA card connects to a color monitor

AND	That ISA card uses the RAMDAC on the PCI card

AND	The Palette Snooping feature (also called RAMDAC shadowing) is working on the PCI card.

Latency Timer (PCI Clocks)

Number between 0 and 256

66 -- default

The Latency Timer (PCI Clocks) field controls the length of time an agent on the PCI bus can hold the bus when another agent has requested the bus.  Its default value is 66.  In general, this should not be changed.  If there are latency sensitive cards in the system (such as audio cards and/or network cards with small buffers) this can be set to a lower value to reduce latency.  If there are no latency sensitive cards in the system, but I/O intensive applications are being run, this can be increased to slightly improve performance.

PCI Burst

Enabled-- default

Disabled

The PCI Burst field controls whether PCI burst transfers are enabled or not.  Some graphics cards that do not support bursting may require this feature be disabled to function properly.

Bank 0 SIMM Detected

Bank 1 SIMM Detected

Fast Page Mode

EDO Mode

None installed

These fields are informative only.  The Bank 0 SIMM Detected and Bank 1 SIMM Detected messages will display either Fast Page Mode, EDO Mode or None installed depending upon what the BIOS found.

#### Power Management Configuration

When the cursor is placed the Power Management Configuration field and selected by hitting the Enter Key, the Power Management Configuration Sub-Menu appears, allowing parameters which affect power management to be set.

Power Management Configuration Sub-Menu

Advanced Power Management

Enabled -- default

Disabled

The Advanced Power Management field enables or disables Advanced Power Management (APM) support in the BIOS.  When Enabled the BIOS will work with APM aware Operating Systems to manage the power consumed by the CPU, IDE drives, and graphics cards.  If set to Disabled, then none of the other fields in the Power Management Configuration Sub-Menu are displayed.

IDE Drive Power Down

Enabled -- default

Disabled

The IDE Drive Power Down field controls whether or not a spin down command will be issued to the IDE drives when the system goes into low power mode.

VESA Video Power Down

Disabled

Standby

Suspend

Sleep -- default

The VESA Video Power Down field controls the VESA power management command issued to the graphics card when the system goes into low power mode.  The proper setting is dependent upon the VESA mode(s) supported by the monitor and User preference.

Inactivity Timer (Minutes)

0 to 255

10 -- default

The Inactivity Timer (Minutes) field controls the number of minutes that the system will detect no user activity before going into low power mode.

Hot Key

CTRL+ALT+User defined alphabetic character

The Hot Key field is used to define a key which when entered will cause the APM BIOS to ask an APM (Advanced Power Management) aware Operating Systems to go into immediate power managed mode.  There will be some delay, however the speaker will issue 2 descending tones.

If there is a User Password in effect, then upon going into power managed mode the Num Lock, Caps Lock, and Scroll Lock LEDs on the keyboard will flash on and off, indicating to the user that the system is in Secure Mode.  In this case the correct password will have to be entered before the user regains control of the system.  This password will not be echoed to the screen nor will a carriage return be required.  This is true whether the screen is active or inactive at the time the password is entered.  The following table describes how the Power Down hot key interacts with the Security Hot Key.

|  | Power-Down Hot Key Entered | Security Hot Key Entered |
| --- | --- | --- |
| Power-Down Hot Key Only Enabled | System powers down (can include screen blanking if enabled) Powers up when any key/mouse activity is detected | N/A |
| Security Hot Key Only Enabled | N/A | System Secure Immediately LEDs flash Input accepted when password typed |

#### Plug and Play Configuration

When the cursor is placed the Plug and Play Configuration field and selected by hitting the Enter Key, the Plug and Play Configuration Sub-Menu appears, allowing parameters which affect expansion card configurations to be set.

Plug and Play Configuration Sub-Menu

Configuration Mode

Use Setup Utility -- default

Use ICU

The Configuration Mode field is used to choose the method in which information about legacy ISA cards (ISA cards which do not have Plug and Play capabilities) is conveyed to the BIOS to allow the BIOS to set up PCI cards and ISA Plug and Play cards without conflicting with the legacy ISA cards.

If Use ICU is chosen, then the BIOS will depend upon information provided by run-time Plug and Play software (Configuration Manager and ISA Configuration Utility) to ensure that there are no conflicts with legacy ISA cards.  The user may also enable the Boot With PnP OS option which will pass control to the OS for Plug and Play management.  Choose Use ICU only if your system is configured with these utilities and they are used.

If Use Setup Utility is chosen, then the BIOS will depend upon information provided by the user in the fields below to prevent conflicts between Plug and Play cards and legacy ISA cards.  If the Use Setup Utility is chosen, then the user should not install and use run-time Plug and Play software (Configuration Manager and ICU).

Boot With PnP OS

Disabled -- default

Enabled

This screen will appear only if Use ICU is selected.  When Boot With PnP OS is Enabled, the BIOS will activate only those Plug and Play add-in cards needed to boot the system.  The BIOS will then pass control for configuring the remaining Plug and Play cards to an operating system that can manage Plug and Play add-in cards..  Note: If set to Enabled and a non-Plug and Play O/S such as DOS is used, the remaining add-in cards (those not needed for booting) will remain unconfigured.

ISA Shared Memory Size

Disabled -- default

16 KB

32 KB

48 KB

64 KB

80 KB

96 KB

The ISA Shared Memory Size field is used to specify a range of memory addresses that will directed to the ISA bus rather than on-board memory.  This field should be set to Enabled only when a non Plug and Play ISA card (legacy card) is used that requires non-ROM memory space.  LAN cards that have on-board memory buffers are one example of this, video capture cards that have video buffer memory are another.

By default, allocation of upper memory is as follows.  Memory from C0000-C7FFF is automatically shadowed.  (This memory range is typically reserved for Video BIOS.)  Memory from C8000-DFFFFh is initially unshadowed.  The BIOS will scan this range for any ISA expansion card BIOSes that may be present and note the location and size.  The BIOS will then autoconfigure the PCI and Plug and Play expansion cards, shadowing the area above E0000h (other than video) until that area is full.  It will then assign additional PCI and Plug and Play expansion cards to the area between C8000h and DFFFFh.  If an ISA legacy card has non-BIOS memory requirements, the autoconfigure routine may write into an area that is needed by the ISA expansion card.  The ISA Shared Memory Size parameter is used to signify to the autoconfigure routine that this block of memory is reserved and should not be shadowed.  (Shadowing is a technique which copies a block of memory from an add in card’s ROM to the same address in system DRAM memory to allow faster access to the code and achieve higher performance.)

If set to a value other than Disabled  the ISA Shared Memory Base Address field will appear below.  Note that if a value of  96 KB is chosen, then the ISA Shared Memory Base Address field can only be set to C8000h; if 80 KB, then a value of C8000h or CC000h, etc.

ISA Shared Memory Base Address

C8000h

CC000h

D0000h

D4000h

D8000h

DC000h

The ISA Shared Memory Base Address field works in conjunction with the ISA Shared Memory Size.  This is the starting address of the unshadowed memory block specified by the ISA Shared Memory Size.

IRQ 3*

IRQ 4*

IRQ 5

IRQ 7*

IRQ 9

IRQ 10

IRQ 11

Available

Used By ISA Card

The IRQ 3 - IRQ 11 fields are used to indicate whether individual interrupts are used by ISA legacy cards.  If not used, then they should be set to Available.  If used they should be set to Used By ISA Card.

* These options will not be visible if consumed by on-board resources (IRQ3 by Serial Port 1, IRQ4 by Serial Port 2, IRQ7 by Parallel Port).

Note: If PCI add-in cards are used, all but one can be set to Used By ISA Card; one must remain available in order for PCI cards which use interrupts to function correctly.

### Security Menu

The objectives of the BIOS security system are to:

1.	Keep unwanted individuals from accessing the computer.

2.	Limit the fields that can be accessed in the CMOS Setup Utility program.

3.	Allow the computer to boot while unattended and still be secure.

4.	Set up a hot key to invoke the security system immediately.

The security system is implemented using two passwords in tandem, a User Password and an Administrative Password.  The level of security available is dependent on whether one or both passwords are set and which password is used to enter the system.  It is assumed that if the Administrative Password is used, it is controlled by a central corporate information services or security organization and that the Administrative Password and User Password will have been set prior to the end user receiving the system.

To meet each of the objectives above, use the following procedures:

1.	End user sets/resets User Password depending upon whether the Administrative Password is in effect.

2.	Administrative Password and User Password were set prior to end user receiving system; no action required by end user.

3.	End user sets User Password then Enables Unattended Start.  If User Password had previously been set, user  Enables Unattended Start.

4.	End user sets User Password then Enables Security Hot Key.  If User Password had previously been set, user Enables Security Hot Key.

The table below shows the interactions of passwords and security capabilities:

| Password in Effect | Password Needed at Boot? | Password Needed to Access Setup? | Can Access All Setup Fields? | Able To Set Unattended Start? | Able To Set Security Hot Key? |
| --- | --- | --- | --- | --- | --- |
| User only | Yes | No | Yes | Yes | Yes |
| Administrative only | No | Yes | Yes | No | No |
| Both -- User password used | Yes | No | No | Yes | Yes |
| Both -- Admin password used | Yes | N/A | Yes | Yes | Yes |
| No password enabled | No | No | Yes | No | No |

If the Administrative Password is in effect and Setup is entered with the User Password, only the following fields may be changed:

System Date

System Time

User Password

Security Hot Key

Unattended Start

Power Management Hot Key

User Password is

Disabled -- default

Enabled

Administrative Password is

Disabled -- default

Enabled

The User Password is and Administrative Password is fields indicate whether the User and Administrative passwords have been set.  The User Password is must be Enabled for the Unattended Start and Security Hot Key (CTRL-ALT-) parameters to be displayed.

Set User Password

When the Set User Password field is chosen the user is presented with one of two sub-menus.  If no User Password is in effect the following menu appears:

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

If there is already a User Password in effect, the following menu appears.  To disable the password, use a carriage return as the new password.  The password may be up to 7 alphanumeric characters.

Enter Current Password_______

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

Set Administrative Password

When the Set Administrative Password field is chosen the user is presented with one of two sub-menus.  If no Administrative Password is in effect the following menu appears:

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

If there is an Administrative Password in effect, the following menu appears.  To disable the password, use a carriage return as the new password.  The password may be up 7 alphanumeric characters.

Enter Current Password_______

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

Unattended Start

Disabled -- default

Enabled

The Unattended Start field controls the point at which the User Password is required.  If set to Disabled, the User Password is required before the system boots, either from disk or the network.  The user will be prompted for the password with the text string “Enter Password (1):”

If set to Enabled the system will go through automatic boot activity (for instance running CONFIG.SYS and AUTOEXEC.BAT when booting DOS from disk), however the keyboard will be locked and the User Password must be entered to unlock it.  The BIOS does not provide any prompt string so it is suggested that the last thing that the automatic login sequence does is prompt the user for the password.

The Unattended Start field can only be set if a User Password is in effect.

Security Hot Key (CTRL-ALT-)

Alpha-Numeric Key

The Security Hot Key (CTRL-ALT-) field can be used to define a key which when entered in conjunction with the CTRL and ALT keys puts the system into secure mode (locks the keyboard).  When this Hot Key combination is entered the Num Lock, Caps Lock, and Scroll Lock LEDs on the keyboard will flash on and off, indicating to the user that the system is in secure mode (as opposed to being hung).

### Exit Menu

Exit Saving Changes

The Exit Saving Changes field is used to exit the Setup Utility saving the current state of all parameters.  When the Enter key is entered the following sub-menu is displayed:

Exit Saving Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then the Setup Utility will be exited with all parameters set to their current values.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any change.

Exit Discarding Changes

The Exit Discarding Changes field is used to exit the Setup Utility discarding any changes made to Setup parameters.  When the Enter key is entered the following sub-menu is displayed:

Exit Discarding Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then the Setup Utility will be exited with changes to Setup parameters lost.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any change.

Load Setup Defaults

The Load Setup Defaults field is used to load default values into all Setup parameters, except for the passwords.  When the Enter key is entered the following sub-menu is displayed:

Load Setup Defaults?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then defaults are loaded into all Setup parameters, except for the passwords.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any changes.

Discard Changes

The Discard Changes field is used undo any changes made to Setup parameters during this Setup session.  When the Enter key is entered the following sub-menu is displayed:

Discard Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then any changes made to Setup parameters are discarded.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any changes.

# Appendix H - INTEL/AMIBIOS Error messages and Beep Codes

Errors can occur during POST (Power On Self Test) which is performed every time the system is powered on. Fatal errors, which prevent the system to continue the boot process, are communicated through a series of audible beeps. Other errors are displayed in the following format:

ERROR Message Line 1

ERROR Message Line 2

For most displayed error messages, there is only one message. If a second message appears, it is "RUN SETUP". If this message occurs, press <F1> to run AMIBIOS Setup.

#### BEEP CODES

| Beeps | Error Message | Description |
| --- | --- | --- |
| 1 | Refresh Failure | The memory refresh circuitry on the baseboard is faulty. |
| 2 | Parity Error | Parity is not supported on this product, will not occur. |
| 3 | Base 64 KB Memory Failure | Memory failure in the first 64 KB. |
| 4 | Timer Not Operational | Memory failure in the first 64 KB of memory, or Timer 1 on the baseboard is not functioning. |
| 5 | Processor Error | The CPU on the baseboard generated an error. |
| 6 | 8042 - Gate A20 Failure | The keyboard controller (8042) may be bad. The BIOS cannot switch to protected mode. |
| 7 | Processor Exception Interrupt Error | The CPU generated an exception interrupt. |
| 8 | Display Memory Read/Write Error | The system video adapter is either missing or its memory is faulty. This is not a fatal error. |
| 9 | ROM Checksum Error | ROM checksum value does not match the value encoded in BIOS. |
| 10 | CMOS Shutdown Register Rd/Wrt Error | The shutdown register for CMOS RAM failed. |
| 11 | Cache Error / External Cache Bad | The external cache is faulty. |

#### ERROR MESSAGES

| Error Message | Explanation |
| --- | --- |
| 8042 Gate - A20 Error | Gate A20 on the keyboard controller (8042) is not working. Replace the 8042. |
| Address Line Short! | Error in the address decoding circuitry on the baseboard. |
| Cache Memory Bad, Do Not Enable Cache! | Cache memory is defective. Replace it. |
| CH-2 Timer Error | Most AT systems include two timers. There is an error in timer 2. |
| CMOS Battery State Low | CMOS RAM is powered by a battery. The battery power is low. Replace the battery. |
| CMOS Checksum Failure | After CMOS RAM values are saved, a checksum value is generated for error checking. The previous value is different from the current value. Run AMIBIOS Setup. |
| CMOS System Options Not Set | The values stored in CMOS RAM are either corrupt or nonexistent. Run Setup. |
| CMOS Display Type Mismatch | The video type in CMOS RAM does not match the type detected by the BIOS. Run AMIBIOS Setup. |
| CMOS Memory Size Mismatch | The amount of memory on the baseboard is different than the amount in CMOS RAM. Run AMIBIOS Setup. |

#### ERROR MESSAGES (cont.)

| CMOS Time and Date Not Set | Run Standard CMOS Setup to set the date and time in CMOS RAM. |
| --- | --- |
| Diskette Boot Failure | The boot disk in floppy drive A: is corrupt. It cannot be used to boot the system. Use another boot disk and follow the screen instructions. |
| Display Switch Not Proper | Some systems require a video switch on the baseboard be set to either color or monochrome. Turn the system off, set the switch, then power on. |
| DMA Error | Error in the DMA controller. |
| DMA #1 Error | Error in the first DMA channel. |
| DMA #2 Error | Error in the second DMA channel. |
| FDD Controller Failure | The BIOS cannot communicate with the floppy disk drive controller. Check all appropriate connections after the system is powered down. |
| HDD Controller Failure | The BIOS cannot communicate with the hard disk drive controller. Check all appropriate connections after the system is powered down. |
| INTR #1 Error | Interrupt channel 1 failed POST. |
| INTR #2 Error | Interrupt channel 2 failed POST. |
| Invalid Boot Diskette | The BIOS can read the disk in floppy drive A:, but cannot boot the system. Use another boot disk. |
| Keyboard Is Locked...Unlock It | The keyboard lock on the system is engaged. The system must be unlocked to continue. |
| Keyboard Error | There is a timing problem with the keyboard. Set the Keyboard option in Standard CMOS Setup to Not Installed to skip the keyboard POST routines. |
| KB/Interface Error | There is an error in the keyboard connector. |
| Off Board Parity Error | Parity error in memory installed in an expansion slot. The format is: OFF BOARD PARITY ERROR ADDR (HEX) = (XXXX) XXXX is the hex address where the error occurred. |
| On Board Parity Error | Parity is not supported on this product, this error will not occur. |
| Parity Error ???? | Parity error in system memory at an unknown address. |

#### ISA NMI Messages

| ISA NMI Message | Explanation |
| --- | --- |
| Memory Parity Error at xxxxx | Memory failed. If the memory location can be determined, it is displayed as xxxxx. If not, the message is Memory Parity Error ????. |
| I/O Card Parity Error at xxxxx | An expansion card failed. If the address can be determined, it is displayed as xxxxx. If not, the message is I/O Card Parity Error ????. |
| DMA Bus Time-out | A device has driven the bus signal for more than 7.8 microseconds. |

# Appendix I - BIOS Recovery

The Advanced ZP ED incorporates the AMIBIOS in a Flash memory component.  Flash BIOS allows easy upgrades without the need to replace an EPROM.  The upgrade utility fits on a floppy diskette and provides the capability to save, verify, and update the system BIOS.  The upgrade utility can be run from a hard drive or a network drive, but no memory managers can be installed during upgrades.

The latest upgrade utility and BIOS code are available in the BIOS section of the iPAN bulletin board.

## Using the upgrade utility

If the utility is obtained from the bulletin board, UNZIP the archive and copy the files to a bootable MS-DOS 3.3, 4.01, 5.0, or 6.0 diskette.  Reboot the system with the upgrade diskette in the bootable floppy drive and follow the directions in the easy to use menu-driven program.

## Recovery mode

In the unlikely event that a FLASH upgrade is interrupted catastrophically, it is possible the BIOS may be left in an unusable state.  Recovering from this condition requires the following steps (be sure a power supply and speaker have been attached to the board, and a floppy drive is connected as drive A:):

1.	Change Flash Recovery jumper (RCVR FLASH) to the recovery mode position.

2.	Install the bootable upgrade diskette into drive A:

3.	Reboot the system.

4.	Because of the small amount of code available in the non-erasable boot block area, no video is available to direct the procedure.  The procedure can be monitored by listening to the speaker and looking at the floppy drive LED.  When the system beeps and the floppy drive LED is lit, the system is copying the recovery code into the FLASH device.  As soon as the drive LED goes off, the recovery is complete.

5.	Turn the system off.

6.	Change the "FLASH RECOVERY" jumper back to the default position.

7.	Leave the upgrade floppy in drive A: and turn the system on.

8.	Continue with the original upgrade.

# Appendix J - Environmental Standards

| Parameter | Condition | Specification |
| --- | --- | --- |
| Temperature |  |  |
|  | Non-Operating | -40oC to +70oC |
|  | Operating | +0oC to +55oC |
| Humidity |  |  |
|  | Non-Operating | 92% Relative Humidity max. @ 36oC |
|  | Operating | 80% Relative Humidity max. @ 36°C |
| Shock | Non-Operating | 30.0G, 11ms, 1/2 sine |

Table J-1. Environmental standards

# Appendix K - Reliability Data

The Mean-Time-Between-Failures (MTBF) data is calculated from predicted data @ 55C.

Advanced ZP ED baseboard			TBD