CALCOMP PROTECTED MODE ADI DRIVER VERSION 4.2

09-2-92

The DGCALCMP.EXP file is a protected mode driver version 4.2. This version

supports Autocad versions 11 and 12. When the ADI driver is selected for

installation, a second file is copied to the CALDB directory called

DGCALCMP.DLL for use with Windows. This file needs to be copied to the ACADWIN

directory for use with AutoCAD for Windows.

USING THE ADI DRIVER WITH AUTOCAD FOR DOS

The installation program will copy the ADI driver to a sub-directory on

the hard drive called DRV in the AutoCad directory. This is the default

directory for AutoCad 12. If you are using AutoCad 11 you need to tell AutoCad

that the ADI driver is in the sub-directory call DRV.

If you are using AutoCad 11 you will need create a sub-directory in the AUTOCAD

directory called DRV. Then tell AutoCad that the ADI driver is in the

sub-directory call DRV. This is done by typeing

SET DGPADI=C:\ACAD\DRV\DGCALCMP.EXP

at your DOS prompt. You may want to add this line to your AUTOEXEC.BAT

file or to another batch file used to start AUTOCAD.

If you want to shell out of Windows and use AutoCad for DOS, you will

need to configure the ADI driver to use interupt mode. If you use

polling mode the cross hairs will not move smoothly.

If you have the digitizer connected to COM 3 be sure to select polling

mode in the AutoCad configuration. If interupt mode is selected you may

not get any cross hairs displayed in the graphic screen.

USING THE ADI DRIVER WITH AUTOCAD FOR WINDOWS

When in the AutoCad for Windows configuration menu, select Digitizer

Setup. Select ADI Driver By CalComp. A list of digitizers will be displayed.

Select the digitizer of your choice. The last selection on the list is

"Windows Tablet". Using this selection allows the digitizer to switch between

ADI mode and mouse mode. This is accomplished by assigning a button on the

cursor during the setup. A toggle area at the lower RIGHT corner of the

digitizer can be used for a mouse/ADI toggle area as well.

When using the tablet congiured as a "Windows Tablet" be sure when

selecting a button for toggle mode that you select a button that you

do not use while in AutoCAD. When assigning a toggle button, the

fuction that was assigned to the button prior will be overwritten.