U. S.  R O B O T I C S ,  I N C .

A P P L I C A T I O N   N O T E S

F O R  M I C R O S O F T  M A I L  W I T H

M I C R O S O F T  M A I L  R E M O T E

December 1991

(Revision 1.0)

RECOMMENDATION

It is recommended by U.S. Robotics, Inc. to use a Courier V.32, Courier

V.32 bis, or a Courier Dual Standard for the following applications in order to

achieve the greatest effective throughput.  All of the following application

notes are based on the use of U.S. Robotics high speed modems.

APPLICATIONS OVERVIEW

The following application notes details the use of remote mail service using

Microsoft Mail and Microsoft Mail Remote using U.S. Robotics' modems.  This set

of notes will detail the configurations\setups to:

*  Configuring the Host Site (Microsoft Mail)

*  Configuring the Remote Site (Microsoft Mail Remote)

*  Modifying the Script File

*  Configuring the Host and Remote Modems

VERIFICATION

The Applications Engineering Department at U.S. Robotics, Inc. has tested and

verified that the following application notes are accurate on Versions V2.1c

Microsoft Mail and Microsoft Mail Remote.

NOTE

If leased lines are used for any of the following applications, U.S. Robotics'

modems can only be placed on Two-Wire Lines.  If a Four-Wire Leased Line has

been installed, the phone company will need to install a Four-Wire to Two-Wire

converter.

CONFIGURING THE HOST SITE

(MICROSOFT MAIL)

MICROSOFT MAIL SETUP

After Microsoft Mail has been loaded on the Host system, have the USR.SCR and

USR.MDM files copied into the \MAILDATA\GLB sub-directory.  These files are

available from the U. S. Robotics BBS at (708) 982-5092.  The file name is

MSMAIL.ZIP under the Communications Section.

Run the Administrator Program by executing:

MAILEXE>ADMIN  -D?<ENTER>.

The -D? specifies the drive where the Data Files are located.  If these files

are not copied into the default directory of M, replace the ? with the Drive

where the Data Files are located.

Select the option of Config.  Within Config, select the option of Serial.

Within Serial, the options of Baud-rate, Mode, and Comm. Port are displayed.

The Baud-rate is set through the Script File, so this does not need to be

changed.  Set the Mode option for either pulse or touch tone dialing.  Set the

Comm. Port to either com1 or com2.  If Com Ports of com3 or com4 are needed,

the script file can be modified to work with these com ports.  See the section

on Modifying the Script File for more information.

SCRIPT FILE EXECUTION

There are three ways in which to have the U. S. Robotics script file executed.

Command Line Execution -

One option is to specify the script file on the command line.  When ready to

receive/send mail, execute the EXTERNAL program.  To specify which script file

to use through the command line, type:

MAILEXE>EXTERNAL -TUSR <ENTER>.

The -T command sets the program to use the following script file.  The -TUSR

specifies the USR script file.

Default Script Execution -

To eliminate the need of specifying the script file through the execution of

EXTERNAL, rename the USR.SCR file to DEFAULT.SCR.  The EXTERNAL program always

runs the DEFAULT script file unless there is a script file command executed on

the command line or a script file has been defined through the Administrator

program.

Admin Execution -

The final option is to configure the External-Admin option through the

Administrator Program.  When the Host site is to connect to an External Post

Office, it is possible to have the script file in place.  To configure for this

option, select the External-Admin option.   Next, select the Setup option.  A

prompt will appear to enter the Network Name.  Enter the Network Name that the

Host will be connecting to.  At the next selection, choose Number.  In the

number field, enter <USR.  This defines that the USR.SCR script file is to be

used when connecting to that network. The USR Script file will need to be

modified in order for this option to work properly.  See the section on

Modifying the Script File for more information.

CONFIGURING THE REMOTE SITE

(MICROSOFT MAIL REMOTE)

MICROSOFT MAIL REMOTE SETUP

After Microsoft Mail Remote has been loaded at the remote site, and the data

disk for that site has been copied, execute the following commands.

Run the Mail Program by executing:

MAILEXE>MAIL <ENTER>

Select the option of Telephone.  Within Telephone select the option of Config.

Within Config, the options of Baud-rate, Mode, and Comm. Port are displayed.

The Baud Rate is set through the Script File, so this does not need to be

changed.  Set the Mode option for either pulse or touch tone dialing.  Set the

Comm. Port to either com1 or com2.  If com3 or com4 is needed, the script file

can be modified to work with these com ports.  See the section on Modifying the

Script File for more information.

SCRIPT FILE EXECUTION

There are two ways in which to execute MAIL and have the U. S. Robotics script

file executed.

Command Line Execution -

One way is to include a command during the execution of MAIL.  The following

command would execute MAIL and Mail will use the U. S. Robotics script file:

MAILEXE>MAIL -TUSR<ENTER>

The -T command sets the program to use the following script file.  The -TUSR

specifies the USR script file.

Default Script Execution -

To eliminate the need of specifying the script file through the execution of

MAIL, rename the USR.SCR file to DEFAULT.SCR.  The EXTERNAL program always runs

the DEFAULT script file unless there is a script file command executed on the

command line.

MODIFYING THE SCRIPT FILE

The U.S. Robotics script file can be modified if certain parameters need to be

added or changed. Some examples include additional Com Port selections, Dialing

External Post Offices using script files, and Technical Support Troubleshooting.

The file that will need to be modified is USR.MDM.  This file should be located

in the MAILDATA\GLB sub-directory.  The USR.MDM file can be modified by using

an editor.  After the changes are completed, recompile the script by executing:

MAILDATA\GLB>SCRCOMP USR.MDM <ENTER>

If no errors are detected, the script file should be ready to use.

MODIFYING THE FILE

Com Port Selections -

If the need to communication through com3 or com4 occurs, the script file can be

modified to include the PORT COM command.  Using an editor, add the following

command line in the script INITIALIZE section.

INITIALIZE:

title "U.S. Robotics HST/V32bis Script - 12/02/91"

baud 19200

add line--->           port com3

. . .  rest of script the same

This additional line allows Microsoft Mail and Microsoft Mail Remote to

communicate through com port 3.  Communications through com4 is also possible

by following the same format.

Dialing External Post Offices -

If the option to dial an external post office using a script file is wanted,

the script file will need to be modified to add the phone number to that

location.  Because this type of change is specializing the script file, copying

the file to another name is recommended if there are multiple locations the

Host will be connecting to.  Using an editor, change the following line in the

script CALL section.

CALL:

$ret = 8

clearrsp

echo 0

change this line--->    sendln "ATD" + dial_mode + phone_number

change line to--->      sendln "ATD?999999999"

Replace ? with T for Touch Tone Dialing

Replace ? with P for Pulse Dialing

Replace 9 with the number to dial.  Include any prefixes necessary to obtain

an outside line or for long distance calls.

Be sure to close the quote at the end of the dialing string.

Technical Support Troubleshooting -

If problems occur and it is thought to have something to do with the modem,

technical support might need the script file to be modified to change some of

the modem parameters.  If this situation occurs, the technical support person

will inform you of what needs to be changed and how to change it.

CONFIGURING THE HOST AND REMOTE MODEMS

Make sure the Courier V.32, V.32 bis, or the Dual Standard modem is configured

to the following:

DIP SWITCH SETTINGS -

Set the following Dip Switch settings on both the Host and Remote modems:

(U=UP or OFF, D=DOWN or ON)

1)U  2)D  3)D  4)D  5)D  6)U  7)U  8)D  9)D  10)U