Notes on using the EtherLink III EISA/PCI packet driver

-------------------------------------------------------

The EtherLink III family of adapters are all software configured.

The driver can be loaded from the command line or in autoexec.bat.

When the packet driver is loaded the only required command line

option is the software interrupt value (/I).  The software interrupt

is used for communication between the driver and the protocol and

should not be confused with the adapter's hardware interrupt vector.

As of version 1.10 of the Packet Driver Specification valid software

interrupt values are in the range 0x20 to 0x80.  Version 1.09 has

software interrupt values in the range of 0x60 to 0x80.

Syntax:

3c59xpd [switches] [options]

The following switches are supported by the driver:

-? = show usage

-u = uninstall driver

-n = novell netware packet conversion

-d = delay initialization (for diskless workstation booting)

-t = disable transmit parallel tasking

-r = disable receive parallel tasking

-m = disable bus master

The -n switch is needed if the packet is in Ethernet Encapsulated IPX format

(Ethernet II) and it needs to be converted to IEEE 802.3 format.  The default

is no conversion .  The BYU packet driver shell requires Ethernet II format.

The -t, -r, -m switches are provided for diagnostic purposes.  3Com's parallel

tasking feature offers much better performance.  However, this improved

performance can cause some software to fail.  The -t and -r switches allow

parallel tasking to be turned off while transmitting or receiving packets.

The default is both transmit and receive parallel tasking are enabled. The -m

option prevents the driver from using bus master to move the data to/from the

adapter.  When this option is used, the packet driver will move the data using

programmed I/O (PIO) only.  The default is bus master is enabled.

The following options are supported:

/I = software interrupt (20H - FFH) default = 70H

/B = bus type ( E = EISA, P = PCI ) default = EISA

/S = slot number (1 - 15 for EISA, 0 - 31 for PCI)

/K = bus number (0 - 7 for PCI only)

Before installing the packet driver, you must choose an entry point (software

interrupt) number in the range between 0x60 and 0x7e inclusive.  This entry

point is specified using the /I parameter.  Some users have reported trouble

with dBase when using interrupts in the low 60's.  These problems go away

when they switch to an interrupt in the high 70's (e.g. 7e). Interrupt 67 is

unavailable because it's used by the EMS interface (EMM386.EXE).  Interrupts

70 through 77 are unavailable because the second interrupt controller uses

them for hardware interrupts IRQ 8 through IRQ 15.  Interrupts 7f and 80

may not be unavailable because at least one package, when locating a packet

driver, stops searching before 0x7f. A list of interrupt numbers and uses is

provided at the end of this document.

The bus type, slot number, and bus number are used to point the packet driver

to a specific adapter not locatable using the driver default parameter values.

The bus type must be specified if the packet driver is using a PCI adapter.

The bus number must be specified if the PCI adapter is not located on PCI bus

0.  This parameter is only needed when the packet driver is used in a PCI

computer which has more than one PCI bus.

The slot number tells the packet driver which slot the adatper is located in.

If this parameter is not used, the Packet driver will scan each slot on the

bus and use the first adapter it finds.  When the slot number parameter is

used, the packet driver will only look in the specific slot for the adapter.

The slot parameter allows the packet driver to be set to use a specific

adapter in a machine where more than one adapter is installed.

EXAMPLE:  To load driver at software interrupt 60h with receive parallel

tasking disabled, PCI bus type, slot number = 1, and PCI bus number = 0 the

command line will look like the following:

PD -r /B=P /S=1 /K=0 /I=60

For a complete usage summary, enter pd -? at the command line.

3Com technical support for packet drivers can be contacted at:

Pkt_Support@3mail.3Com.com

Appendix A - Interrupt usage in the range 0x60 through 0x80,

from Ralf Brown's interrupt list.

60 -- -- reserved for user interrupt

60 -- -- FTP Driver - PC/TCP Packet Driver Specification

60 01 FF FTP Driver - DRIVER INFO

60 02 -- FTP Driver - ACCESS TYPE

60 03 -- FTP Driver - RELEASE TYPE

60 04 -- FTP Driver - SEND PACKET

60 05 -- FTP Driver - TERMINATE DRIVER FOR HANDLE

60 06 -- FTP Driver - GET ADDRESS

60 07 -- FTP Driver - RESET INTERFACE

60 11 -- 10-NET - LOCK AND WAIT

60 12 -- 10-NET - LOCK

60 13 -- 10-NET - UNLOCK

60 20 -- FTP Driver - SET RECEIVE MODE

60 21 -- FTP Driver - GET RECEIVE MODE

60 24 -- FTP Driver - GET STATISTICS

61 -- -- reserved for user interrupt

62 -- -- reserved for user interrupt

63 -- -- reserved for user interrupt

64 -- -- reserved for user interrupt

65 -- -- reserved for user interrupt

66 -- -- reserved for user interrupt

67 -- -- LIM EMS (EMM386.EXE)

67 DE 00 Virtual Control Program Interface - INSTALLATION CHECK

68 01 -- APPC/PC

69 -- -- unused

6A -- -- unused

6B -- -- unused

6C -- -- system resume vector (CONVERTIBLE)

6C -- -- DOS 3.2 Realtime Clock update

6D -- -- VGA - internal

6E -- -- unused

6F -- -- Novell NetWare - PCOX API (3270 PC terminal interface)

6F 00 -- 10-NET - LOGIN

70 -- -- IRQ8 - REAL-TIME CLOCK

71 -- -- IRQ9 - RESERVED

72 -- -- IRQ10 - RESERVED

73 -- -- IRQ11 - RESERVED

74 -- -- IRQ12 - BUS MOUSE INTERRUPT

75 -- -- IRQ13 - MATH COPROCESSOR ERROR

76 -- -- IRQ14 - FIXED DISK

77 -- -- IRQ15 - RESERVED

78 -- -- not used

79 -- -- not used

7A -- -- Novell NetWare - LOW-LEVEL API

7A -- -- AutoCAD Device Interface

7B -- -- not used

7C -- -- not used

7D -- -- not used

7E -- -- not used

7F -- -- HDILOAD.EXE - 8514/A VIDEO CONTROLLER INTERFACE

7F -- -- HLLAPI (High-Level Language API)

80 -- -- reserved for BASIC