NetWare Driver Installation             FEBRUARY 1993

===========================

This diskette is labeled as LAN_DRV_ETH and the files are organized as follows:

Directory            Contents

----------------  -------------------------------------------------------

\NETWARE          A pre-generated IPXETH16.COM

\NETWARE\V2xx     NetWare v2.15b, v2.15c or v2.2 server & bridge drivers

\NETWARE\V215C    NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\NET386   NetWare 386 server or bridge drivers

\NETWARE\ODI.DOS  ODI workstation driver for DOS

\NETWARE\ODI.OS2  ODI workstation driver for OS2

*********************************

*  QUICK SETUP FOR WORKSTATION  *

*********************************

A pre-generated workstation driver IPXETH16.COM is included in this disk.

You can simply run the IPXETH16 on a workstation to access the Novell network:

>IPXETH16    --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx        --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXETH16.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_ETH.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_ETH

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

ME-16ODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example 1:  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\NET386\*.*  B:\  --SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load  A:\386ME-16.LAN     --Select your network card settings

:Bind ipx to 386ME-16

:load install              --Follow the NetWare installation instructions

Example 2:  NetWare v2.2 Server/Bridge Setup

============================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of the diskettes labeled SYSTEM-1 and LAN_DRV_ETH.

>Copy A:\NETWARE\V2XX\*.LAN  B:\  --SYSTEM-1 disk must be in drv B

>Copy A:\NETWARE\V2XX\*.OBJ  A:\  --Copy the files to the root dir

2)  Execute Novell's INSTALL program, which resides in the SYSTEM-1 diskette.

Choose the "ETHER ETHER-16 Ethernet" driver when asked for a network

driver.

Example 3:  NetWare v2.15b Server/Bridge Setup

==============================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled LAN_DRV_ETH.

>Copy A:\NETWARE\V2XX\*.LAN  A:\ --AUXGEN disk must be in drive A

>Copy A:\NETWARE\V2XX\*.OBJ  A:\ --LAN_DRV_ETH disk must be in drive A

2)  Execute Novell's NETGEN program, which resides in the NETGEN diskette.

Choose the "ETHER ETHER-16 Ethernet" driver, when asked for a network

driver.

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

ETHER ODI Driver program allows you to run multiple protocol stacks.  It also

allows you to install up to two network adapters in a workstation. ETHER ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.  For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the ME-16ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the ME-16ODI.COM whenever a reference is made to the Hardware Specific

Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>ME-16ODI          --Hardware Specific Module

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy ME-16ODI.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\ME-16ODI.SYS

3)  ETHER ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".