;************************************************************************;

;                                                                        ;

;  The InterLan AT Application Development Toolkit                       ;

;                                                                        ;

;  Synopsis/Tutorial                                                     ;

;                                                                        ;

;  Copyright (c) 1992 by Racal-Datacom, Inc.  All rights reserved.       ;

;                                                                        ;

;  Aug 26 92     Jim Dunham              Beta release                    ;

;                                                                        ;

;************************************************************************;

The InterLan AT Development Toolkit is designed to allow users

to write applications that run directly over ethernet with no

intervening network-layer protocol stack, or with a custom stack.

The toolkit (like the InterLan AT drivers) works transparently on

both the InterLan AT and the InterLan XT cards.

The toolkit is currently distributed as a .LIB file that will allow

a large-model C program to link to the driver.  Using a few simple

services, the user has quick and simple access to packets of a given

ethertype on an ethernet network.

Two different .LIB files are provided in this toolkit: one produced

using MASM v5.1 and MS LIB v3.10, and another produced using Borland's

TASM v2.01 and TLIB v3.01.  Either one should work with your application,

but we are providing them both, just in case.  The demo app, written using

Microsoft C v5.1, links fine with either one.

There should be no problem using compilers other than Microsoft's

to write applications using the InterLan AT application toolkit.

Note that the demo app uses some Microsoft-specific library features

(int86(), for instance) that would need to be changed in order to

successfully link using a non-Microsoft compiler.

The following six services are supported:

InitInterLanAT()             - Initialize the card and driver.

GetEthernetAddress()         - Retrieves the card's network address.

RegisterEtherType()          - Add an ethertype to the receive routine.

SendPacket()                 - Transmit a packet onto the media.

ResetInterLanAT()            - Reset the InterLan AT and its driver.

ReleaseInterLanAT()          - Release resources (such as the

hardware interrupt) associated

with the InterLan AT card.

Use of the InterLan AT Toolkit

Using InitInterLanAT()

int InitInterLanAT(void);

After your application has initialized itself and is ready to begin

communication across the ethernet, the first routine you must call

is InitInterLanAT().  This routine will reset the InterLan AT and

bring it into operation, step by step.  A non-zero return code, if

you ever see one, would indicate the step at which an initialization

problem occured.

Allocating Data Structures

You will need to allocate a few structures in order to make use of

the embedded ethernet driver:

Allocate at least two packet structures (PacketBuf's).  You will need

one in order to send packets, and at least one to receive packets.

Note that you need one receive structure (RcvBlock) for each

receive PacketBuf.

If you will always be using the same ethertype to send, this is

a good place to set the ethertype in your transmit PacketBuf.

Using RegisterEthertype()

int RegisterEthertype(unsigned int etype, unsigned int far *handleptr);

You will need to call the routine RegisterEthertype() once for every

ethertype you intend to receive.  You must pass both the ethertype

you want to snag and the address of an unsigned int to hold the

handle that will be returned by the driver.

Each time that you call RegisterEthertype, the driver adds the given

ethertype to the list of ethertypes that will cause the driver to

call your ReceiveProcedure.  This gives your application the chance

to examine the packet before committing to receive it.

Notes on ReceiveProcedure():

The InterLan AT toolkit will fully link to your application only if

your application exports the C routine name "ReceiveProcedure".

IMPORTANT:

Make sure you DISABLE stack checking ("stack probes") around

"ReceiveProcedure" (and any procedure called by it).  Since the

driver will have switched to a separate stack before calling you,

stack probes from your C compiler will fail and jump to an invalid

address, hanging your computer.

ReceiveProcedure will be called by the driver whenever a packet

with one of your registered ethertypes is received.  This gives

your application the chance to examine the packet before committing

to receive it.  The driver will pass your receive routine a pointer

to a RcvBlock.  To reject the packet, place the value NULL into

the RcvBlock's 'buffer' field; to accept the packet, place a valid

pointer to a packet buffer into the 'buffer' field, and set the

buffer length in the 'buffer_len' field.

Using SendPacket()

int SendPacket(byte far * packet, unsigned int length);

This function returns non-zero if transmission is currently

refused by the driver due to another pending transmission,

or zero if the driver is attempting to transmit the frame.

Note that zero doesn't necessarily mean that the transmission

is succeeding, only the the driver is trying.

This is the routine to call in order to place a packet onto the

media.  You are responsible for passing down the correct length

of the MAC packet, which is the length of the data field + 14.

You should keep your data field lengths between 46 bytes (min)

and 1500 bytes (max).  Pad your data with unused bytes if necessary,

because frames that are too small can cause your ethernet to operate

incorrectly.  The value of pad bytes is undefined in the 802.3 spec.

Calling this routine will cause the driver to block long enough to

copy the given packet onto the card's transmit buffer and start the

transmit hardware logic.  This routine will then return to your

application.  If the hardware's first attempt to place the frame

onto the media does not succeed, it will retry up to fifteen times.

If the media is still too busy for the hardware to completely send

your packet, the driver will perform the I/O instructions necessary

to re-initiate hardware transmission attempts up to nine times,

while your application is busy doing other things.

Please note that some conditions, such as a completely saturated

network, or an unterminated network, can cause the datagram

delivery method employed by this routine, to fail.  Since this

routine does not block long enough for all retries to succeed, it

is impossible for a true completion code to be returned.  Therefore

your application will only be able to tell whether transmission

succeeded based on packets received from other nodes on your network.

Using GetEthernetAddress()

void GetEthernetAddress(byte far *addr_buf);

This routine can be called to get the driver to copy the card's

ethernet address into your buffer of choice; for instance, the

source address field of your transmit PacketBuf.

Using ResetInterLanAT()

void ResetInterLanAT(unsigned int handle);

Use of this routine is optional.  It is included in case your

application needs to reset the physical card and/or driver for

some reason.  The reset is unconditional, there is no error return.

If SendPacket always returns busy (non-zero), give this a try.

Note that you must pass down the actual handle you received

during RegisterEthertype.

Using ReleaseInterLanAT()

void ReleaseInterLanAT(void);

Your application absolutely MUST call this routine before

returning to DOS.  Failure to do so will hang your machine.

Fleshing out ReceiveProcedure()

ReceiveProcedure will be passed a (ptr to a) RcvBlock structure

on the stack.  Simply being called indicates the presence of a frame

with one of your ethertypes in the receive FIFO of the InterLan AT

card.  You have the opportunity to examine the first 60 bytes of the

in the look-ahead buffer before deciding whether or not the driver

should copy the frame into one of your receive buffers.

Your routine must place either the value NULL or a valid packet

buffer address into the RcvBlock 'buffer' field.  If the address

is non-NULL when your routine returns to the driver, the driver

will copy the packet into the non-NULL address, then call your

receive routine AGAIN, this time with the 'function_flag' field

of the RcvBlock set to COMPLETE.  (There is no second call if

the 'buffer' field is NULL upon return to the driver.)

Since your receive routine is being called (twice!) inside of an

interrupt handler, it must be extremely brief and efficient.  It

should do little more than set and clear semaphores for coordinating

with your foreground application.  Since DOS and the BIOS are NOT

re-entrant, calling their services (or a routine that calls their

services) can deadlock the machine running your application.  Use

as little stack as you can.  The toolkit stack has been made 256

bytes in size as a safety net.  Code as if there is no net, however.

Enabling interrupts is strongly discouraged.

Upon entry to your C receive routine, the RcvBlock will describe

the incoming packet.  'len' will contain the actual packet length,

'lookahead_len' will contain the current lookahead length of the

buffer pointed to by 'lookahead_buf'.  'function_flag' will be

either the value INSPECT or COMPLETE.  You should remember to place

the length of your receive buffer into the 'buffer_len' field of the

RcvBlock before returning to the driver, if you want the packet.

On the second (completion) call to your C receive routine, the

RcvBlock 'bytes_copied' field will indicate how many bytes were

copied into your buffer.

Each time your application finishes with a receive buffer, it

should make it available for consumption by ReceiveProcedure().