AI-LANBIOS Low-Level Driver For E3000II Pocket Ethetnet Adapter (ADDK.DOC)

--------------------------------------------------------------------------

INTRODUCTION

============

The ADDK.EXE low-level AI-LANBIOS driver allows you to use your

E3000II Pocket Ethernet adapter in a LANtastic network.

In order for a E3000II Pocket Ethernet adapter to function with the

LANtastic Network Operating System (NOS), you must first install the

adapter as instructed in your adapter installation guide, then run the

supporting software (ADDK.EXE and AILANBIO.EXE).

ADDK.EXE communicates with the adapter, while AILANBIO.EXE provides

high-level communications across the local area network.  Because of this,

you cannot run any network software until after you load ADDK.EXE and

AILANBIO.EXE.

Note: For information on running AILANBIO.EXE, refer to AILANBIO.DOC

on your LANtastic/AI diskettes.

ADDK.EXE V5.0

==============

There are command line switches that you can use when you run ADDK.EXE

and AILANBIO.EXE.  These switches allow you greater flexibility in using

these programs.  For information on the AILANBIO.EXE command line options

refer to AILANBIO.DOC on your LANtastic/AI disks.  For information on

ADDK.EXE command line switches, refer to "ADDK.EXE COMMAND LINE OPTIONS"

in this file.

RUNNING ADDK.EXE

================

The syntax for running ADDK.EXE is

ADDK[switches . . . ] [ ; comment]

where "switches" denotes any optional command line switches.  If you are

using more than one switch in a command line, use a space or a forward

slash (/) to separate the arguments.  If a switch takes a value (such as

when you designate the DOS multiplex number with the MPX= switch) the

switch must be followed by an equal sign (=) or a colon (:).  All switch

values are checked to make sure they are valid.  Illegal values produce

error messages.

These are the valid switch formats:

SWITCH

/SWITCH

SWITCH:VALUE

/SWITCH=VALUE

You would use one of the first two formats for a switch that does not take

a value.  For example:

ADDK/HELP

You would use the third or fourth formats for a switch that takes a value.

For example:

ADDK/MPX=C8

ADDK.EXE COMMAND LINE OPTIONS

=============================

The valid ADDK.EXE switches appear below.  The letters "ddd"  denote

that you must provide a decimal number.  The letters "hh" denote a

required hexadecimal value.  Values enclosed in brackets "[]" indicate

default values if the switch is omitted.  Values after the brackets denote

the valid range of variables for the switch.

----------

@switch-file

This option specifies that further switches are to be taken from a

switch file.  Any switches after the @switch-file name will be used,

and the switch settings take precedence over any settings in the switch

file.  You may invoke switch files from within switch files as often as

you wish.  The file should contain valid switches and may contain comment

characters (;) at the beginning of each line or after switches.

For example,

ADDK @setup

The file SETUP contains

; ADDK AI-LANBIOS(R) driver setup file

;

XEROX                 ; Use packets in the XEROX format

VERBOSE               ; Display verbose information

----------

HELP or ?

This switch instructs ADDK.EXE to display information about the valid

command line switches, but does not install the ADDK.EXE driver.  For

example,

ADDK/help

ADDK ?

See the messages section for a detailed description of the HELP display

output.

----------

XEROX

This switch causes all transmitted packets to be in the XEROX format

instead of the IEEE 802.3 standard.  When using this switch ADDK.EXE

will receive both XEROX and IEEE 802.3 packets. This switch is the default

as of the release of LANtastic/AI V5.00.

----------

IEEE

This switch causes all transmitted packets to be in the IEEE 802.3 format

instead of the XEROX standard.  When using this switch ADDK.EXE

will receive both IEEE 802.3 packets and XEROX.

MPX=hh [C7] C0-FF

This switch specifies the DOS multiplex (MPX) number to use for

communication with AILANBIO.EXE.  You will need to change this value only

if the default MPX number is in use by another application or NETBIOS.

Multiplex numbers 00 through BF are reserved for DOS use; therefore, you

must use a multiplex number between C0 and FF.

----------

PACKET_SIZE=dddd [1500] 600-1500

This switch specifies the maximum size of transmitted IEEE 802.3 packets.

You may need to change this value when using certain bridges that do not

support the full 1500-byte packet size.  Even when you reduce the packet

size, the low-level driver can still receive larger packets.

----------

REMOVE

This switch removes ADDK.EXE from memory.  You must remove TSR programs

in the opposite order they were loaded.  For example, if you loaded

ADDK.EXE and then AILANBIO, you must remove AILANBIO before ADDK.EXE

In addition, if you have more than one invocation of ADDK.EXE and

AILANBIO you must use the MPX= switch on the command line to specify which

copy of ADDK.EXE and AILANBIO that you want to unload.  For

example, if you load two copies of ADDK.EXE and AILANBIO:

ADDK.EXE/MPX=C7

AILANBIO/ADAPTER=0/MPX=C7

ADDK.EXE/MPX=C8

AILANBIO/ADAPTER=1/MPX=C8

To unload these copies of ADDK.EXE and AILANBIO:

AILANBIO/REMOVE/MPX=C8

ADDK.EXE/MPX=C8/REMOVE

AILANBIO/REMOVE/MPX=C7

ADDK.EXE/MPX=C7/REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

----------

VERBOSE

This switch causes the ADDK.EXE driver to be installed and detailed

information about the configuration of the ADDK.EXE driver to be

displayed.  See the messages section for a detailed description of the

VERBOSE output.

----------

USING MULTIPLE ADAPTERS

=======================

ADDK.EXE and AILANBIO.EXE communicate with each other using a DOS

multiplex (MPX) number.  Each ADDK.EXE and AILANBIO.EXE pair supports

one adapter.  You will need to run an additional copy of ADDK.EXE and

AILANBIO.EXE for each additional adapter installed in your computer.

Using command line switches, you must assign each ADDK.EXE and

AILANBIO.EXE pair a unique multiplex and adapter number.  You must also

make sure each copy of ADDK.EXE has a unique IRQ line and I/O port address.

For example:

ADDK

AILANBIO

ADDK/MPX=C8

AILANBIO/MPX=C8/ADAPTER=1

The first invocation of ADDK.EXE and AILANBIO.EXE loads into the first

adapter using the default MPX number (C7) and adapter number (0).

The command line switches in the second invocation instruct the NETBIOS

to load into the second card, (adapter number 1), and DOS MPX number (C8).

ADDK.EXE MESSAGES

===================

The ADDK.EXE driver produces two types of messages: Informative and

Error.  All error messages are preceded by the text "ERROR:" and in those

cases the ADDK.EXE driver is not installed.

--------

MESSAGE:

AI-LANBIOS ADDK driver V5.0  - (C) Copyright 1993  CO., LTD.

This message is displayed each time the low-level driver is invoked.

--------

MESSAGE:

---- ADDK driver installed ----

This message is displayed after the low-level driver is successfully

installed.

--------

MESSAGE:

---- ADDK driver NOT installed ----

This message is displayed if the low-level driver is not installed.  The

driver will not be installed if HELP information is displayed or an error

occurred.

--------

MESSAGE:

Command line             /verbose

IEEE 802.3 node address  00006E20CE36   Network packet size      1500

MPX interface number     C7             IO base address          0378

Interrupt request (IRQ)  5              Packet type              XEROX

Bytes of memory used     7617           Data Mode :

This message is displayed if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.

--------

MESSAGE:

Valid command line switches:

HELP

XEROX

IEEE

MPX= Range C0 to FF hex

PACKET_SIZE= Range 600 to 1500 decimal

REMOVE

VERBOSE

?

@ Range 0 to 2048 byte file

This message is displayed when the HELP or ? switch is used.  Each valid

switch is listed with the acceptable range of values (if any) and the base

(hex or decimal) that the numbers must be entered in.  The acceptable file

size range for indirect files (@) is also listed.

--------

MESSAGE:

ADDK not loaded

You have issued the ADDK/REMOVE command and do not have ADDK.EXE

loaded into memory.  Only issue this command after the program is loaded.

-------

MESSAGE:

---- ADDK removed ----

The ADDK.EXE driver has been removed from memory.

-------

ERROR:    Can't REMOVE -- ADDK interrupts re-hooked

Meaning:  You have attempted to remove ADDK.EXE and the interrupts are

in use by another TSR, or ADDK.EXE was not the last TSR

loaded.

Remedy:   Remove any TSRs loaded after ADDK.EXE.  For example, if you

load ADDK.EXE then AILANBIO.EXE, you must first remove

AILANBIO.EXE, then you can remove ADDK.EXE.

-------

ERROR:    Installed ADDK is different version

Meaning:  You have loaded a version of the ADDK.EXE program and

attempted to remove it from memory with the /REMOVE switch.  The

version with which you are attempting to remove ADDK.EXE is

not the same version that was loaded.

Remedy:   Remove ADDK.EXE with the same version of the program that was

loaded.

------

ERROR:   Cannot open indirect file - @file

Meaning: The file "file" cannot be opened as an indirect file.

Remedy:  The file must exist before it can be used as an indirect file.

------

ERROR:   Illegal character after switch name - ??????xnnnn

Meaning: The switch ?????? is followed by a character other than "=" or

":" or the switch does not take a value.

Remedy:  Only use the characters "=" or ":" to specify switch values and

do not place characters after switches that do not take values.

------

ERROR:   Illegal digit in switch value - ??????=nnnn

Meaning: An illegal digit (character) was encountered in the numeric

switch value.

Remedy:  You must restrict numeric decimal values to the numbers 0-9.

You must restrict hexadecimal values to the numbers 0-9 and the

letters A-F.

------

ERROR:   Illegal switch - ??????=nnnn

Meaning: The switch ??????=nnnn is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

------

ERROR:   MPX number hh is already in use - Try another number

Meaning: The multiplex (MPX) interrupt number hh is being used by another

application or low-level driver.

Remedy:  Try another MPX number.

------

ERROR:   No network hardware found at I/O base address hhhhH

Meaning: ADDK.EXE did not find an adapter at the I/O port address

specified on the command line.

Remedy:  Make sure the adapter is correctly installed and that the IOBASE

command line switch corresponds to the I/O base jumper setting on

the board.

------

ERROR:   Incorrect IRQ jumper setting

Meaning: The jumper setting of the adapter doesn't match the value

given on the command line.

Remedy:  Make sure the command line IRQ switch matches the IRQ jumper

setting of the adapter.

------

ERROR:   Switch value not in range - ??????=nnnn

Valid range is ssss to eeee

Meaning: The value nnnn is not in proper range.  The value "ssss"

represents the lowest acceptable value and the value "eeee"

represents the highest acceptable value.

Remedy:  Limit your range of values to the acceptable range.

TESTING ADDK.EXE RETURN CODES (ERROR LEVELS)

==============================================

You may test the ADDK.EXE return code or error level in a batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No error encountered.  ADDK.EXE driver installed

1     MPX number is in use

2     Switch error

3     No network hardware present

4     ADDK not loaded

5     Can't remove, different version loaded

6     Cannot remove, interrupts re-hooked

7     ERROR: Can't REMOVE -- AI-LANBIOS (R) still installed

------------------

AI-LANBIOS(R) is a registered trademark of Artisoft, Inc.