*****************************************************************

* 8-bit / 16-bit Ethernet Adapter diagnostic reference guide    *

* Copyrights Registered (C) 1992       All rights reserved.     *

*****************************************************************

This diagnostic program, running under DOS System, helps you test

our Ethernet LAN card easily. (Note: Ethernet Lan card connecting

with cable is required)

This program was designed for the BNC/AUI ports only, not use for

RJ-45 twisted pair. So basically you can use in our Ethernet and

COMBO Ethernet in BNC and AUI ports during test diagnostic. If you

test on RJ-45 twisted pair port will report an error message on screen.

The diagnostic program will perform following functions:

. check Ethernet card universal ID

. check I/O base address & decorder

. check boot ROM address & decorder (enable/disable)

. check on-board packet buffer memory & size

. check on-board LAN chipset function (NIC, SNI, CTI/TTI)

and it will show the following messages (message explanation):

Message (M):

Condition (C):

Explanation (E):

Solution (S):

M:      (No any message shown in screen except company, product name etc.)

C:      The PC system just hang up and keyboard is no action.

E:      There maybe main board compatible problem or the decorder PAL is

error.

S:      1. Change main board compatible jumper (only 16-bit Ethernet card).

2. It maybe decorder PAL error.

(U7/8-bit card; U2 & U11/16-bit card; U2 & U12/10BaseT card)

M:      This diagnostic program doesn't support WD-Compatibles.

C:      The test procedure will stop and return back to DOS prompt.

E:      Your LAN card is an WD Ethernet card or its compatible card.

M:      The Ethernet card serial ID is xxxxxxxxxxxx.

C:      The test procedure will continue.

E:      Show Ethernet card ID.

M:      The I/O port base is xxxh ----- testing OK

C:      The test procedure will continue.

E:      It find the card I/O base and check the I/O decorder is no problem.

M:      I/O port test failure !

C:      The test procedure will stop and return back to DOS prompt.

E:      Can not find proper I/O port (300h, 320h, 340h, 360h)

S:      1. It maybe decorder PAL error.

(U7/8-bit card; U2 & U11/16-bit card; U2 & U12/10BaseT card)

2. It maybe LAN controller error (8390 or 83901).

3. It maybe TTL IC error (74LS374 etc.)

M:      No remote boot ROM in socket or ROM disable.

C:      The test procedure will continue.

E:      Find no boot ROM code.

S:      1. If there is a ROM on board, please check the enable jumper.

2. If there is a ROM on board, it maybe TTL (74LS521) error.

M:      A remote boot ROM enable and address is 0xx00 : 0

C:      The test procedure will continue.

E:      Find boot ROM code and display the memory address in screen.

S:      1. If the memory address shown is not the LAN card setting,

please check other interface card having ang boot ROM.

2. If no other boot ROM in your system and the address is not

correct, please check the jumpers or TTL (74LS521).

M:      The memory buffer xxK bytes ----- testing OK

C:      The test procedure will continue.

E:      Check the on board memory size and write/read to/from them.

M:      Memory buffer test failure !

C:      The test procedure will stop and return back to DOS prompt.

E:      The memory buffer can not pass the read/write test or PROM read error.

S:      1. It maybe memory error.

2. It maybe decorder PAL error.

(U7/8-bit card; U2 & U11/16-bit card; U2 & U12/10BaseT card)

3. It maybe LAN controller error (8390 or 83901).

4. It maybe TTL IC error (74LS374 etc.)

5. PROM ID data error.

Don't replace PROM on 16-bit card with PROM on 8-bit card.

Don't replace PROM on 8-bit card with PROM on 16-bit card.

M:      NIC 8390/83901 loopback test OK.

C:      The test procedure will continue.

E:      NIC pass the test.

M:      NIC 8390/83901 loopback test failure !

C:      The test procedure will stop and return back to DOS prompt.

E:      NIC can not pass the test.

S:      1. To pass this test, a cable must connect to LAN card.

2. NIC is error, replace it.

M:      SNI 8391/83901 loopback test OK.

C:      The test procedure will continue.

E:      SNI pass the test.

M:      SNI 8391/83901 loopback test failure !

C:      The test procedure will stop and return back to DOS prompt.

E:      SNI can not pass the test.

S:      1. To pass this test, a cable must connect to LAN card.

2. SNI is error, replace it.

M:      CTI 8392 or TTI 92C03/83C94 loopback test OK.

C:      The test procedure will continue.

E:      CTI or TTI pass the test.

M:      CTI 8392 or TTI 92C03/83C94 loopback test failure !

C:      The test procedure will stop and return back to DOS prompt.

E:      CTI or TTI can not pass the test.

S:      1. To pass this test, a cable must connect to LAN card.

2. CTI or TTI is error, replace it.