CRTC  version 2.1 (dated oct 23, 1994)

Shareware Version

(c) 1993 Frank Knot

Javalaan 43a

9715 GR Groningen

++31-50-772198

internet: not available at this moment

0) Introduction

Have you had this problem ?

When switching form text to graphics, or to super VGA mode, your

carefully centered display all of a sudden has a big black border

on one side and a negative border on the other side.

What is the solution

1) Buy a digital controlled monitor of at least $750

2) A software solution is more elegant than a hardware solution, so try

CRTC, that will cost you barely anything. It might do the same for you!

1) What is CRTC ?

CRTC is a small tsr program that traps the int $10 (video interupt)($10 is

Borland Pascal notation for hexadecimal nr's, used throughtout this document)

and sets the CRTC-parameters (actually the 5 CRTC registers 0,3,4,5,6), so

that the image on the monitor fills the entire screen of the monitor. It

reads a parameterfile with the according CRTC-parameters for every video

mode, and changes them with every 'set video mode' or 'set VESA video mode'

bios call.

2) Can I use CRTC ?

Yes !, CRTC is designed to be adjustable to any kind of video card,

including all tricky non-standard video modes and all VESA modes

If a mode has a mode nr, you can adjust it with CRTC. If however the mode

is set via another mechanism, you are in trouble. This program supports

only mode changes via the standard set-VGA-mode or set-VESA-mode

interrupt call.

3) Installing CRTC on your system

You must assure that there is a CRTC.dat in the same dir as CRTC.com

resides (I delivered a sample file), and then just run CRTC. If the program

is run from DOS versions below 3.0, you have to put the CRTC.dat file

in the current directory. I suggest that you put CRTC somewhere in your

autoexec.bat, so the CRTC.dat file must reside in the root dir of your

boot disk if you run DOS 2.x. DOS versions higher than 3 may have the CRTC.dat

file in the same dir as the CRTC.com, or in the current dir. If the file

CRTC.dat can't be found, the program will exit with an appropriate error message.

If all video modes that your card has show perfect now, you are finished.

If not, you have to find the appropriate line for the video mode in the

CRTC.dat file, and edit the coefs accordingly. Or see section 5 to

use the CRTCread program to obtain valid coefs. Please read this manually

carefully before running the CRTC.com program !!!

4) The CRTC.dat file

This is a file with on every line 7 numbers, meaning

video-mode-nr   VESA-mode-nr   r0   r3   r4   r5   r6   any_comment

All numbers must be separated with a space, and after the r6-number

there can be any comment, which also has to be separated from r6 with

a space. Missing numbers on a line causes the program to continue on the next

line, resulting in wrong numbers. If there are more video modes

than line no's (the full version supports 30 modes, the shareware version

only 5), the program detects empty lines, and fills the mode entry in

memory with -1, not a valid mode. (so no problems with empty lines)

There is hardly any check on the inputfile, so make sure that information

in the file is meaningfull. Hexadecimal numbers can be used by preceding

them with a $. (Also, if a video-mode doesn't have a VESA-mode nr, give

it VESA-mode nr $FF (in 2-complement this means -1, the invalid entry).

Please specify only numbers from 0 to 255, or from $0 to $FF, because

only one byte is used. The VESA mode nr's here specify only the least

significant byte of the VESA mode nr (because they all are in

between $100 and $17F, resulting in values $00-$17).

5) Handy ways to obtain the right parameters

Because the CRTC.dat file supplied has been made for my video card and

monitor (Cirrus Logic 5426 + Wescom GW500E), it probably won't be appropriate

for any other system. In particular, the VGA video-mode-nr's > $14 are highly

card specific. Furthermore, the params may not be right. For this, I supply

you 2 programs, CRTCread.exe and CRTCadj.exe. The CRTCread program can be

used to obtain the current parameters for a certain mode. With these values,

you can vary the parameters with CRTCadjust, a tsr that allows you to change

the registers via hotkeys. Combining the data yields the right values

for the registers, which can then be placed in the CRTC.dat file. After

booting, the mode should look fine.

6) How to use CRTCREAD

CRTCread is an easy to use filler utilitie for the CRTC.dat file.

After starting the program, it asks you if you will specify a VGA or VESA mode

number, after which it asks the mode nr (Remember that the programs VESA

mode nr's have only the least significant byte of the real mode, so they lie

in between $00 and $7F). After this, the program tries this mode and returns

it's register values. It also presents you both the VGA and the VESA mode nr

(if appropriate, if a VGA mode nr is entered, which has no corresponding VESA

mode nr, VESA mode is returned to be 255 ($FF)). After this, a question is

asked wether you want to add this mode as a line in the CRTC.dat file. If yes,

you can add a comment, and the program adds the line. Remember that the

program does not delete previous lines. Another problem is that there isn't

a onen tot one mapping between VESA and VGA modes, so check the numbers.

7) How to use CRTCADJ

This program is a tsr that gives you control over the 5 registers by the use

of hotkeys. You can use it by having another program selecting a video mode

(and filling the sreen with a nice picture) (eg Cshow or Vpic), and then by

pushing <ctrl> together with one of the keys 1-5 you select a register to

change; while pushing <ctrl> down and <ctrl> up changes the selected

register with resp one down and one up. In this, <ctrl> 1 selects r0,

<ctrl> 2 selects r3, <ctrl> 3 selects r4, <ctrl> 4 selects r5 and

<ctrl> 5 selects r6. After this, you have to update the CRTC.dat file by

yourself with the appropriate changes!. You will find that in text mode

screens, it is illegal to change r0 without changing r3, while in graphics

mode screens, it is not necessary to change r3. Remember to reboot your

computer after this.

8) How does it work (technical bla bla).

CRTC is a program originally written in Borland Pascal, but from version 2.0

it is written fully in assembly. It reads the file CRTC.dat in an internal

array, and then traps the int 10h. The interrupt handler calls the original

int 10h, and then checks if register ah = 0 (set video mode) or ah = $4f and

al = $02 (set VESA video mode). If either one is the case, it searches the array

if there is data in it for the particular video mode set. If so, it changes

registers 0,3,4,5,6 of the CRTC registers with the values provided by CRTC.dat.

In this (shareware) version, there can be at most 5 video modes read

from CRTC.dat, the rest is ignored. Also, if a mode is set by a nonstandard

mode setting opperation (I noticed the Compaq Qvision does this for it's

Hi-color modes), those modes cannot be adjusted by CRTC.

VESA modenr's are:

$100	640x400x256

$101	640x480x256

$102	800x600x16

$103	800x600x256

$104	1024x768x16

$105	1024x768x256

$106	1280x1024x16

$107	1280x1024x256

$108	80x60 text

$109	132x25 text

$10A	132x43 text

$10B	132x50 text

$10C	132x60 text

$10D	320x200x32K

$10E	320x200x64K

$10F	320x200x16M

$110	640x480x32K

$111	640x480x64K

$112	640x480x16M

$113	800x600x32K

$114	800x600x64K

$115	800x600x16M

$116	1024x768x32K

$117	1024x768x64K

$118	1024x768x16M

$119	1280x1024x32K

$11A	1280x1024x64K

$11B	1280x1024x16M

It might be a good thing to print this list and try to obtain the VGA mode

nr's, from any kind of documentation. They should be the same as CRTCREAD

says. Also, it might not be neccessary to use a VESA mode nr, as most cards

perform the VGA-set-mode call if they have to set the VESA mode by calling

int 10 again. So VESA support might not be neccesssary. Windows drivers

use the VGA-set-mode call. If CRTC behaves strange for you (bug?),

or you have information that might interest me, please contact me on the

adresses below

8) Troubleshooting

If CRTC doesn't load, make sure there is a valid CRTC.dat in the dir

you call it from. Don't try loading it in a dos box under windows,

tsr's aren't designed for that. If your monitor does strange things when

starting up, try to change the inititial parameters to the original state of

your card. That is, supposing you start it from the dos text screen (mode 3),

get the standard parameters of your video card by using CRTCread,

selecting VGA-mode 3 (after rebooting ofcourse, without CRTC in memory).

Then use CRTCadj to obtain better parameters, and change the CRTC.dat file

accordingly. Change the line in the CRTC.dat file starting with 3 or $3 to

match your 5 parameters. Then start CRTC again, and you should see it

starting with a screen change (because the program loads the new parameters).

You can follow this procedure with any other mode then afterwards. The

program checks for double loading, by checking all memory for a string in

the program. This might not work always, but does most times.

-----------------------------------

PLEASE REPORT (POSSIBLE) BUGS TO ME

-----------------------------------

9) Revision history

oct 31, 1994 : Version 2.1

Fixed a bug that prevented VESA suport from working

(It didn't do anything on a VESA set video mode call,

because of a too early register clearance)

I didn't notice it because the VESA bios extensions

on the Cirrus 5426 call the VGA set video mode interrupt

on a VESA set video mode interrupt call, which did the

work for me. I hope it works fine now.

Fixed a bug on compaq computers (among others) who can

give arbitrary results for the 8th bit of a get video mode call.

(Only 7 bits are significant). I now mask out the eighth bit.

Fixed a bug that prevented the initial adjustment of the current

mode

jan 14, 1994 : Version 2.0

Fully rewritten in assembly language

Resident memory usage is now only 0.7 K

Changed double loading check by searching memory

Changed CRTCread and CRTCadj to be more user friendly

dec 18, 1993 : Version 1.6

Changed double loading check so it not anly check register 0

of mode 3, but also register 6 (register 6 is easier to change)

nov 24, 1993 : Version 1.5

Reduced memory usage by removing beeps

Added check for existance CRTC.dat file

nov 11, 1993 : Version 1.4: fixed bug with video mode 0 (yes, it does exists)

Invalid modes are now represented by mode $FF

(and that mode doesn't exists)

oct 31, 1993 : Version 1.3: first shareware release

Added CRTC.dat file

aug 1993     : Version 1.2: never released, but extensively used

CRTC register tables internal

jul 1993     : Version 1.1 : first version, never used nor released.

10) Other programs written by me

Recently, version 1.1 of timeset saw the light. This is a utility that runs

once a day (by checking it's own filedate), and adjusts the system clock with

any user specified number of seconds. Furthermore, it has an optional parameter

to run another program once a day, so if you like murphyism's or lawyer's

jokes, but not every time you reboot, try this program! Timeset is available

in the archive timset11.zip, timeset11.arj or withe a higher version stamp.

You will probably find it on a Bulletin Board near you, otherwise you can

get it at TATORT BBS in Groningen (the spectrum emulator BBS) at ++31-50-264840

I am also a registered user there, so you can reach me there too.

11) Legal stuff

CRTC Version 2.1 is Shareware. It is NOT FREE SOFTWARE, but it is also

NOT Crippleware.  It is fully functional, but for the 5 video modes

restriction. If you like CRTC and find it useful, you are requested

to support it by registering.  All suggestions, comments, criticisms,

and modification requests are welcome -- those coming from registered

users will have priority.

This program is being made available on an "as is" basis, and carries no

warranties, express or implied.  The author shall in no way be held liable

for any damages resulting from the use of this program or the media on which

it is distributed, including, without limitation, loss of business profits,

interruption of business, loss of information, damage to equipment, or any

other incidental or consequential damages.

If you register the program, you will get the source code of the most recent

version of the program, without the 10 modes upper bound. Furthermore, you will

receive the source of the utility programs CRTCadj and crctread. The

registration will cost you only f20,- for dutch users, or $15 for users

everywhere else on this beautiful world. As a bargain, I offer you a dual

registration (Timeset + CRTC ) for only f 25,- ($20). See Timeset.doc

for details.

CRTC Registration Form

Version 2.1

[ ] Registration ($15)                                           ______

[ ] Registration with Timeset included                           ------

Check one: [ ] 5 1/4" crunchy floppy

[ ] 3 1/2" nice rigid floppy

[ ] Other (tokens of esteem, unclaimed inheritances, ...)        ______

Total:     ______

Name:     _________________________________________________________________

Address:  _________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Comments: _________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

All funds should be in Dutch guilders or US dollars -- cash, check (drawn

on dutch banks only), money order, or postal money order. No credit cards

please. Users in Holland can also pay by 'girobetaalkaart' or 'Eurocheque'

or by depositing on gironr 5034479.

Please make all checks payable to Frank Knot, and forward your payment

along with this form to:

Frank Knot

Javalaan 43a

9715 GR Groningen

gironr. 5034479