SVADI.DOC - User Manual for SVADI Super VGA Driver

DOS Real and Protected Mode ADI 4.0/4.1 Driver

Last Updated: May 04, 1992 (v1.6f)

FILES:

svadi.exe       - real mode ADI 4.0/4.1 driver

svadidl.exe     - real mode ADI 4.0/4.1 driver with display list

rcpsvadi.exp    - protected mode ADI 4.0/4.1 driver with display list

svadi.cfg       - A file containing the driver's configuration data.

svconfig.exe    - The configuration utility for svadi.com.

svadi.doc       - This documentation file.

Svadi.Doc Contents:

Introduction

Explanation of Notation

1 Installation and Operation

1.1 Installation

1.2 Deciding Between the Real and Protected Mode Drivers

1.3 Loading and Unloading the Real Mode Driver

1.4 Linking to the Protected Mode Driver

2 Configuring the Autodesk Products for Use with Svadi

2.1 Deciding Between ADI 4.0 and 4.1

2.2 Configuring AutoCAD for Use with Svadi

2.3 Configuring AutoShade/RenderMan for Use with Svadi

2.4 Configuring AutoSketch for Use with Svadi

2.5 Configuring 3D Studio for Use with Svadi

3 Svadi Configuration

3.1 The Configuration Data File (SVADI.CFG)

3.2 Global Reset and Default Config Values

3.3 Configuration with SVCONFIG

3.4 Configuring Svadi for Use with AutoCAD

3.5 Configuring Svadi for Use with AutoShade/RenderMan

3.6 Configuring Svadi for Use with AutoSketch

3.7 Configuring Svadi for Use with 3D Studio

4 Screen Text

4.1 Text Fonts

4.2 Configuring Screen Text Areas

4.3 Dual Screen Operation

4.4 Display/Text Flipscreen Save/Restore

5 Colors

5.1 320x200 x 256 colors mode

5.2 Logical and Physical Colors

5.3 AutoShade Logical Colors

5.4 Drawing Colors in 16 Color Modes

5.5 Drawing Colors in 256 Color Modes

6 VGA Cards

6.1 VGA Display Memory Requirements

6.2 VGA Modes and Monitor Frequencies

7 Known Problems

7.1 AutoShade 386 1.1 Pull-Downs and Dialogue Boxes not Restoring Properly

7.2 AutoShade 1.1 Text Colors not Configurable

7.3 AutoShade Rendering in 16 Color Modes

7.4 Rendering Colors not Matching Wireframe Colors

7.5 Crosshairs Hidden By Some Colors in 256 Color Modes

7.6 Pull-downs not Correctly Replaced When Side Menu Width > 10

8 Troubleshooting

8.1 Driver Not Installed (Real Mode ADI)

8.2 Protected Mode Driver Not Found

8.3 Flipscreen (F1 function key) Not Restoring Graphics Screen

8.4 Loss of Graphics Mode or Failure to Enter It

8.5 VGA Screen Goes Dim or Blank During Dual Screen Mode

8.6 AutoShade Menus Missing After Flipscreen

8.7 Rendering Not Restored on FlipScreen (Real Mode Only)

INTRODUCTION

------------

Svadi is a Super VGA ADI 4.1 display driver for AutoCAD, AutoSketch,

AutoShade and RenderMan.  It is used in place of the built-in display

driver for the IBM Video Graphics Array (VGA).  Its advantages over the

built-in VGA driver include:

o Super VGA resolutions.  The built-in driver operates at 640x480

dots in 16 colors.  Svadi supports 640x480, 800x600, 1024x768

and 1280x1024 resolutions, in both 16 and 256 colors, or whatever

subset of those is allowed by your particular VGA card.

o More flexible configuration.  Svadi allows you to change the size

and color of text areas, the text font size, flipscreen behavior,

the red-green-blue shades of display colors, and more.

o Dual screen operation.  If you have both VGA and monochrome

adapters in your system, Svadi can display text on the monochrome

and graphics on the VGA.

o Performance.  Svadi is approximately 15 percent faster at redraws

than the built-in VGA driver.

EXPLANATION OF NOTATION

-----------------------

Many of the examples below use DOS commands and filenames.  They

are surrounded by quotes, to indicate that the text within the quotes is

literally what you should type.  The quotes themselves are not part of

the command and should not be typed by you.  For example: "SVADI -U".

Filenames are generally shown in capital letters in the examples.

Of course, the DOS file system is case independent, so you can actually

use capitals or small letters.  This applies to environment strings, also.

That is, you can say "SET DSPADI=C:\rcpsvadi.exp" or "set dspadi=c:\rcpsvadi.exp".

1.1 INSTALLATION

----------------

Installation is simply a matter of copying the driver files

onto your hard disk:

1. Create a subdirectory on your hard disk which will contain

the driver files.  Eg:  mkdir C:\SVADI

2. Copy the files from the product diskette to the hard disk

directory.  Eg:  copy A:\*.* C:\SVADI /v

This will copy in both the real mode and protected mode drivers.

Even though you will probably end up using only one of these, it does

not hurt to have both installed.  The files are relatively small and they

don't interfere with each other.

It's not strictly necessary that the Svadi files live in their

own dedicated directory, but it is recommended for simplicity.  Some people

like to put everything into one big \ACAD directory.  That works too.

Note: The real mode driver is now a .EXE file.  Older versions

of the driver were .COM files.  To avoid confusion, you should delete

or rename any old copies of Svadi.  DOS gives precedence to the .COM

extension.  That is, if both SVADI.EXE and SVADI.COM are present,

then typing "svadi" will cause DOS to load the .COM file.

1.2 DECIDING BETWEEN THE REAL AND PROTECTED MODE DRIVERS

--------------------------------------------------------

The first question is whether you can run a protected mode driver

at all.  Your Autodesk product must be "DOS Extender based", such as

AutoCAD/386, in order for it to support protected mode drivers.  These

products only run on PC's based on the 386 processor and above (no 8086's

or 286's).  Consult the installation guide and other materials that come

with the Autodesk product.  They will tell you if it supports protected

mode drivers.

Typically, the products which support protected mode drivers also

will run with real mode drivers.  In this case, you can use either the

real or protected mode version of Svadi.  The two drivers are functionally

nearly identical, so it does not matter which one you use.  You can also

switch back and forth any time you wish.  Most people will want to use

the protected mode driver because it has these advantages:

o Automatically loaded by Autodesk program.  No fooling with

TSR's is necessary.

o Does not take any real (640k) memory.

o Is much faster at rendering flipscreens, when it can save

the image to extended memory while the real mode driver

saves it to a hard disk.

There is not much reason to prefer a real mode driver over a

protected one.  One case would be when you have another third party

application, such as a drawing viewer or a raster display tool that only

piggybacks onto a real mode driver.

1.3 LOADING AND UNLOADING THE REAL MODE DRIVER

----------------------------------------------

This section applies to the real mode driver only.  You can skip

it if you are only using the protected mode driver.

The real mode driver, SVADI.COM, is a DOS TSR program.  It needs to

be loaded into memory after you boot up and before you go into the Autodesk

product.  To load it, simply type the command "SVADI".  It can be unloaded

with the command "SVADI -U".  The driver will complain if you try to load

an already loaded driver or to unload a not loaded one.  When loaded, the

Svadi TSR takes up from 8k to 10k of DOS memory.

You may want to add a line to your AUTOEXEC.BAT file which loads

Svadi upon boot up.  Or you may have a batch file for invoking AutoCAD or

whatever which loads the driver before calling the product and unloads it

afterwards.  If you attempt to use the Autodesk product without the driver

loaded, then it will give you an error message about a missing driver and

will refuse to enter graphics mode.

A couple of other switches are useful for managing the real mode

Svadi driver.  Use "SVADI -?" to get a screen of help information.  You

can see where Svadi is loaded and how big it is, as well as some of its

configuration values with "SVADI -M".  To see just the configuration

values, use "SVADI -F".

If you will be frequently loading and unloading (and configuring)

Svadi, you may want to include its home directory in your PATH environment

string.  The PATH is typically set in your AUTOEXEC.BAT file, with a

command like "SET PATH=C:\DOS;C:\BIN;C:\ACAD;C:\SVADI".

1.4 LINKING TO THE PROTECTED MODE DRIVER

----------------------------------------

This section applies to the protected mode driver only.  You can

skip it if you are only using the real mode driver.

rcpsvadi.exp is the protected mode driver file.  It is not a TSR, and

you cannot run it as a program.  It is a protected mode executable image,

which is automatically loaded and unloaded by AutoCAD, AutoShade, etc

as needed.

You do need to tell the Autodesk product where to find the Svadi

protected mode driver file.  This is done by setting the environment string

"DSPADI" to the full name (including path) of the file.  For instance,

if rcpsvadi.exp is in the directory C:\SVADI, then you would issue the

DOS command "SET DSPADI=C:\SVADI\rcpsvadi.exp".  You may want to insert this

command into your AUTOEXEC.BAT file, so that it is done automatically

every time you boot up.  The DOS command "SET" displays a list of all

currently defined environment strings.  You can delete an environment

string by leaving off the right hand side, as in "SET DSPADI=".

For Autodesk products which perform rendering, ie. AutoShade and

RenderMan, there is an additional environment variable, "RDPADI".  Both

"DSPADI" and "RDPADI" must be set when using Svadi with a rendering

product, and they must both be set to the same filename.

Unfortunately, there are limitations in DOS that sometimes cause

it to not accept any more environment string definitions.  When that

happens, you get the message "Out of Environment Space" right after the

"SET ..." command is issued.  The easiest way around this is to put your

environment definition commands at the top of your AUTOEXEC.BAT file,

before any TSR's are executed.

2.1 DECIDING BETWEEN ADI 4.0 AND 4.1

------------------------------------

This version of Svadi is both a 4.0 and a 4.1 ADI driver.  The

Autodesk product will tell it which type of driver to be, so you do not

have to set anything in Svadi.  You do have to set the ADI level in the

Autodesk product.

Depending on the version of AutoCAD, AutoShade, etc, which you

are using, you may have the option to choose between different ADI

levels (4.0, 4.1, et. al.) during configuration.  You should select

ADI 4.1 when possible.  No harm is done by selecting ADI 4.0, except

that some newer display features of the Autodesk product may not be

available.

Alternatively, there may be a single pick for "ADI 4.0 and 4.1".

You should select that one.  Svadi and the Autodesk product will agree to

use the highest mutually supported ADI level, in this case 4.1.

2.2 CONFIGURING AUTOCAD FOR USE WITH SVADI

------------------------------------------

You need to tell AutoCAD to use the ADI driver as its display

interface.  First, get into the main AutoCAD menu.  Then select "Configure

AutoCAD" and then "Configure Video Display".  It will show you the currently

selected driver and ask if you want to change it.  Say "y".  You will then

be presented with a list of available drivers.  Typically, the ADI picks are

at the top of the list.  You will also see a pick for "IBM Video Graphics

Array".  This is the built-in VGA driver.  Perhaps you are currently using it.

Now you simply select ADI as your video display driver.  There may

be several ADI picks, with different levels (4.0 and 4.1) and modes (real

and protected).  You should pick the one you want, based on the discussions

of real vs. protected and 4.0 vs 4.1 above.

If you have selected a real mode ADI, then you will next be asked

what interrupt will be used.  The value you give here must match the one

you set during Svadi configuration (see below).  The recommended value is

interrupt 7a.

Next AutoCAD will ask you a bunch of questions about the screen

display, such as whether you want a status line and a command area.  These

are standard settings that apply to all AutoCAD display drivers, not just

to the ADI.  You should probably already be familiar with these values

and have them set as desired.

2.3 CONFIGURING AUTOSHADE/RENDERMAN FOR USE WITH SVADI

------------------------------------------------------

As with AutoCAD, you need to tell AutoShade/RenderMan to use the

ADI display interface.  You can enter the configuration menu by either

typing "SHADE -r" or by deleting the file SHADE.CFG and entering

AutoShade normally.  Be prepared to configure your pointing, printing

and other devices, also.  AutoShade does not currently allow you to

selectively configure just the video devices.

The two menus of interest to us are those for the display device

and the rendering device.  To work with Svadi, you must pick the ADI

driver in both of these.  The pick "Autodesk Device Interface" refers

to the real mode driver.  If protected mode support is available, it

will be listed as "P386 Autodesk Device Interface".  You must pick the

same entry for both the display and rendering devices (unless you want

to set the rendering device to "Null", effectively disabling rendering).

If you select the real mode ADI, then you will be asked for an

interrupt vector.  As with AutoCAD, the value you enter here must match

that you set when configuring Svadi (see below).  It is not necessary

that you use the same interrupt for both display and rendering, but

there is no reason not to.  The default value of 7a is recommended.

After selecting the rendering device, you will be asked a couple

of questions about the relation between the display and rendering screens.

Answer YES to the questions "do the devices share a single screen?" and

"is a redraw required on flipscreen".  If you do not answer YES to these,

then AutoShade/RenderMan will refuse to start up.  After the last

configuration menu, you should be launched into AutoShade.

Note also that it is possible to use Svadi as an AutoShade driver

and someone else's (or even another copy of Svadi!) as the AutoCAD driver.

You simply configure AutoCAD and driver A for one interrupt (eg 7ah)

and AutoShade and driver B for another (eg 7b) and they will peacefully

coexist.  It is not possible, however, to split AutoShade support between

two different drivers, ie. to use driver A for AutoShade's display

mode and driver B for AutoShade's rendering mode.

2.4 CONFIGURING AUTOSKETCH FOR USE WITH SVADI

---------------------------------------------

Currently, AutoSketch does not support protected mode ADI drivers,

so you will have to use the real mode Svadi.  Configuration is done by

typing "SKETCH -r" or by deleting the SKETCH.CFG file.  When you get to

the menu "Configure Display Device", select "Autodesk Device Interface

Display".  Again you will be asked for an interrupt number and it must

match the one that you set as Svadi's display interrupt vector.

2.5 CONFIGURING 3D STUDIO FOR USE WITH SVADI

---------------------------------------------

3D Studio uses protected mode ADI drivers only.  To configure

it for use with an ADI driver, you must edit the file 3DS.SET.  You

should add the following three lines (including quotes, where shown):

DEFAULT-DISPLAY = "RCPADI"

MAIN-DISPLAY = RCPADI

MATERIAL-DISPLAY = RCPADI

A semicolon is used to "comment out" lines in the 3DS.SET file.

You should comment out the old lines containing the DEFAULT-DISPLAY, etc

definitions.  You will also need to set the environment string RCPADI to

the full pathname of the Svadi driver file.

For more information, see the installation and user manuals on

3D Studio from Autodesk.

3.1 THE CONFIGURATION DATA FILE (SVADI.CFG)

-------------------------------------------

Svadi keeps its configuration information in a separate file called

"SVADI.CFG".  (This is a change from previous versions of Svadi.)  The

driver files themselves are not modified by Svconfig.  All variable

information is kept in this config file.  When Svadi is loaded, it looks for

and then loads the .CFG file.  Svconfig does the same thing.  If Svconfig

cannot find the file, then it asks you whether you want to create one.

If you only keep one copy of the SVADI.CFG file on your disk,

then your life will be simpler.  You may, however, desire to keep several

of them around, allowing you to change to different Svadi configurations

rapidly without having to go into the Svconfig utility.  This works, but

it can bring on confusion and frustration if you do not keep careful track

of which .CFG file is being used at all times.

The environment variable "SVADICFG" helps to deal with multiple

(or even single) .CFG files.  It should be assigned to the full path name

of the .CFG file you wish to use.  For example, you could add to your

AUTOEXEC.BAT file the command: "SET SVADICFG=C:\SVADI\SVADI.CFG".  Note

that the file does not even have to be named "SVADI.CFG".  You can name

it anything you want, as long as you set the SVADICFG environment string

to the same name.  Also, you can set SVADICFG to a directory name and

Svadi will look in that directory for a file called "SVADI.CFG".

Svadi and Svconfig search for the SVADI.CFG file in several

different places and use the first one that they find.  The places searched

are:

1. The SVADICFG environment string (if any).

2. The current directory.

3. The program's home directory (only for SVADI.COM and SVCONFIG.EXE).

4. The DSPADI environment string (only for SVCONFIG.EXE and the

protected mode Svadi).

It is highly recommended that you use method 1.  In fact, your AUTOEXEC.BAT

file should have four commands in a row, setting the SVADICFG, DSPADI,

RDPADI and RCPADI environment variables.

3.2 GLOBAL RESET AND DEFAULT CONFIG VALUES

------------------------------------------

The SVADI.CFG file is shipped in the "default" configuration, with

a display resolution of 640x480 in 16 colors and a rendering resolution of

320x200 in 256 colors.  It's recommended that you try the Svadi right away

after installing it, before trying other configurations.  Svconfig has a

pick in its main menu called "Global Reset".  This pick resets all of the

configuration parameters back to their original default values.

3.3 CONFIGURATION WITH SVCONFIG

-------------------------------

SVCONFIG.EXE is the program which lets you change the various

configuration values of Svadi.  As mentioned above, it operates on the

file SVADI.CFG (or whatever your SVADICFG environment string points to).

You may want to put SVCONFIG.EXE into a directory that is included in

your PATH.  Otherwise, you will have to change to its directory before

running it.

If you type "SVCONFIG -?", then you will be presented with a

brief help screen.  You can get a dump of the main configuration values

with the command "SVCONFIG -i".  If you invoke the program with no

switches, ie. as "SVCONFIG", then you are launched into the configuration

process.  You can override the default config data file name by supplying

the new name as an argument, as in "SVCONFIG SVOTHER.CFG".

Once in the configuration utility, you can navigate the menus

by using the arrow keys or the first letters of the menus' fields.  Every

menu pick has a couple of help lines at the bottom of it, indicating the

meaning of that pick and possibly the allowed values for it.  It is

now simply a matter of going to the fields which you wish to change,

keying in the new values and continuing until you're done.  Svconfig

does not actually update the config data file on disk until you exit

the program.  When you select the exit pick, it gives you a choice

of saving or discarding the changes, or staying in Svconfig.

Much of the rest of this document deals with configuration of

specific parts of Svadi.  The Table of Contents can help you find help

on the dial or knob you are interested in.  Don't think you have to

understand all of the various settings.  Many of them are cosmetic

in nature and can be safely ignored.

3.4 CONFIGURING SVADI FOR USE WITH AUTOCAD

------------------------------------------

AutoCAD works in two screen modes: display and text.  There is

no rendering mode.  Even when AutoCAD does "shading", it is using the

display mode (this might be a reason to use a 256 color display mode).

Any configuration values having to do with rendering (eg. the rendering

resolution, interrupt and flipscreen values) are irrelevant when you

are using AutoCAD.

When using the real mode Svadi, the interrupt vector must be

set to the same value in both Svconfig and AutoCAD (it's usually 7a).

Other than that, the only thing you have to make sure of is that the

selected display resolution is supported by your VGA card.

3.5 CONFIGURING SVADI FOR USE WITH AUTOSHADE/RENDERMAN

------------------------------------------------------

AutoShade/RenderMan works in two screen modes: display and

rendering.  It does not have a text mode.  Rendering mode is used,

obviously, for displaying rendered and shaded images.  Display mode is

used for the actual user interface, where you can traverse menus and

manipulate the settings before and after rendering.  The F1 key flips

the display between the display and rendering screens.

You need to set Svadi's display and rendering resolutions,

using the "Resolution/Modes" menu.  Note that the display and rendering

screens are independently configured.  Rendering should be done in

256 colors, not 16, since its quality depends on simulating smooth

changes in shades, and the more colors the smoother it looks.  In

general, a lower resolution rendering in 256 colors will look better

than a higher resolution one in 16 colors.  In fact, some drawings

will cause problems in AutoShade if rendered in 16 colors.  Also,

RenderMan will only work with 256 colors, and the SAVE IMAGE and

REPLAY IMAGE commands will only work with 256 colors.

The protected mode Svadi will always save the rendering image

when you flip back to the display screen and restore it when you return

to the rendering screen.  It uses AutoShade's memory as the swap space,

and you do not have to do any special configuration for this.

In real mode, however, Svadi must save the rendering image in a

DOS file.  You can specify its disk drive in the General Configuration

menu.  It is highly recommended that you set the save drive to a RAM

disk, since a hard disk will slow the flip screen down drastically.

The help window at the bottom of the screen will show the maximum free

space needed for saving the rendering image at the currently selected

resolution.

In real mode, it is also possible to disable the flipscreen

save altogether.  This will speed up the flipscreens, but you will be

faced with a blank image when you flip back to the rendering screen.

Also, you will not be able to use AutoShade's SAVE IMAGE command for

creating .TIF and other image files.  The enable/disable switch has

no effect on the protected mode Svadi.

There is no scroll area or side menu area used in AutoShade, so

their configuration values do not apply.  Since there is no text mode,

the dual screen setup also does not apply.  For real mode ADI, you must

set the display and rendering interrupts to match those that AutoShade

has been configured for.

3.6 CONFIGURING SVADI FOR USE WITH AUTOSKETCH

---------------------------------------------

AutoSketch does not currently support protected mode ADI, so you

will have to use the real mode Svadi.  It uses display mode only, there

being no text mode or rendering mode.  Thus, the configuration values for

dual screen operation and rendering modes do not apply.  AutoSketch also

does not use the scroll area or side menu, so you can ignore configuration

of those objects.

AutoSketch supports drawing colors 1 through 127, unlike AutoCAD

and AutoShade, which support 1 through 255.  Therefore you should set Svadi

to use a 128 color distribution instead of a 256 color spread.  See the

section below on configuring colors.

3.7 CONFIGURING SVADI FOR USE WITH 3D STUDIO

--------------------------------------------

Like AutoShade/RenderMan, 3D Studio uses a display screen and

a rendering screen (it also goes into text mode when you call up a

DOS shell).  It controls logical colors internally in both display

and render modes, so there is not much color configuration you can do.

The main thing to configure is the screen resolution and number of

colors.

Unlike AutoShade/RenderMan, 3D Studio does not have a "hot key"

for flipping between the rendering and display screens.  Nor does it allow

a "save image" operation.  Therefore, there is no reason for Svadi to

save the rendering image when it exits rendering mode.  You can disable

this save operation by entering the GENERAL CONFIG VALUES menu, and then

DISABLING the FLIP RENDER SCREEN SAVE option.  This may eliminate a slight

pause when you exit rendering mode.  If you use AutoShade/RenderMan, too,

then you should leave this save option enabled.

4.1 TEXT FONTS

--------------

The text font is a table of data in the VGA card's ROM which is used

to create the driver's message text characters on the screen.  Note that

this "message text" has nothing to do with "drawing text".  Message text

includes things like pull-down menus and command prompts (see section 2.2:

"Configuring Screen Text Areas" below), while drawing text is that which

you actually include in your drawing, using AutoCAD's TEXT command.

Svconfig allows you to select which font is used for Svadi's message

text.  The default is the 8 by 16 (8 pixels wide, 16 pixels high) font.

There are two smaller ones: 8x10 and 8x14, which probably don't have any

advantages over the 8x16, other than allowing a few more scan lines in the

drawing screen.  Then there is a larger one, 12x24, which is actually

created by stretching, or scaling, the 8x16 font.  It is intended for use

at screen resolutions of 1024x768 and above, when the smaller fonts become

relatively tiny and hard to read.

With the 12x24 font, however, there is the following "fine print":

First, because it is a scaled version of a lower resolution font, it will

not be as smooth and pleasing to the eye as might be expected.  Second, if

it is used at VGA resolutions lower than 1024x768 it will cause some

AutoCAD dialogue boxes and icon menus to not fit on the screen.

When you use Svconfig to change the resolution of Svadi's display

screen, the font size is adjusted if necessary to fit the new resolution.

In particular, if the font is set to 12x24 and you change to a display

resolution below 1024x768, then the font is automatically set to 8x16.

If the current font is 8x16 and you change to a display resolution of

640x350, then the font is automatically adjusted to 8x14 (thus using

the EGA font at the EGA resolution).

4.2 CONFIGURING SCREEN TEXT AREAS

---------------------------------

The three text areas in graphics mode are: the top status line, the

side menu bar, and the bottom scroll area.  The color of the characters

themselves is called the "foreground color".  The "background color" is of

course the color of the screen behind the characters.  Svconfig allows you

to change these logical colors.

These areas are turned on and off by configuring AutoCAD, but their

height and width is determined by Svconfig.  This is done in the "General"

menu.  You can pretty much set them to any value that fits on the screen,

but there are some special considerations regarding the status line width

and the side menu width.

If the status line is set to less than 37 characters wide, then

AutoCAD will abbreviate the status messages.  You may get "L:PLUMBING"

instead of "LAYER:PLUMBING".  Also, if you set it too wide, then the

coordinates may be truncated at the right edge of the screen.  The default

width of the status line is half the screen width, which works pretty well.

Changing the side menu width requires you to recompile AutoCAD's

menu files.  Menu compilation does, among other things, the formatting of

the menu entries for the given menu width.  To cause recompilation of a

menu, you simply delete its ".MNX" file.  AutoCAD will then recompile the

".MNU" file the next time you enter the drawing editor.  Since most menus

are probably designed for the standard 8 character width, it may not be

useful to go fooling with this value.  In addition, icon menus tend to

misbehave at menu widths smaller than 8.

4.3 DUAL SCREEN OPERATION

-------------------------

If your system has a monochrome controller and monitor (MDA), you

can run AutoCAD in "dual screen" mode.  In this mode, the AutoCAD prompts

and text messages appear in text mode on the monochrome screen and the

drawing in graphics mode on the VGA screen.  There is no F1 flipscreen

necessary.

In the General Configuration menu of Svconfig, there is a pick

for Dual Screen Mode.  This mode can take three values: ON, OFF and AUTO.

When it is set to ON, Svadi always runs AutoCAD in dual screen mode.

When it is set to OFF, Svadi always runs AutoCAD in single screen mode.

When it is set to AUTO, Svadi selects the mode based on what the current

screen is when you enter AutoCAD.  If MDA is current, then Svadi runs in

dual screen mode.  If VGA is current, then it runs single screen mode.

You can change the current screen from the DOS command line by

issuing the command "mode mono" to select MDA and "mode co80" for the

VGA.  You cannot change current screens when inside an AutoCAD Shell.

4.4 DISPLAY/TEXT FLIPSCREEN SAVE/RESTORE

----------------------------------------

In single screen mode, AutoCAD uses the F1 function key to toggle

between text and graphics mode.  There are also the TEXTSCR and GRAPHSCR

commands, and several others which cause AutoCAD to change to text or

graphics mode.  SVADI can save the graphics screen's contents when you

enter text mode, and then restore them when you go back to the graphics

screen.  The alternative is to have AutoCAD redraw the graphics screen

when you return to graphics mode, but this is usually a much slower

operation.  You can use SVCONFIG (see the General Configuration Menu) to

enable and disable the flipscreen save/restore capability.

When you do an AutoCAD shell, the graphics screen is always redrawn

upon returning to graphics mode.  This is because shell programs often modify

the VGA controller memory, thus destroying the saved graphics image.

5.1 320x200 x 256 COLORS MODE

-----------------------------

This mode is the only 256 color mode available in the standard

IBM VGA.  It is allowed in Svadi for the rendering display only.

The resolution is too low to support AutoCAD/Sketch/Shade menus.

If you want to work in 256 colors then you ought to get a super vga card

with 512k of RAM that can do 640x480x256 and 800x600x256.

5.2 LOGICAL AND PHYSICAL COLORS

-------------------------------

AutoCAD currently supports 256 drawing colors and thirty or so

non-drawing colors.  The non-drawing colors are those used by various

elements of the user interface, such as the menus, crosshair cursor

and graphics background.  These 256 drawing and thirty or so non-drawing

colors comprise the set of "logical" colors.

A VGA card can display 16 or 256 colors at a time, depending on the

mode you select.  These are referred to as "physical colors".  Svadi

translates logical colors from AutoCAD into physical colors on the VGA

display.  Svconfig allows you to define this mapping, so that basically

each logical color can be mapped to any of the physical colors.

To further complicate matters, each physical color is translated

by the VGA DAC (Digital to Analog Converter) also known as a LUT (Look-up

Table) into actual RGB (Red, Green, Blue) values.  RGB values are triplets

with each value between 0 and 63.  For example, (63,0,0) is the most intense

shade of red, (0,0,0) is black, and (0,40,40) is a cyan.  Svconfig allows

you to specify RGB values for all of the physical colors.

+---+   AutoCAD Drawing    +---+                                    +---+

| A |   or Non-Drawing     | S |   Physical    +---+     RGB        | V |

| C |   Logical Color      | V |    Color      | D |   Triplets     | I |

| A |  ------------------> | A |  ---------->  | A | -------------> | D |

| D |     (-33..256)       | D |   (0..15 or   | C |  (0..63, 0..   | E |

+---+                      | I |    0..255)    +---+   63, 0..63)   | O |

+---+                                    +---+

5.3 AUTOSHADE LOGICAL COLORS

----------------------------

The logical color settings do not work on versions of AutoShade

through 1.1.  Text will be drawn in color 1 (red) on a background that

matches the graphics screen background.

In version 2.0 of AutoShade the Svadi logical colors do work.

You can configure the colors for the pulldown menus, the dialogue boxes

and the top and bottom command lines.  Use the "Cmd Scroll Area" pick

to change the color of the top and bottom command lines.

5.4 DRAWING COLORS IN 16 COLOR MODES

------------------------------------

When the VGA is used in 16 color mode, it is normally set up to

provide 2 shades each of red, yellow, green, cyan, blue, magenta and

gray plus 1 shade each of white and black.  Since AutoCAD's drawing color

numbers range from 1 to 255 and AutoSketch's from 1 to 127, the obvious

question is: what do we do with all the extra drawing colors?

The lower drawing colors are firmly standardized (see

AutoCAD Reference Manual) as:

Background    0      1    2    3    4    5    6    7       8

Light    lt gray  red  yel  grn  cyn  blu  mag black  dk gray

Dark      black   red  yel  grn  cyn  blu  mag  wht   dk gray

In Svconfig's "Logical Colors" menu there is a selection

called "Drawing Logical Colors" which allows you to specify the way

in which drawing colors above 8 are laid out.  The selections are

1: Approximate 256 Mapping.  This is the default scheme.  Color 9 is white

(if light background) or light gray (if dark background).  Colors 10

through 249 are mapped to the "nearest available" color.  For example,

the 230's (maroon), 10's (red), 20's (reddish orange) and 30's (orange)

are all mapped to red or dark red.  The dimmest colors, 18, 19, 28, 29,

etc. are mapped to dark gray.  Colors 250 through 255 are shades of

white, gray and black.

2: Approximate 128 Mapping.  This is similar to the above, but the range

is squeezed to fit into 128 drawing colors for AutoSketch.  Colors

128 through 255 are set to white.  They should not be used.

3: Set all to Gray.  In this scheme we admit that it is a 16 color device

and don't bother with the higher drawing colors.  Colors 9 through 14

are dark versions of 1 through 6.  Color 15 is white if the background

is light and light gray if it is dark.  All higher colors are mapped

to gray.  This scheme is used by AutoCAD's internal VGA driver (with

higher colors mapped to white).

4: Repeat 0..15.  In this scheme we also admit that it is a 16 color device

but we are too lazy to even turn off the higher colors.  Drawing

colors 0 through 15 are mapped as in scheme 3 above, and the same

16 color pattern is repeated all the way through 255.  For bits and

bytes aficionados, Color = Color and 0fh.  This scheme is not

recommended.

Finally, it is possible to make up your own custom palette through

the "Detailed Logical Colors" menu.  You can even change the standard

drawing colors below 9 if you wish.  Of course, you will then have non-

standard drawings and might have difficulty interfacing with other drawings,

documentation, software and CAD operators.

5.5 DRAWING COLORS IN 256 COLOR MODES

-------------------------------------

When the VGA is used in 256 color mode we are able to assign a

unique shade to each drawing color.  In the standard layout, each color

consists of 3 components: its hue, brightness and saturation.  The hue

changes every 10 drawing colors, like this:

10: red, 20: reddish orange, 30: orange, 40: yellowish orange,

50: yellow, ... 90: green, ... 130: cyan, ... 210: magenta.

The brightness changes every 2 drawing colors, so 10, 12, 14, 16 and 18

are the brightest to darkest reds.  The saturation changes every drawing

color.  Odd colors are half saturated, or pastel.  Thus, 11, 13, 15, 17

and 19 are the brightest to darkest pastel reds.  Colors 250 through

255 are shades of gray.

The AutoCAD sample drawings CHROMA and COLORWH are useful for

understanding the 256 color palettes.  There is also a discussion similar

to this one in the AutoCAD Reference Manual.

AutoSketch only supports 128 drawing colors.  We cannot use the

above palette because the highest color, 127, only gets us into the

aquamarines!  The cyans, blues, purples and magentas would be inaccessible.

Therefore, a 128 color palette is provided for use with AutoSketch.

The 128 color palette uses 18 hues, 3 intensities and 2 saturations,

(We have seen above that the 256 color palette uses 24 hues, 5 intensities

and 2 saturations.)  Thus, 10, 12 and 14 are bright to dark red and

11, 13 and 15 are bright to dark pastel red.  Drawing colors 118 through

127 are shades of gray.

The 128 and 256 color palettes are selected by going into the

"Physical Colors" menu and then selecting "Reset all DAC Values".  The

128 color distribution should only be used with AutoSketch.  Note also

that if you import and export drawings from AutoCAD and AutoSketch, the

higher drawing colors will be perceived differently.  There is not really

a solution to this problem, other than to not use the higher drawing

colors if you are going to view the drawing in both products.

6.1 VGA DISPLAY MEMORY REQUIREMENTS

-----------------------------------

VGA cards store the raster display data in on-board memory.  The

maximum screen resolution is thus limited by how much memory is on the

particular VGA card.  Here are the minimum memory requirements for the

various VGA resolutions:

16 Colors               256 Colors

---------               ----------

640x480  256k           640x400  256k

800x600  256k           640x480  512k

1024x768  512k           800x600  512k

1024x768  1Meg

6.2 VGA MODES AND MONITOR FREQUENCIES

-------------------------------------

You will most likely be using your VGA card with a "multi-scan"

or "multi-sync" monitor.  Unlike "fixed frequency" monitors, the multi-scan

monitors are capable of operating in a range of frequencies.  Following is

a table of typical refresh rates for various VGA modes.  You should check

with your hardware supplier to be sure that your monitor will handle all

of the resolutions of your super VGA card.

Resolution        Vertical        Horizontal

----------        --------        ----------

640x350          70 Hz           31.50 kHz

640x480          60 Hz           31.50 kHz

800x600         55/60 Hz        35.20/37.88 kHz

1024x768          60 Hz           48.00 kHz

1024x768         43.48 Hz         35.52 kHz (interlaced)

7. KNOWN PROBLEMS

7.1 AUTOSHADE 386 1.1 PULL-DOWNS AND DIALOGUE BOXES NOT RESTORING PROPERLY

--------------------------------------------------------------------------

The screen area underneath some pull-down menus and dialogue boxes

are not restored after the menu or dialogue box is exited.  It is more

likely to happen with the larger menus and dialogue boxes.  For example,

try SHADING MODEL under the SETTINGS menu.  Then pick BACKGROUND COLOR,

and pick CANCEL.  Pieces of the last dialogue box will remain on the screen.

This only happens with version 1.1 of AutoShade 386, and with

Svadi configured to either: (a) a display mode other than 640x480 and 1024x768

(in either 16 or 256 colors); or (b) any display mode with the text font

configured for the 12x24 (large) size.

The problem is not correctable.  If you have this version of

AutoShade 386 then you should configure the display resolution to 640x480

or 1024x768 in either 16 or 256 colors, and do not use the 12x24 text font.

7.2 AUTOSHADE 1.1 TEXT COLORS NOT CONFIGURABLE

----------------------------------------------

The logical color settings do not work on versions of AutoShade

through 1.1.  See section above on AutoShade Logical Colors.

7.3 AUTOSHADE RENDERING IN 16 COLOR MODES

-----------------------------------------

Some drawings will not properly render if Svadi is set to a

16 color rendering mode.  You may get an alert message saying "Too

many shades generated, some lost.", after which it will still produce

a rendering.  In the worst case it will hang up while displaying the

screen message "Calculate shading and assign colors.".  You can control-C

out at this point, and then use F1 to flip back to the menu screen.

This is known to happen with the sample CHROMA file.  It is recommended

that you do rendering in 256 color modes.

7.4 RENDERING COLORS NOT MATCHING WIREFRAME COLORS

--------------------------------------------------

Colors 7, 8 and 9 are shades of white, gray and black (depending

on the graphics background setting) in the wireframe, while they are all

a light gray in the rendering.  This can be observed in the sample CHROMA

rendering file.  This is normal and is a result of the difference between

the rendering algorithms and the default Svadi drawing colors.

7.5 CROSSHAIRS HIDDEN BY SOME COLORS IN 256 COLOR MODES

-------------------------------------------------------

In 256 color modes, the crosshairs do not show up on objects of

some colors.  They do show up against the background and against the

basic drawing colors 1 through 15.  The easiest way to see this is to

bring up the drawing "CHROMA", which shows all of the 255 drawing colors,

and move the crosshairs across the different boxes.  The crosshairs will

appear on some boxes and not on others.  On some it will be very dim.

This problem will be fixed in a later release.

7.6 PULL-DOWNS NOT CORRECTLY REPLACED WHEN SIDE MENU WIDTH > 10

---------------------------------------------------------------

This only occurs in AutoCAD Release 11, when Svadi's side menu

width is configured to 11 or more characters.  It occurs with the pull-

down menu fields that have submenus.  These fields have a ">" (greater

than symbol) next to them.  The problem is that the submenu appears but

the bottom of the parent menu extends below it.

For example, select the OPTIONS menu from the top status line.

Then select HATCH.  The HATCH menu will appear correctly, but instead

of ending at the "OPTIONS >" field, there will be several more fields

below it which are inherited from the previous OPTIONS menu.

As long as you do not select one of these additional fields,

this problem should not affect anything.  If you want to avoid it

completely, then you would have to set the side menu width to less

than 11 characters (8 is the normal width).

8 TROUBLESHOOTING

8.1 DRIVER NOT INSTALLED (REAL MODE ADI)

----------------------------------------

If, when entering an AutoCAD drawing, you get a message like:

"ERROR: No display driver is installed at interrupt vector xx" then

either Svadi is not installed or it is configured for a different

interrupt vector from AutoCAD's.  You should double check that Svadi

and AutoCAD are both configured for the same vector (eg 7a) and that

Svadi has been loaded into memory.

AutoShade will give a slightly different error message, such

as "Fatal Error: Rendering driver not installed at interrupt ...".

This indicates the same problem: either Svadi is not installed or it

is not configured for the same interrupt vector as AutoShade.

Note that AutoShade uses two interrupts, one for displaying and

one for rendering, and they must both be matched to Svadi's.  If you get

the message "Error status returned by ADI driver" then AutoShade has found

a driver at the interrupt but it is not the correct type (display or

rendering).  In this case, Svadi is probably installed but is not

configured with the same display and/or rendering interrupt as AutoShade.

One final caution about interrupt vectors: you need to watch out

for conflicts with other software packages that use them.  One common

example is the driver software used by many LANs.  There are some which

are shipped with a default setting to use interrupt 7a, which is also

the default for Svadi and AutoCAD.  In that case, you must change either

the network driver or Svadi and AutoCAD to use a different vector (7b,

for example).  Svadi will display a warning message if there is already

somebody using its interrupt vector when it is loaded.

8.2 PROTECTED MODE DRIVER NOT FOUND

-----------------------------------

If you get a message from AutoCAD, AutoShade, etc, like "Cannot

find ADI P386 display driver...", then you do not have your DSPADI and/or

RDPADI environment strings pointing to the rcpsvadi.exp driver file.  Look

at your environment strings using the DOS "SET" command.  DSPADI is

probably either not defined at all or it is defined with the name of a

nonexistent file.

8.3 FLIPSCREEN (F1 FUNCTION KEY) NOT RESTORING GRAPHICS SCREEN

--------------------------------------------------------------

See the section above on Flipscreen Save/Restore for a description

of this subject.  If the save/restore is not working, then you may see

streaks or blotches at the top of the graphics screen.  The part that is

in the drawing will be cleared up with a REDRAW, but any garbage in the

status bar and menu areas will remain there.  A SHELL command will cause

the entire screen to be redrawn and the garbage to disappear.

This behavior indicates that Svadi does not know how to save

and restore the graphics screen for this particular card.  If this happens,

then you should go into Svconfig's GENERAL menu, then select FLIPSCREEN

SAVE/RESTORE, and then DISABLE.  This will cause Svadi to redraw the entire

screen whenever you change from text back to graphics.

8.4 LOSS OF GRAPHICS MODE OR FAILURE TO ENTER IT

------------------------------------------------

Because there is such a variety of species of PC's and of VGA

cards, there may be cases where a particular card does not work properly

in your PC at a certain display resolution.  The types of problems you

may get are:

o AutoCAD (AutoShade, etc) may return an error message when going

into graphics mode (the drawing editor) saying it cannot initialize

the display adapter.

o AutoCAD (etc) may proceed into graphics mode, thinking the VGA

is functioning properly, while you see some form of scrambled

graphics, blank screen or other unexpected display.  Or, the

trouble may occur after AutoCAD has been running for a while,

although this is less likely.

There are many ways for these problems to occur, and diagnosing

them can be difficult and frustrating.  You should seek help from your

dealer or the VGA vendor's technical support.  You should not open

your machine or fiddle with card slots, dip switches and jumpers

unless you are familiar with such practices.  Following is a list of

possible things that you can check out, but it is not meant to be

comprehensive or to take the place of competent technical support:

o Check the monitor and VGA card frequencies.  Make sure that the

mode displays at a scanning frequency that the monitor can handle.

o Try reconfiguring the application to use the built-in VGA driver.

(This will only run at the standard 640x480x16 resolution, so

this will not help if you are only experiencing problems at

higher resolutions.)

o Try both the real and protected mode versions of Svadi (if the

application supports both).  If one works and the other doesn't,

then it is most likely a software problem in Svadi.

o Check the display memory on the card.  Make sure that there is

enough display RAM on board (see table above).

o Make sure jumpers and switch settings are correct.  Most cards

have a switch for 8-bit and 16-bit BIOS.  Try it both ways.

o Try the card in both 8-bit and 16-bit slots.  Some cards will only

work in one or the other in certain modes on certain machines.

o Run any diagnostic programs that came with the card.  Try to

bring up the graphics mode that you are having trouble with.

This can help determine whether it is a hardware or software

problem.

o Try a different VGA card.  If it works then you may have a

bad VGA card or one that is incompatible with your PC.

o Try the VGA card in a different PC.  If it fails then you may

have a bad card.

o Some PC's have more than one speed setting, perhaps "turbo" and

"normal" modes.  See if the card works at any of the speeds.  If

so then it is probably a compatibility problem between card and PC.

o If you have other application software that uses VGA graphics,

try to run it.  This will also help distinguish between software

and hardware problems.  You may get different results even if

the problem is in the hardware, however, since different software

may access the VGA in different ways.

o Try it with or without a monochrome card in the system.  Some VGA

cards are affected by this.

8.5 VGA SCREEN GOES DIM OR BLANK DURING DUAL SCREEN MODE

--------------------------------------------------------

Before you even enter AutoCAD, when you change the current

screen from the VGA to the monochrome (by typing "mode mono"), the

VGA screen goes dim or blank.  This is caused by an incompatibility

between the VGA card and the PC.  It may be fixable or not.  See if

the card has any jumpers relating to companion monochrome cards.  See

if the PC's user manual or setup program discuss dual video cards.

Try changing the VGA card from an 8-bit to 16-bit slot, or vice versa.

Try changing the jumper on the card from 8-bit BIOS to 16-bit BIOS.

If nothing works, then you will have to use single screen mode.

8.6 AUTOSHADE MENUS MISSING AFTER FLIPSCREEN

--------------------------------------------

After you do a rendering and then flip back to the display

screen, you should see the menu bar at the top of the screen.  If

it is not there, then probably AutoShade was configured not to

redraw after flipscreens.  You should reconfigure AutoShade (see

section above on configuring AutoShade/RenderMan for use with

Svadi), and be sure to answer YES when it asks you if redraw is

required after flipscreens.

8.7 RENDERING NOT RESTORED ON FLIPSCREEN (REAL MODE ONLY

---------------------------------------------------------

If flipping back to the rendering screen with the F1 key gives

you a blank screen instead of the last rendering that was there, there

may not have been enough disk space with which to save the rendered

image.  Check which disk is configured as the save disk, and then check

how much free space it has.  Svconfig will show how much free space is

needed for the specified resolution (the value is mentioned in the help

message at the bottom of the screen when you pick the DISK DRIVE FOR

RENDER SAVE FILE item).

Note: this only applies to the real mode driver.  The protected

mode driver does not use a disk drive to save the rendered image.