Diamond Computer Systems STEALTH 32 Power Management vers. 1.0c

05/11/94

Diamond Display Power Management supports three power management states

defined by VESA  (Video Electronics Standards Association, version 1.0p,

revision 0.7p, August 20, 1993) in DOS environment:

STANDBY

SUSPEND

OFF

Power management states in DOS environment can be individually set from 1

to 99 minutes through options of STL32PM (Stealth 32 power management TSR

intallation program) as following:

Usage:  STL32PM [option]

Options:

h - Displays this help message

u - Unloads utility from memory

bxx - sets Standby timer in minutes (1 to 99) in DOS environment

sxx - sets Suspend timer in minutes (1 to 99) in DOS environment

oxx - sets Off timer in minutes (1 to 99) in DOS environment

For example:

STL32PM b7 s15 o27 <enter>

Stealth 32 will be in STANDBY state after 7 minutes in DOS environment.

It will be in SUSPEND state 8 minutes after STANDBY state in DOS

environment.  Then in OFF state 12 minutes after SUSPEND state in DOS

environment.

After Stealth 32 power management TSR is loaded, you can use STL32PM to show

current power management timer settings in DOS environment.

For example: C:\STL32PM <enter>

Display adapter is set to ON state with input from keyboard or mouse (if mouse

device driver is loaded) in DOS environment.

DISKETTE CONTAINS:

STL32PM.COM     Stealth 32 Power Management BIOS TSR 1.0c

INSTALLATION:

1.  Copy SSPROPM.COM to root directory C:\.

2.  Modify AUTOEXEC.BAT.  Add C:\STL32PM [option] to set Standby,

Suspend and Off timers in DOS environment.

Stealth 32 Power Management BIOS TSR will be loaded with every

system boot, you can change those timer settings later by unload

it and load again with new settings.

NOTES:

STANDBY minutes must be <= SUSPEND minutes

SUSPEND minutes must be <= OFF miniutes

LIMITATIONS:

In protected mode application (i.e. AutoCad, Links386 game), DPMS is

not properly supported at this release.

PROGRAMMER'S REFERENCES:

This STEALTH 32 DMPS BIOS TSR supports following VBE/PM services

(VESA Display BIOS Extensions / Power Management, VBE Sub-function 10h)

as defined in VESA VBE/PM version 1.0p, revision 0.4p (Sept. 1, 1993):

a. Report VBE/PM Capabilities:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 00h       Report VBE/PM Capabilities

ES:DI          Null pointer 0000:0000h (reserved for future

use)

Output:  AX =           Status (*)

BH =           Power saving state signals supported

1 = supported, 0 = not supported

bit 0         STANDBY

bit 1         SUSPEND

bit 2         OFF

bit 3         REDUCED ON (intended for flat panel)

bits 7-4      reserved for future power control

BL =           VBE/PM Version number (00010000b - 1.00)

bits 3-0      Minor Version number

bits 7-4      Major Version number

ES:DI          Unchanged

b. Set Display Power State:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 01h       Set Display Power State

BH =           Requested Power State

00h       ON

01h       STANDBY

02h       SUSPEND

04h       OFF

08h       REDUCED ON (intended for flat panel)

Output:  AX =           Status (*)

BH =           Unchanged

c. Get Display Power State:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 02h       Get Display Power State

Output:  AX =           Status (*)

BH =           Power state currently requested by display

00h       ON

01h       STANDBY

02h       SUSPEND

04h       OFF

08h       REDUCED ON (intended for flat panel)

* Status information:

AL == 4Fh:     Function is supported

AL != 4Fh:     Function is not supported

AH == 00h:     Function call successful

AH != 00h:     Function call failed