# BEFORE STARTING ENGINES

- Gear -- CHECK DOWN

- BATT Switch -- ON

- L and R GEN Switches -- OFF

- Flight Controls -- CHECK

- Engine Instruments -- CHECK

- Fuel Quantity -- CHECK

- De-ice -- OFF

- Aircraft Lighting -- OFF

- Parking Brake -- SET

- Trim -- AS DESIRED

- Altimeters -- SET

- Standby Instruments -- SET

- Takeoff Data (V1, VR, V2) -- COMPUTED and SPEEDS SET

- Avionics Master Switch -- ON

- Avionics -- SET FOR DEPARTURE

# STARTING ENGINES

- Beacon/Strobe Switch -- BCN/STROBE

- PARKING BRAKE -- SET

- Thrust Levers -- IDLE

- Fuel Flow -- ON

- L Start Switch -- ON

- L FF (Fuel Flow) -- CHECK

- L N1 increasing as L N2 increases -- CHECK

- L ITT -- CHECK

- L Oil Pressure -- CHECK

- L Engine Instruments -- CHECK NORMAL

- R Start Switch -- ON

- R FF (Fuel Flow) -- CHECK

- R N1 increasing as L N2 increases -- CHECK

- R ITT -- CHECK

- R Oil Pressure -- CHECK

- R Engine Instruments -- CHECK NORMAL

- L and R GEN Switches -- ON

# BEFORE TAXI

- De-ice -- ON

- Spoiler Lever -- ARM

- Flaps -- SET 8 OR 20 DEGREES

- Flight Controls -- CHECK

- Aircraft Lighting -- ON AS REQUIRED

- Brakes -- RELEASE

# TAXI AND BEFORE TAKEOFF

- Brakes -- CHECK

- Flight Instruments -- CHECK

- Engine Instruments -- CHECK

- Takeoff Data (V1, VR, V2) -- CHECK

- Nav Equipment -- CHECK

- Transponder -- ON

- Flaps -- SET 8 OR 20 DEGREES

- Trim -- SET FOR TAKEOFF

- De-ice -- AS REQUIRED

# RUNWAY LINEUP

- Lights -- ON

# TAKEOFF

- Thrust Lever -- T/O

- Brakes -- RELEASE

- VR -- ROTATE

# AFTER TAKEOFF

- Gear -- UP AFTER POSITIVE RATE OF CLIMB

- Flaps -- UP AT V2+25

- Yaw Damper -- ON AS REQUIRED

- SPOILER LEVER -- RET

- De-ice -- AS REQUIRED

- L and R LDG-TAXI Light Switches -- OFF

# CLIMB

CAUTION: Maximum thrust lever setting is MCR with full anti-ice on between 15,500 and 30,000 feet.

- Altimeters -- RESET TO 29.92 CROSSING 18,000 FEET

# DESCENT

- De-ice -- AS REQUIRED

CAUTION: Maximum thrust lever setting is MCR with full anti-ice on between 15,500 and 30,000 feet.

- Altimeters -- RESET TO LOCAL CROSSING 18,000 FEET

- Fuel Quantities and Balance -- CHECK

# APPROACH

- Landing Data (VREF, VAPP) -- COMPUTED and SPEEDS SET

- Avionics and Radios -- SET

- Flaps -- SET 8 OR 20 DEGREES

- Spoiler Lever -- ARM

# BEFORE LANDING

- Gear -- DN (Check for 3 green lights)

- L and R LDG-TAXI Light Switches -- ON

- Flaps -- DN, check indication

# LANDING

- Brakes -- AS REQUIRED

- Spoilers -- VERIFY EXTENDED

# AFTER LANDING/CLEARING RUNWAY

- Transponder -- OFF

- De-ice -- AS REQUIRED

- Lights -- AS DESIRED

- Spoiler Lever -- RET

- Flaps -- UP

- Thrust Lever -- CUTOFF

# SHUTDOWN

- Parking Brake -- SET

- De-Ice -- OFF

- Thrust Levers -- IDLE

- Fuel Flow -- OFF

- Lights -- OFF

- L and R GEN Switches -- OFF

- BATT Switch -- OFF

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

19,907 lbs

VMO - Maximum Operating Speed (airspeed)

330 KIAS (primary display)

325 KIAS (standby airspeed indicator)

MMO - Maximum Operating Speed (Mach)

.78 Mach (autopilot disengaged)

.81 Mach (autopilot engaged)

VA  -  Maneuvering Speed

12,500 lbs.

150 KIAS (sea level)

163 KIAS (20,000')

195 KIAS (40,000')

20,200 lbs.

198 KIAS  (sea level)

225 KIAS (20,000')

245 KIAS (40,000')

VLO - Maximum Gear Operating Speed

200 KIAS

VLE - Maximum Landing Gear Extension Speed

260 KIAS

VFE - Maximum Flaps Extended Speed

250 KIAS (flaps 8 degrees)

200 KIAS (flaps 20 degrees)

150 KIAS (flaps 40 degrees)

V1 - Takeoff Decision Speed (dry runway; standard temperature)

107 KIAS (for all conditions, with anti-skid off)

VR - Rotation Speed (dry runway; standard temperature)

Sea Level

108 KIAS (13,000 lbs)

118 KIAS ( 20,200 lbs)

5,000'

108 KIAS (13,000 lbs)

119 KIAS ( 20,200 lbs)

V2 - Takeoff Safety Speed (dry runway, standard temperature)

Sea Level

120 KIAS (13,000 lbs)

129 KIAS (20,200 lbs)

5,000'

120 KIAS (13,000 lbs)

130 KIAS ( 20,200 lbs)

VAPP - Approach Climb Speed (flaps 20 degrees; gear up)

<4,000'

106 KIAS (13,000 lbs)

127 KIAS (19,200 lbs)

5,000'

107 KIAS (13,000 lbs)

128 KIAS ( 19,200 lbs)

VREF - Landing Approach Speed (flaps 40 degrees; gear down)

<4,000'

102 KIAS (13,000 lbs)

123 KIAS (19,200 lbs)

5,000'

103 KIAS (13,000 lbs)

123 KIAS ( 19,200 lbs)

Best Glide Speed

160 KIAS (gear and flaps up; both engines inoperative)

Specifications

Cruise Speed				Mach 0.81, 464 kts, 859 kmh

Engines					Allied Signal TFE731-20 3,500 lb thrust

Maximum Range				2,200 nm, 2,532 sm, 4,074 km

Service Ceiling				51,000 ft 15,545 m

Fuel Capacity				6,062 lb, 904.8 U.S. gal, 2,722 kg, 3,341 L

Maximum Gross Weight			20,450 lb, 9,276 kg

Maximum Takeoff Weight – HGW		20,200 lb, 9,163 kg

Length					58.4 ft, 17.7 m

Wingspan					47.8 ft, 14.6 m

Height					14.3 ft, 4.3 m

Seating					Up to 9

Useful Load				2,650 lb, 1,202 kg