# BEFORE STARTING ENGINE

- Magneto/Starter Switch -- OFF

- Master Switch -- OFF

- Fuel Boost Pump Switch -- OFF

- Throttle -- CLOSED

- Propeller -- HIGH RPM

- Mixture -- IDLE CUT-OFF

- Parking Brakes -- SET

- Master Switch -- ON

- Cowl Flaps -- OPEN

- Flaps -- UP

- Master Switch -- OFF

- Landing Gear Switch -- DOWN

# ENGINE START

- Throttle -- OPEN 1/4 INCH

- Propeller -- HIGH RPM

- Mixture -- RICH

- Master Switch -- ON

- Fuel Boost Pump Switch -- ON for 0-40 sec.

- Fuel Boost Pump Switch -- OFF

- Propeller Area -- CLEAR

- Magneto/Starter Switch -- START (release when engine starts)

- Throttle -- IDLE (700-750 RPM)

Then:

- Oil Pressure -- CHECK

- Ammeter -- CHECK

- Engine Instruments -- CHECK

# BEFORE TAXI

- Avionics Master Switch -- ON

- Heading Indicator -- SET

- Instruments -- NORMAL OPERATION

- Radios and Avionics -- CHECKED and SET

- Altimeter -- SET

- Fuel Selector -- SWITCH TANKS (verify engine operation on both)

- Cowl Flaps -- FULL OPEN

# TAXI

- Rudder Trim -- AS DESIRED

- Parking Brake -- RELEASE

- Brakes -- CHECK during taxi

- Directional Gyro -- PROPER INDICATION during turns

- Turn Coordinator -- PROPER INDICATION during turns

- Artificial Horizon -- ERECT during turns

- Throttle -- MINIMUM POWER

- Cowl Flaps -- OPEN

- Propeller -- HIGH RPM

# BEFORE TAKEOFF

- Parking Brake -- SET

- Fuel Selector Valve -- FULLEST TANK

- Throttle -- 1000 RPM

- Cowl Flaps -- OPEN

- Propeller -- HIGH RPM

- Mixture -- RICH

- Alternator Switch -- VERIFY ON

- Throttle -- 2000 RPM

- Magnetos – CHECK <150 RPM drop on each, <50 RPM drop between

- Propeller -- CYCLE high RPM/low RPM/high RPM

- Ammeter -- CHECK Positive charge indication

- Throttle -- 1000 RPM

- Fuel Boost Pump Switch -- ON (verify annunciator lit)

- Elevator Trim -- SET for takeoff

- Rudder Trim -- SET for takeoff

- Flaps -- CHECK and SET for takeoff (10 degrees)

- Flight Controls -- FREE AND CORRECT

- Radios and Avionics -- SET

- Annunciator Lights -- CHECK

- Strobe Lights/Beacon -- ON

- Oil Temperature -- CHECK

- Cylinder Head Temperature -- CHECK

- Parking Brake -- RELEASE

# TAKEOFF

- Throttle -- FULL

- Annunciator Lights -- CHECK

- Lights -- AS DESIRED

- Engine Instruments -- CHECK

- Liftoff Speed -- 60 KIAS

- Climb Speed -- 85 KIAS

- Landing Gear -- RETRACT IN CLIMB after clearing obstacles

- Flaps -- UP

- Fuel Boost Pump Switch -- OFF

# BEST RATE CLIMB

- Throttle -- FULL

- Propeller -- 2575 RPM

- Mixture -- RICH

- Cowl Flaps -- FULL OPEN

- Rudder Trim -- AS DESIRED

- Airspeed -- 105 KIAS

# BEST ANGLE CLIMB

- Throttle -- FULL

- Propeller -- 2575 RPM

- Mixture -- RICH

- Cowl Flaps -- FULL OPEN

- Rudder Trim -- AS DESIRED

- Airspeed -- 85 KIAS

# CRUISE CLIMB

- Throttle -- 34" MP

- Propeller -- 2400 RPM

- Mixture -- RICH

- Cowl Flaps -- FULL OPEN or as required

- Rudder Trim -- AS DESIRED

- Airspeed -- 120 KIAS

# CRUISE

- Airspeed -- ACCELERATE to cruise airspeed

- Throttle -- AS DESIRED

- Propeller -- AS DESIRED

- Mixture -- LEAN TO PEAK TIT (<1750)

- Engine Instruments -- CHECK

- Cowl Flaps -- AS REQUIRED (to maintain normal Cylinder Head and Oil temperatures)

- Engine Temperatures -- STABILIZE at cruise condition

- Rudder Trim -- AS DESIRED

# FUEL TANK SELECTION

- Fuel Boost Pump Switch -- ON

- Fuel Selector -- OPPOSITE TANK

- Fuel Boost Pump Switch -- OFF

# DESCENT

- Flaps -- UP

- Landing Gear -- CHECK UP

- Throttle -- ABOVE 15" MP (keep CHT in green)

- Propeller -- 2400 RPM

- Mixture -- PEAK TIT

- Cowl Flaps -- CLOSED

- Cylinder Head Temperature (CHT) -- MONITOR

- Airspeed -- AS DESIRED (195 KIAS max.)

- Rudder Trim -- AS DESIRED

# APPROACH FOR LANDING

- Lights -- AS DESIRED

- Landing Gear -- EXTEND (below 140 KIAS) and CHECK DN

- Mixture -- RICH (on final)

- Propeller -- HIGH RPM (on final)

- Fuel Boost Pump -- ON

- Fuel Selector -- FULLEST TANK

- Flaps -- FULL DOWN (below 110 KIAS)

- Elevator Trim -- AS DESIRED

- Rudder Trim -- CENTERED OR AS DESIRED

- Parking Brake -- VERIFY OFF

# LANDING

- Airspeed -- 75 KIAS

- Touchdown -- MAIN WHEELS FIRST

- Landing Roll -- LOWER NOSE WHEEL GENTLY

- Brakes -- MINIMUM REQUIRED

# TAXI AFTER LANDING

- Throttle -- AS REQUIRED

- Fuel Boost Pump -- OFF

- Cowl Flaps -- OPEN

- Flaps -- RETRACT

- Elevator Trim -- TAKEOFF SETTING

- Avionics/Radios -- AS REQUIRED

- Lights -- AS DESIRED

# SHUTDOWN

- Parking Brake -- SET

- Throttle -- IDLE

- Avionics Switch, Electrical Equipment -- OFF

- Lights -- OFF

- Pitot Heat -- OFF

- Mixture -- IDLE CUT-OFF (pulled fully out)

- Master Switch -- OFF

- Magneto/Starter Switch -- OFF

# SECURING AIRCRAFT

- Magneto/Starter Switch -- Verify OFF

- Master Switch -- Verify OFF

- Avionics Switch -- Verify OFF

- Electrical Switches -- Verify OFF

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

3,326 lbs.

VA  -  Maneuvering Speed

127 KIAS (3368 lbs.)

123 KIAS (3200 lbs.)

117 KIAS (2900 lbs.)

111 KIAS (2600 lbs.)

VFE - Maximum Flaps Extended Speed

110 KIAS (flaps full down)

VLE - Maximum Landing Gear Extension Speed

165 KIAS

VLO - Maximum Gear Operating Speed

140 KIAS (extension)

106 KIAS (retraction)

VNO - Maximum Structural Cruising Speed

174 KIAS

VNE - Never Exceed Speed

195 KIAS

VS  -  Stalling Speed (maximum weight, forward c.g.)

66 KIAS

VSO - Stalling Speed in the Landing Configuration

59 KIAS

VX  -  Best Angle-of-Climb Speed

85 KIAS

VY  -  Best Rate-of-Climb Speed

105 KIAS

Best Glide Speed

93.5 KIAS (3368 lbs.)

89 KIAS (3200 lbs.)

84.5 KIAS (2900 lbs.)

80 KIAS (2600 lbs.)

## Specifications

Maximum Speed				220 kts, 253 mph, 407 kmh

Cruise Speed				195 kts, 224 mph, 361 kmh

Engine					Textron Lycoming TIO-540-AF1B 270 hp

Propeller					McCauley three-bladed constant speed

Maximum Range				1,050 nm, 1,204 sm, 1,945 km

Service Ceiling				25,000 ft, 7,620 m

Fuel Capacity				89 U.S. gal, 337 L

Empty Weight				2,268 lb, 1,029 kg

Maximum Gross Weight			3,368 lb, 1,528 kg

Length					26.75 ft, 8.15 m

Wingspan					36 ft, 11 m

Height					8.33 ft, 2.5 m

Seating					Up to 4

Useful Load				1,100 lb, 500 kg