# BEFORE STARTING ENGINE

- Brakes -- TEST AND SET

- Electrical Equipment, Autopilot -- OFF

- Avionics Master Switch -- OFF

- Fuel Selector Valve -- BOTH

# ENGINE START

- Throttle -- OPEN 1/4 INCH

- Mixture -- RICH

- Propeller Area -- CLEAR

- Master Switch -- ON

- Auxiliary Fuel Pump Switch -- ON

- Ignition Switch -- START (release when engine starts)

Then:

- Oil Pressure -- CHECK

- Auxiliary Fuel Pump -- OFF

- Flashing Beacon and Nav Lights -- ON as required

- Avionics Master Switch -- ON

- Flaps -- RETRACT

# BEFORE TAKEOFF

- Parking Brake -- SET

- Flight Controls -- FREE AND CORRECT

- Flight Instruments -- CHECK and SET

- Fuel Quantity -- CHECK

- Mixture -- RICH

- Fuel Selector Valve -- RECHECK BOTH

- Elevator Trim -- SET for takeoff

- Throttle -- 1800 RPM

- Magnetos – CHECK <150 RPM drop on each; <50 RPM drop between

- Suction Gauge -- CHECK

- Engine Instruments and Ammeter -- CHECK

- Annunciator Panel -- CHECK

- Throttle -- 1000 RPM or less

- Strobe Lights -- AS DESIRED

- Radios and Avionics -- SET

- Autopilot -- OFF

- Wing Flaps -- SET for takeoff (0-10 degrees)

- Brakes -- RELEASE

# TAKEOFF

- Wing Flaps -- 0-10 degrees

- Throttle  -- FULL OPEN

- Mixture -- RICH (above 3000 feet, LEAN to obtain maximum RPM)

- Elevator Control -- LIFT NOSE WHEEL (at 55 KIAS)

- Climb Speed --  70-80 KIAS

# EN ROUTE CLIMB

- Airspeed -- 70-85 KIAS

- Throttle -- FULL OPEN

- Mixture -- RICH (above 3000 feet, LEAN to obtain maximum RPM)

# CRUISE

- Power -- 2100-2700 RPM

- Elevator Trim -- ADJUST

- Mixture -- LEAN

# DESCENT

- Power -- AS DESIRED

- Mixture -- ADJUST  for smooth operation (Full Rich for idle power)

- Fuel Selector Valve -- BOTH

# BEFORE LANDING

- Fuel Selector Valve -- BOTH

- Mixture -- RICH

- Landing/Taxi Lights -- ON

- Autopilot -- OFF

# LANDING

- Airspeed -- 65-75 KIAS (flaps UP)

- Wing Flaps -- AS DESIRED

- 0-10 degrees <110 KIAS

- 10-30 degrees <85 KIAS

- Airspeed --  60-70 KIAS (flaps DOWN)

- Touchdown -- MAIN WHEELS FIRST

- Landing Roll -- LOWER NOSE WHEEL GENTLY

- Braking -- MINIMUM REQUIRED

# AFTER LANDING

- Wing Flaps -- UP

# SECURING AIRPLANE

- Parking Brake -- SET

- Avionics Master Switch, Electrical Equipment, Autopilot -- OFF

- Mixture -- IDLE CUT OFF (pulled fully out)

- Ignition Switch -- OFF

- Master Switch -- OFF

- Fuel Selector Valve -- LEFT or RIGHT to prevent crossfeeding

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

2,550 lbs

VA - Manoeuvring Speed

105 KIAS (2550 lbs)

98 KIAS (2200 lbs)

90 KIAS (1900 lbs)

VFE - Maximum Flaps-Extended Speed

110 KIAS (10 degrees flaps)

85 KIAS (10-30 degrees flaps)

VNO - Maximum Structural-Cruising Speed

129 KIAS

VNE - Never Exceed Speed

163 KIAS

VS  -  Stalling Speed (maximum weight, flaps up)

48 KIAS

VSO - Stalling Speed in the Landing Configuration

(maximum weight, flaps 30 degrees)

40 KIAS

VX  -  Best Angle-of-Climb Speed

62 KIAS (sea level)

67 KIAS (10,000 feet msl)

VY  -  Best Rate-of-Climb Speed

74 KIAS (sea level)

72 KIAS (10,000 feet msl)

Maximum Glide Speed

68 KIAS

Specifications

Maximum Speed				126 kts, 203 km/hr

Cruise Speed				124 kts, 200 km/hr

Engine					Textron Lycoming IO-360-L2A 180 bhp

Propeller					Macauley Fixed Pitch Two Blade

Maximum Range				638 nm

Service Ceiling				14,000 ft, 4,267 m

Fuel Capacity				56 gal, 212 L

Empty Weight				1,665 lb, 1,002 kg

Maximum Gross Weight			2,550 lb, 1,157 kg

Length					27 ft, 2 in, 8.2 m

Wingspan					36 ft, 1 in, 11 m

Height					8 ft, 11 in, 2.72 m

Seating					Up to 4

Useful Load				893 lb, 423 kg