# BEFORE STARTING ENGINE

- Brakes -- TEST and SET

- Electrical Equipment -- OFF

- Avionics Power Switch -- OFF

- Landing Gear Lever -- DOWN

- Cowl Flaps -- OPEN

- Fuel Selector Valve -- BOTH

# ENGINE START

- Prime -- AS REQUIRED

- Throttle -- PUMP once, then OPEN 1/4 INCH

- Propeller -- HIGH RPM

- Mixture -- RICH

- Propeller Area -- CLEAR

- Master Switch -- ON

- Ignition Switch -- START (release when engine starts)

Then:

- Oil Pressure -- CHECK

- Avionics Power Switch -- ON

- Flashing Beacon and Nav Lights -- ON as required

- Flaps -- RETRACT

# BEFORE TAKEOFF

- Parking Brake -- SET

- Flight Instruments -- CHECK and SET

- Primer -- LOCKED

- Mixture -- RICH

- Fuel Quantity -- CHECK

- Fuel Selector Valve -- RECHECK BOTH

- Elevator Trim -- SET for takeoff

- Rudder Trim -- SET for takeoff

- Throttle -- 1700 RPM

- Magnetos – CHECK <175 RPM drop on each; <50 RPM drop between

- Carburetor Heat -- CHECK (for RPM drop)

- Propeller -- CYCLE high RPM/low RPM/high RPM

- Vacuum Gauge -- CHECK

- Engine Instruments and Ammeter -- CHECK

- Throttle -- 800-1000 RPM

- Strobe Lights -- AS DESIRED

- Radios and Avionics -- SET

- Flaps -- SET for takeoff (0-20 degrees)

- Cowl Flaps -- OPEN

- Brakes -- RELEASE

# TAKEOFF

- Flaps -- 0-20 degrees

- Carburetor Heat -- COLD

- Mixture -- RICH (may be leaned above 3000')

- Propeller -- 2400 RPM

- Throttle -- FULL

- Elevator Control -- LIFT NOSE WHEEL (at 50 KIAS)

- Climb Speed --  70 KIAS (flaps 20 degrees)

- 80 KIAS (flaps UP)

- Brakes -- APPLY momentarily when airborne

- Landing Gear -- RETRACT in climbout

- Flaps -- RETRACT

# NORMAL CLIMB

- Airspeed -- 90-100 KIAS

- Throttle --  23" MP or FULL (whichever is less)

- Propeller -- 2400 RPM

- Fuel Selector Valve -- BOTH

- Mixture -- FULL RICH (may be leaned above 3,000')

Cowl Flaps -- OPEN as required

MAXIMUM PERFORMANCE CLIMB

- Airspeed -- 88 KIAS at sea level to 75 KIAS at 10,000'

- Throttle -- FULL

- Propeller -- 2400 RPM

- Fuel Selector Valve -- BOTH

- Mixture -- FULL RICH (may be leaned above 3000')

- Cowl Flaps -- FULL OPEN

# CRUISE

- Throttle -- 15-23" MP

- Propeller -- 2100-2400 RPM

- Elevator Trim -- ADJUST

- Rudder Trim -- ADJUST

- Mixture -- LEAN

- Cowl Flaps -- CLOSED

# DESCENT

- Fuel Selector Valve -- BOTH

- Throttle -- AS DESIRED

- Propeller -- AS DESIRED

- Mixture -- ENRICH as required

- Carburetor Heat -- AS REQUIRED to prevent carb. icing

- Cowl Flaps -- CLOSED

- Flaps -- AS DESIRED

- 0-10 degrees <140 KIAS

- 10-FULL <95 KIAS

# BEFORE LANDING

- Fuel Selector Valve -- BOTH

- Landing Gear -- DOWN

- Mixture -- RICH

- Propeller -- HIGH RPM

- Carburetor Heat -- ON

- Autopilot -- OFF

# LANDING

- Airspeed -- 70-80 KIAS (flaps UP)

- Flaps -- AS DESIRED

- 0-10 degrees <140 KIAS

- 10-20 degrees <120 KIAS

- FULL <95 KIAS

- Airspeed --  65-75 KIAS (flaps DOWN)

- Trim -- ADJUST

- Touchdown -- MAIN WHEELS FIRST

- Landing Roll -- LOWER NOSE WHEEL GENTLY

- Braking -- MINIMUM REQUIRED

# AFTER LANDING

- Flaps -- UP

- Carburetor Heat – COLD

- Cowl Flaps -- OPEN

# SECURING AIRPLANE

- Parking Brake -- SET

- Throttle -- IDLE

- Avionics Switch , Electrical Equipment -- OFF

- Mixture -- IDLE CUT-OFF (pulled fully out)

- Ignition Switch -- OFF

- Master Switch -- OFF

- Cowl Flaps -- CLOSE

- Fuel Selector Valve -- LEFT or RIGHT to prevent cross feeding

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

3,100 lbs

VA - Maneuvering Speed

112 KIAS (3100 lbs)

101 KIAS (2550 lbs)

89 KIAS (2000 lbs)

VFE - Maximum Flaps Extended Speed

140 KIAS (0-10 degrees flaps)

120 KIAS (10-20 degrees flaps)

95 KIAS (20 degrees-Full flaps)

VLE - Maximum Landing Gear Extension Speed

140 KIAS

VLO - Maximum Gear Operating Speed

140 KIAS

VNO - Maximum Structural Cruising Speed

159 KIAS

VNE - Never Exceed Speed

181 KIAS

VS  -  Stalling Speed (maximum weight; forward c.g.)

41 KIAS

VSO  - Stalling Speed in the Landing Configuration

(maximum weight; forward c.g.)

39 KIAS

VX  -  Best Angle-of-Climb Speed

65 KIAS (sea level)

67 KIAS (10,000')

VY  -  Best Rate-of-Climb Speed

88 KIAS (sea level)

75 KIAS (10,000')

Best Glide Speed

80 KIAS (3100 lbs)

72 KIAS (2550 lbs)

64 KIAS (2000 lbs)

Specifications

Maximum Speed				145 kts, 167 mph, 269 kmh

Cruise Speed				140 kts, 161 mph, 259 kmh

Engine					Textron Lycoming IO-540-AB1A5 230 hp

Propeller					McCauley 3-bladed constant speed

Maximum Range				968 nm, 944 sm, 1,519 km

Service Ceiling				18,100 ft, 5,517 m

Fuel Capacity				92 U.S. gal, 333 L

Empty Weight				1,910 lb, 854 kg

Maximum Gross Weight			3,110 lb, 1,411 kg

Length					29 ft, 8.84 m

Wingspan					36 ft, 11 m

Height					9 ft, 2.77 m

Seating					Up to 4

Useful Load				1,200 lbs, 557 kg