# BEFORE START

- Parking Brake -- SET

# ENGINE START

- Engine Start Levers – ON

- Throttle – IDLE

- Left Engine Start Switch – START (hold until engine starts)

- Right Engine Start Switch  - START (hold until engine starts)

# AFTER START

- Engine Start Switches -- CHECK GEN

- Pitot Heat -- ON

- De-ice -- AS REQUIRED

- Engine Start Levers -- CHECK IDLE

- Lights -- AS REQUIRED

- Autopilot -- SET and OFF

- Instruments -- CHECKED

- Autobrake Switch -- RTO (Rejected Takeoff)

- Avionics Switch -- ON

- Avionics -- SET

# DURING TAXI/BEFORE TAKEOFF

- Brakes -- RELEASE

- Thrust Levers-- INCREASE SLIGHTLY TO ROLL

- Thrust Levers -- IDLE FOR TAXI

- Flight Controls -- CHECK

- Flaps -- SET FOR TAKEOFF (5)

- Brakes -- SET

# CLEARED FOR TAKEOFF

- Engine Start Switches -- GEN

- Flight Director -- ON

- Autothrottle – ARM (if using TO/GA mode for takeoff)

- Transponder -- ON

- Lights -- AS REQUIRED

# TAKEOFF

- Brakes -- RELEASE

- Thrust Levers -- ADVANCE SMOOTHLY TO 40% N1 (wait for engine stabilization)

- Thrust Levers -- ADVANCE SMOOTHLY TO 100% N1 --or-- TO/GA Mode -- Engage

- Engine Instruments -- MONITOR

- Airspeed 80 KIAS -- CALLOUT "80 KNOTS" -- VERIFY PROPER THRUST SET

- Airspeed V1 -- CALLOUT "V1"

- Airspeed VR -- CALLOUT "ROTATE" -- ROTATE TO APPROX. 10 DEGREES PITCH UP

- Airspeed V2 -- CALLOUT "V2"

- Landing Gear -- UP (WHEN POSITIVE CLIMB ESTABLISHED)

- Airspeed --MAINTAIN V2+20 KIAS

- Autopilot -- ENGAGE AT 1,000' AGL

- Flaps-- RETRACT TO 1 AT 1,000' AGL

- Flaps-- UP AT 210 KIAS

- Autothrottle -- OFF

- Thrust Levers-- 90% N1

- Airspeed -- 250 KIAS AT 3,000' AGL

- Landing Gear Lever -- Verify OFF

# CLIMB

- Airspeed -- MAINTAIN 250 KIAS

- Landing Lights -- OFF ABOVE 10,000' MSL

- Airspeed -- 280 KIAS ABOVE 10,000' MSL

- Altimeter -- SET TO 29.92 CROSSING 18,000' MSL

# CRUISE

- Thrust Levers-- AS DESIRED

- Trim -- AS NECESSARY

# DESCENT

- Autobrake Switch -- AS DESIRED

- Airspeeds (VREF, VAPP) -- COMPUTED and SET

- De-ice -- AS REQUIRED

- Autopilot -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Altimeter -- SET TO LOCAL SETTING CROSSING 18,000' MSL

- Avionics -- SET

- Airspeed -- <250 KIAS BELOW 10,000' MSL

- Landing Lights -- ON BELOW 10,000' MSL

- Approach Procedure -- REVIEW

# APPROACH

- Airspeed -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Flaps -- AS DESIRED

- Autopilot -- AS DESIRED

- Autothrottle – ARM (if using TO/GA mode for go-around)

# LANDING

- Airspeed -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Speedbrake -- ARMED

- Landing Gear -- DOWN AND CONFIRMED

- Flaps -- AS DESIRED

- Start Switches -- GEN

- Autopilot -- AS DESIRED

# LANDING ROLL

- Thrust Levers -- IDLE

- Autopilot -- CHECK DISENGAGED

- Autothrottle -- CHECK OFF

- Speedbrake Lever -- CHECK FULL UP

- Brake -- AS NECESSARY

- Thrust Levers-- REVERSE

- Thrust Levers -- IDLE AT 60 KIAS

# TAXI-IN

- Speedbrake Lever -- DN

- Flap Lever -- UP

- Pitot Heat -- OFF

- Engine Start Switches -- OFF

- Landing Lights -- AS DESIRED

- Flight Director -- OFF

- Transponder -- AS REQUIRED

# PARKING

- Parking Brake -- SET

- Start Levers -- CUTOFF

- De-ice -- OFF

- Lights -- AS REQUIRED

- Speedbrake Lever -- DOWN

- Transponder -- AS REQUIRED

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

135,844 lbs

VMO - Maximum Operating Speed (airspeed)

340 KIAS

MMO - Maximum Operating Speed (Mach)

.82 Mach

Turbulent Air Penetration Speed

280 KIAS/.73 Mach

VLO - Maximum Gear Operating Speed

235 KIAS

VLE - Maximum Landing Gear Extension Speed

270 KIAS/.82 Mach

Maximum Flap (Placard) Speeds

Flaps      KIAS

1	230

2	230

5	225

10	210

15	195

25	190

30	185

40	158

V1 - Takeoff Decision Speed (dry runway; flaps 5)

Standard temperature, sea level pressure altitude

150 KIAS (143,000 lbs)

143 KIAS (132,000 lbs)

Standard temperature, 5,000' pressure altitude

152 KIAS (143,000 lbs)

145 KIAS (132,000 lbs)

VR - Rotation Speed (dry runway, flaps 5)

Standard temperature, sea level pressure altitude

154 KIAS (143,000 lbs)

147 KIAS (132,000 lbs)

Standard temperature, 5,000' pressure altitude

157 KIAS (143,000 lbs)

149 KIAS (132,000 lbs)

V2 - Takeoff Safety Speed (dry runway, flaps 5)

Standard temperature, sea level pressure altitude

162 KIAS (143,000 lbs)

155 KIAS (132,000 lbs)

Standard temperature, 5,000' pressure altitude

161 KIAS (143,000 lbs)

155 KIAS (132,000 lbs)

VREF - Landing Approach Speed (flaps 30, gear down)

147 KIAS (136,400 lbs)

144 KIAS (132,000 lbs)

737-400 Specifications

Cruise Speed			477 kts, 550 mph, 885 kph

Engines				CFM56-3C1

Maximum Range			2,059 nm, 2,370 mi, 3,810 km

Service Ceiling			36,089 ft, 11,000 m

Fuel Capacity			5,311 U.S. gal, 20,104 L

Empty Weight-Standard		76,180 lbs, 34,550 kg

Maximum Takeoff Weight		138,500 lbs, 62,800 kg

Length				120 ft, 36.45 m

Wingspan				94 ft, 9 in, 25.9 m

Height				36.5 ft, 11.13 m

Seating Seats			147 to 168

Cargo Capacity			1,373 ft, 3 38.9 m3