# BEFORE STARTING ENGINE

- Brakes -- TEST and SET

- Electrical Equipment -- OFF

- Avionics Power Switch -- OFF

- Cowl Flaps -- OPEN

- Fuel Selector Valve -- BOTH

# ENGINE START

- Throttle -- OPEN 1/4-INCH

- Propeller -- HIGH RPM

- Mixture -- RICH

- Propeller Area -- CLEAR

- Master Switch -- ON

- Auxiliary Fuel Pump Switch -- ON

- Ignition Switch -- START (release when engine starts)

Then:

- Oil Pressure -- CHECK

- Auxiliary Fuel Pump -- OFF

- Flashing Beacon and Nav Lights -- ON as required

- Avionics Power Switch -- ON

- Flaps -- RETRACT

# BEFORE TAKEOFF

- Parking Brake -- SET

- Flight Controls -- FREE and CORRECT

- Flight Instruments -- CHECK and SET

- Fuel Quantity -- CHECK

- Mixture -- RICH

- Fuel Selector Valve -- RECHECK BOTH

- Elevator Trim -- SET for takeoff

- Rudder Trim -- SET for takeoff

- Throttle -- 1800 RPM

- Magnetos – CHECK <150 RPM drop on each; <50 RPM drop between

- Propeller -- CYCLE high RPM/low RPM/high RPM

- Suction Gauge -- CHECK

- Engine Instruments and Ammeter -- CHECK

- Annunciator Panel -- CHECK

- Throttle -- 800-1000 RPM

- Strobe Lights -- AS DESIRED

- Radios and Avionics -- SET

- Flaps -- SET for takeoff (0-20 degrees)

- Cowl Flaps -- OPEN

- Brakes -- RELEASE

# TAKEOFF

- Flaps -- 0-20 degrees

- Mixture -- RICH (may be leaned if necessary)

- Propeller -- HIGH RPM

- Throttle  -- FULL

- Elevator Control -- LIFT NOSE WHEEL (at 50-60 KIAS)

- Climb Speed --  70 KIAS (flaps 20 degrees)

- 80 KIAS (flaps UP)

- Flaps -- RETRACT

# NORMAL CLIMB

- Airspeed -- 85-95 KIAS

- Throttle --  23" MP or FULL (whichever is less)

- Propeller -- 2400 RPM

- Mixture -- 15 GPH or FULL RICH (whichever is less)

- Fuel Selector Valve -- BOTH

- Cowl Flaps -- OPEN as required

MAXIMUM PERFORMANCE CLIMB

- Airspeed -- 80 KIAS at sea level to 72 KIAS at 10,000'

- Throttle -- FULL

- Propeller -- 2400 RPM

- Mixture -- LEAN

- Cowl Flaps -- OPEN

- Fuel Selector Valve -- BOTH

# CRUISE

- Throttle -- 15-23" MP

- Propeller -- 2000-2400 RPM

- Elevator Trim -- ADJUST

- Rudder Trim -- ADJUST

- Mixture -- LEAN

- Cowl Flaps -- CLOSED

# DESCENT

- Throttle -- AS DESIRED

- Propeller -- AS DESIRED

- Mixture -- ENRICH as required

- Cowl Flaps -- CLOSED

- Fuel Selector Valve -- BOTH

- Flaps -- AS DESIRED

- 0-10 degrees <140 KIAS

- 10-20 degrees <120 KIAS

- FULL <100 KIAS

# BEFORE LANDING

- Fuel Selector Valve -- BOTH

- Mixture -- RICH

- Propeller -- HIGH RPM

- Landing/Taxi Lights -- ON

- Autopilot -- OFF

# LANDING

- Airspeed -- 70-80 KIAS (flaps UP)

- Flaps -- AS DESIRED

- 0-10 degrees <140 KIAS

- 10-20 degrees <120 KIAS

- FULL <100 KIAS

- Airspeed --  60-70 KIAS (flaps FULL)

- Trim -- ADJUST as desired

- Touchdown -- MAIN WHEELS FIRST

- Landing Roll -- LOWER NOSE WHEEL GENTLY

- Braking -- MINIMUM REQUIRED

# AFTER LANDING

- Flaps -- UP

- Cowl Flaps -- OPEN

# SECURING AIRPLANE

- Parking Brake -- SET

- Throttle -- IDLE

- Electrical Equipment, Avionics Switch, Autopilot -- OFF

- Mixture -- IDLE CUT-OFF (pulled fully out)

- Ignition Switch -- OFF

- Master Switch -- OFF

- Fuel Selector Valve -- LEFT or RIGHT to prevent crossfeeding

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

3,100 lbs

VA - Manoeuvring Speed

110 KIAS (3100 lbs)

101 KIAS (2600 lbs)

88 KIAS (2000 lbs)

VFE - Maximum Flaps Extended Speed

140 KIAS (0-10 degrees flaps)

120 KIAS (10-20 degrees flaps)

100 KIAS (20 degrees-Full flaps)

VNO - Maximum Structural Cruising Speed

140 KIAS

VNE - Never Exceed Speed

175 KIAS

VS  -  Stalling Speed (maximum weight; rearward c.g.)

40 KIAS

VSO - Stalling Speed in the Landing Configuration

(Maximum weight; rearward c.g.)

35 KIAS

VX  -  Best Angle-of-Climb Speed

63 KIAS (sea level)

66 KIAS (10,000')

VY  -  Best Rate-of-Climb Speed

80 KIAS (sea level)

72 KIAS (10,000')

Best Glide Speed

75 KIAS

Specifications

Maximum Speed				145 kts, 167 mph, 269 kmh

Cruise Speed				140 kts, 161 mph, 259 kmh

Engine					Textron Lycoming IO-540-AB1A5 230 hp

Propeller					McCauley 3-bladed constant speed

Maximum Range				968 nm, 944 sm, 1,519 km

Service Ceiling				18,100 ft, 5,517 m

Fuel Capacity				92 U.S. gal, 333 L

Empty Weight				1,910 lb, 854 kg

Maximum Gross Weight			3,110 lb, 1,411 kg

Length					29 ft, 8.84 m

Wingspan					36 ft, 11 m

Height					9 ft, 2.77 m

Seating					Up to 4

Useful Load				1,200 lbs, 557 kg