ELLEHAMMER II - Technical description

It had been 1 and a ½ wing of mixed design with linen wings, framework made of pipes, three - cylinder radial engine and a classical three - cycle gear.

There was no fuselage (at least not in nowaday’s sense). There was a framework with attached upper wing. Lower wing was in the middle of the framework, under the wing was the engine, pilot’s seat and the landing gear. The design of the wings was very simple having one beam and ribs made of bended reinforcing bars with the sewed on fabric. The movable elevator was attached to the rear edge of the wing.

The landing gear comprised three unsprung bicycle - type wheels.

The engine had been one of Ellehammer’s design and it was very simple. The engine was air - cooled and the propeller was of four blade type very similar to that used later by Grade (coal shovel type). It was a simple design made of two tubes welded together with attached steel blades having riveted reinforcing ribs.

Data

Span:			9.35 m

Length:		            6.2 m

Wing area:		37.0 m2

Take off weight:	180 kg

“Cruise” speed:	            57 km/h

Ellehammer II had been an improved design of Ellehammer I. First flight took place on 16th August 1906. The plane had been tied by the rope and supposedly there was no pilot. First piloted “flight” took place on 12th September 1906. The flight was really just a series of jumps and so Ellehammer decided to rebuild the plane again by adding one more wing (the more wings the more lift) and to increase the power of the engine to 35 hp. The plane had thus been completely changed and had been named Ellehammer III - but that is a different story.

Translated and Composed by Ladislav Sebo