Cockpit Check

- Check Parking Brakes ON

- Check Spoilers DISARMED

- Check Flaps UP

- Check Throttle IDLE

- Check Fuel CUTOFF

- Check Autopilot OFF

- Check Autothrottle DISARMED

- Check Fuel Quantity on Board

Before Startup

- Master Battery & Avionics ON

- Air Conditioning ON

- Autobrake RTO

- A/P Yaw Damper ON

- Nav. & Strobe Lights ON

- Tune COM & NAV radios

- Get ATIS Information

- SET Altimeter QNH

- Review Departure Procedure

- SET A/P ALT, IAS, HDG, CRS

Startup & Pushback

- Request Startup & Pushback

- Passenger Signs ON

- Set Pushback Distance & Direction

- Call for Pushback

- Parking Brakes OFF

- Wait for Push Back

- Parking Brakes ON

- Wait for Hand Signal

- Start Engine 2, 3, 1 then 4

- Check Engine Instruments

Taxi to Runway

- Landing & Taxi Lights ON

- Pitot Heat  &  Anti-Ice ON

- Request Taxi

- Check for Traffic

- Parking Brakes OFF

- Test Brakes

- Test Flight Controls

- Taxi to Holding Point

- Parking Brakes ON

Before Takeoff

- Flaps T/O position (10°, 3 notches)

- Trim T/O position (showing +18)

- Transponder ON & Squawk SET

- Report Ready for Departure

Takeoff

- Check for Traffic

- Parking Brakes OFF

- Line Up Runway

- Stand on Brakes

- Throttle Spool - Up (40% N1)

- Check Engine Instruments

- Throttle T/O power (96% N1)

- Release Brakes

- Rotate to 12° at Vr (145-175)

- Gear UP at Positive Climb Rate

 Flaps UP at V2+15 (175-203)

- Landing & Taxi Lights OFF

- Autopilot ON at 1000 AGL

- Autobrake OFF

Climb

- Throttle CLIMB power (92% N1)

- Check Engine Instruments

- Autothrottle ARMED at 240 KIAS

- SET IAS Hold 250 and VS 3200

- SET Altimeter 1013 at Trans. Alt.

- SET IAS Hold 300 at FL 100

- Adjust VS to maintain 92% N1

- SET Mach Hold 0.69 at FL 240

Cruise

- Passenger Signs OFF at EOC Pt.

- SET Mach Hold 0.85 in Level Flight

- Monitor Engine Instruments

Descent

- Request Descent

- Passenger Signs ON at BOD Pt.

- SET Mach Hold 0.69 and VS 2500

- Check Engine Instruments

- SET IAS Hold 300 at FL 240

- Review Arrival Procedure

- Reduce Speed to 250 at FL 100

- Tune COM & NAV radios

- Get ATIS Information

- SET Altimeter QNH at Trans. Alt.

Approach

- Request Landing Clearance

- Check Spoilers DISARMED

- Reduce Speed to 215 at IAF

- Flaps DOWN 10° (3 notches)

- Check Engine Instruments

- Review Go Around Procedure

- SET Radio Altimeter DH / MDA

- SET A/P CRS Approach Course

- Landing & Taxi Lights ON

- Report Established

Final Approach

- A/P APP or NAV Hold Mode SET

- Reduce Speed to 185 when G/S active or 1nm before descending

- Throttle APPR power* (58% N1)

- Flaps DOWN 20°  25°

- Gear DOWN at FAP / FAF (on G/S)

- Flaps DOWN 30° at OM or 6nm out

- Autobrake AUTO (1, 2, 3 or MAX)

- Autospoilers ARMED

- A/P Yaw Damper OFF

- Autopilot OFF at 1000 AGL**

- Throttle Back at 500 AGL to reach Vtt over RWY Threshold (141-154)

- Decide Land or Go Around at MAP

( Go Around )

- Autopilot & Autothrottle OFF

- Throttle T/O power (96% N1)

- Rotate to 15° pitch

- Flaps UP 20° (by 2 notches)

- Gear UP at Positive Climb Rate

- Proceed Takeoff Checklist 

* Autothrottle is allowed during approaches, but manual throttle control result in a much smoother ride.

** Automatic approach is allowed on ILS runways down to DH but not under 100 feet

Landing

- Reduce VS to 500 at 100 feet AGL

- Throttle Back & Flare at 50 feet AGL

- Spoilers DEPLOYED on Touchdown*

- Reversers ENGAGED when Nose Gear is down

- Throttle IDLE at 60 knots

Taxi to Terminal

- Flaps UP

- Spoilers DISARMED

- Transponder STANDBY

- Check for Traffic & Vacate

- Report Clear of the Active

- Taxi to Terminal Gate

* In case they don't deploy automatically.

Before Shutdown

- Throttle IDLE

- Parking Brakes ON

- Passenger Signs OFF

- Pitot Heat  &  Anti-Ice OFF

- Landing & Taxi Lights OFF

Shutdown

- Request Shutdown

- Shutdown Engine 4, 1, 3 then 2

- Nav. & Strobe Lights OFF

- Air Conditioning OFF

- IMPORTANT NOTICE: these checklists and operating procedures are intended for simulated flight use only! Do not use them in real life aviation!