B58 INFORMATION, CREDITS AND COPYRIGHT

By Robert Kirkland

We want to thank the following people for their instance in developing this project.  It has taken 10 months  to get her ready to fly  and  without these folks  and their contributions  we would still not be ready.

THE FSD STAFF

Robert Kirkland

Jim Goldman

Gerry Schmidt

Larry Teele

PHOENIX SIMULATION SOFTWARE

Alex Bashkatov for assistance with the gauges

David Lewis for FSMP

OTHERS

Mike Hambly for the sounds

Randy Brewer who  provide  invaluable technical information, you can visit his site at: hppt://www.teleport.com/~brewer/b58/index.html
Richard Melville  for his gauges

Chuck Dome  for his advice and great programs

All the gauge designers whose efforts have assisted us in completing this project

NEW FOR FS98:

This aircraft has been created exclusively for FS98.

IT DOES NOT NEED TO BE CONVERTED FOR USE WITH FS98 (WE ALREADY DID THAT)

Check the HTML document file for more information on the original aircraft.

The sound files are original to this aircraft and are the work of Mike Hambly.

The airframe, textures and flight dynamics are the work of Jim Goldman

The Panels and gauges are the work of Robert Kirkland and Alex Bashkatov

# INSTALLATION

The easiest way to install the files is using WINZIP or a similar unzipper that uses long file names. Simply set the target for extraction to your FS98 main directory and WIN ZIP will do the rest. Make sure to have the "USE FOLDERS NAMES" checked before extracting

# FOR EXAMPLE

If you main directory for FS98 is C:\FltSim98 then that should be your target directory in WINZIP

SHORT COURSE IN FILE STRUCTURES

The following files go in the following directories

All *.gau files 	to the FS98 Gauges directory

ALL *.bmp	to the FS98\aircraft\B58\panel directory

Panel.cfg	to the FS98\aircraft\B58\panel directory

All *.wav files  	to the FS98\aircraft\B58\sound directory you

All #.af files  	to the FS98\aircraft\B58\texture directory

aircraft.cfg file	to the FS98\aircraft\B58 directory

.MDL File	to the FS98\aircraft\B58\model directory

Model.cfg 	to the FS98\aircraft\B58\model directory

Sound.cfg 	to the FS98\aircraft\B58\sound directory

ADDITIONAL INSTRUCTIONS:

IF you have any problems let us know at WWW.FLIGHTSIM-DEVELOPERS.COM.

FLY!

COPYRIGHTS ETC:

These files are copyright (c) 1999 by Robert Kirkland, Mike Hambly, Gerry Schmidt, Larry Teele and Jim Goldman, Flight Sim Developers.

All rights reserved. No duplication or bundling of this package for redistribution without prior written consent of this author. This means shareware or payware. No exceptions. No modification of the enclosed files is permitted without the written permission of the authors. Posting of this aircraft package to AOL or to CIS is not permitted without the written permission of the authors. Posting of this aircraft package to any other site is not permitted without the written permission of the authors. You may not add or subtract any files to this archive without the written permission of this author.

Although we appreciate the desire to use our files on other websites, we MUST insist on the following guidelines regarding their use. It is certainly not acceptable to simply use the files without credit as this is misleading and could be construed as an attempt to defraud Flightsim Developers.

Please also understand that FSD is US Corporation and not just a group of amateurs. We must protect our livelihood.

Let us explain.. Flightsim Developers is both a Commercial Producer and a Freeware organization. We spend many hours creating the files you see around the World Wide Web. Sometimes a file appears on a download site  unauthorized and occasionally these pirated files have been altered or cut down. In these cases we are unable to provide the high level of support that we usually offer but inevitably get loads of e-mail asking why they don't work. Bearing this in mind, If you agree to our terms we will fully support the files and reply to any user problems that may arise. We Hope you understand.

Terms and conditions

#1 	Any file belonging to Flightsim Developers must be used in its original, complete and unaltered form. It must remain intact complete with text files and images.

#2 	Alongside EACH file used there must be a Full credit for the AUTHOR and for FLIGHTSIM-DEVELOPERS.

#3 	A link to www.Flightsim-Developers.com MUST appear on EVERY Page where our files are used.

We hope you find these terms agreeable but understand that If you do not you will refrain from using FSD Files on your Site.

SPECIAL THANKS

A special thanks to all our test pilots for their many hours of testing. They told their wives it was work.<g>

The majority of gauges are FlightSim Developers originals, others are renamed stock MICROSOFT GAUGES, the Fuel switch and Auto Coordination switch are the work of Chuck Dome, and the GPS Gauge which is by David Markovitch.

Feel free to drop us a message on any improvements you would like to see.

Thank you for downloading our work....

Robert Kirkland 	 turk@pilot.infi.net

Jim Goldman     	 jgoldman@hern.org

Test of the Convair B-58 Hustler

By Larry Teele

Flightsim Developers have really pulled out all the stops with their newest release of the U.S. Air Force Convair B-58 “Hustler”.  This four engine delta wing supersonic bomber is an absolute visual treat and delivers equally impressive performance.    With completely new and innovative techniques, outstanding attention to detail and a reputation for “excellence”,  FSD’s B-58 is sure to satisfy even the most demanding.

The visual prominence and anticipated power of this aircraft is immediately evident at the runway numbers.  The aircraft’s graceful lines are clear and bold – there’s lots to see.  3-D effects come into prominence as they enhance the aluminum skin, color trains, detailing and squadron emblems.  Panning views of the bomber will bring out some unique features;  Highly detailed landing gear/wheels and doors are seen and even the engines inlet turbine vanes can be seen rotating.  Visual movements of the combined aileron/elevator control surfaces and rudder further heighten the Hustler.  The deep, unique engine sounds are realistic and further enhance the aircraft.

Panel configuration, functions and clarity simply couldn’t be better on the B-58.  This is all achieved by the accurate inner cockpit transformation of the angular windshield frame, the correct balance of forward viewing area to panel, the wonderful colors and textures and the spectacular array of flawless gauges, many which are seen in flight simulation for the first time.  The left main panel houses the landing gear selector and indicator, drag chute handle (it really works) and lighting panel.  Shift+2 brings up the throttle panel complete with chronometer and fuel tank selectors switches.  Shift+3 is the navigational and communications package/panel complete with the SST inertial (read separate instructions) and the VOR, ADF and VHF clusters.  A simple GPS is included as well.  These switches are easily operated and clearly marked.   Center main panel is split right down the center with basic aircraft /navigation/attitude instrumentation on the left and the amply array of analog engine indicators on the right.  Right main panel houses fuel quantity, fuel flow, afterburner indicator lights, blinking oxygen regulator, aircraft trim indicators and pneumatic pressure indicators.  The G-meter is a first time incorporation in flight simulation and gauges aircraft stress and “G” loads.  Shift+4 brings you into the Tactician Navigation station and Shift+5, the Bombardiers.  Small outside windows and minimal gauges accompany these views.  Shift+6 brings up the full scale inertial navigational system.  Be sure and read the documents on this superb piece of equipment.  Once you get the hang of it, it’s quite simple to operate and adds immensely to the simulation.

Ground handling of the Hustler couldn’t be easier – the aircraft will turn a full circle in the width of a normal runway with only rudder inputs.  The aircraft accelerates quickly and has a rotate speed of 170 knots with liftoff of 175.  A very favorable climb ratio was attained at 18 to 20 degrees, 4500 Feet per minute while maintaining 200 knots.  The flight characteristics offer a formidable task in pilot control as roll inputs must be countered with opposite stick to maintain the bank angle.  The aircraft is simply a joy to fly.   With an abundance of power, speeds in excess of mach 2 are achieved.  The Hustler stalls at 165 knots (and recovers well)  and so a target approach speed of 170 is about ideal.   (remember, this is a controlled crash).  The B-58 handles very well under autopilot command and you’ll need it to free yourself in setting up your radios and inertial system.  Setting up for landing requires a long approach in order to bleed off airspeed.  Early gear down is recommended.  A hard brake at pattern altitude will also produce the desired results in obtaining the correct approach speed.  Controlling your line up to runway centerline while at the same time maintain approach speed will be challenging to all pilots.  At these high approach speeds, drogue chute deployment is critical after touchdown and is most effective.

All in all, I found Flightsim Developers Convair B-58 Hustler to be a well presented and a most powerful addition to the flight simulation community.

The aircraft has great visual appeal, formidable speed, outstanding flight characteristics and puts you to task with its operation.  The aircraft will definitely on my flight line for some time to come.  Go see for yourself.

HISTORY

The Convair B-58 Hustler was the first supersonic bomber to be put into operational service, entering service with the USAF in March of 1960. Although the B-58 was destined never to fire a shot or drop a bomb in anger, it provided a key component of the Strategic Air Command's deterrent capability during the 1960s. Despite its high performance and sophisticated equipment, the service of the B-58 was destined to be rather brief, the aircraft serving with the Strategic Air Command for only a decade before being consigned to storage. Part of the reason for this rather short service was the B-58's rather high accident rate, but the major factor was the intercontinental ballistic missile, which entered service at the same time as the B-58.

The origin of the B-58 can be traced back to the period just after the end of the Second World War, at the time of the creation of the independent United States Air Force. In May of 1947, Maj. Gen. Curtis E. LeMay, at that time Deputy Chief of Air Staff for Research and Development, wrote a letter to Lt. Gen. Nathan F. Twining, chief of the Air Materiel Command, requesting that work begin on a new jet-powered medium bomber that would be ready for service by the late 1950s. The bomber should have a combat radius of 2500 miles, a cruising speed of at least 500 mph, and a gross weight of 170,000 pounds. It was proposed that the development of such an aircraft would follow the development of the B-52.

General LeMay's proposal led the Air Staff to solicit ideas from the leading US maker of bombing aircraft, the Boeing Airplane Company, as well as to several other manufacturers. At this stage, the project was rather ill-defined. In October of 1947, things had begun to firm up sufficiently that the War Department submitted a requirement for a new medium bomber to the aviation industry. The aircraft was to weigh less than 200,000 pounds, have a 2000 mile radius, and be able to carry a 10,000 pound bomb load. It was tentatively assigned the designation XB-55. Boeing submitted the winning proposal, and a Phase I contract was initiated with FY 1948 funds.

However, in the immediate postwar environment, funding for military projects was in short supply and it was decided that the initial design study for the XB-55 would be converted into a paper study to explore new aeronautical technologies. In particular, the Air Force began to explore the potential of delta wing configurations and began to consider the possibility of bomber designs capable of supersonic flight.

Some of this work had actually gotten started before the advent of the XB-55 project, and several companies had launched informal studies. Among the initial approaches to the design of a long-range supersonic bomber was the Generalized Bomber Study (better known as GEBO). GEBO began with the exploration of the feasibility of a delta-winged aircraft weighing about 150,000 pounds. This began in October 1946 under an Air Force contract given to Convair, since that company had the most experience with delta-winged aircraft. Pre-GEBO studies by Convair of the abortive XF-92 delta-winged interceptor had given that company valuable experience with delta wings.

The delta wing was basically the idea of Dr. Alexander M. Lippisch, a German aeronautical engineer, although NACA had independently explored many of the advantages of delta wings during the war. In postwar years, US government agencies and many US aircraft corporations studied captured German reports on delta-winged aircraft.

By June of 1948, Convair had looked at 10,000 different configurations which explored the effects of different wing areas, aspect ratios, thickness and sweep. In addition, several different types of propulsion systems were considered, both turbojet and turboprop.

At the suggestion of the AMC, the USAF asked Convair to continue its study on the development of future long-range supersonic bombers. This study was formalized in June of 1949 with the granting of a contract for further detailed study.

On January 27, 1949, the AMC was directed to cancel the XB-55, since the projected B-47 production rate had reached the point that another subsonic medium bomber would be unnecessary. However, the general requirement for a high-performance medium bomber remained intact.

One of the positive results of the cancellation of the XB-55 project was that it freed up some scarce funds for additional development. Brig. Gen. Donald Putt, Director of the Research and Development Office and Deputy Chief of Staff for Materiel, recommended that the AMC ask the aircraft industry for new and possibly unconventional proposals for intercontinental bombers. A second Generalized Bomber Study (known as GEBO II) was initiated. The design parameters were a radius of 1200 to 2500 miles with a 10,000 pound bomb load, a cruising speed of more than 450 knots, a combat altitude greater than 35,000 feet and a takeoff distance of less than 6000 feet.

In January of 1950, Convair, as part of its work on GEBO II, began to explore a parasite concept. They proposed a fairly small delta-winged aircraft which would be carried part-way to its target underneath a B-36. The aircraft was to carry a two-man crew and would have four turbojet engines. The aircraft would have a composite construction, with a droppable pod containing a bomb bay, a radar scanner, three expendable engines, and the fuel. Launch weight was 100,000 pounds, but landing weight was only 17,900 pounds. Maximum altitude was 48,500 feet. Maximum speed over the target was Mach 1.6, and service ceiling before bomb drop of 52,000 feet. No defensive armament was to be carried, since it was expected that the high performance of the aircraft would make it immune to interception.

The Air Force was highly interested in this parasite concept, but there was some opposition. Many felt that a parasite bomber would be much more expensive than a single aircraft designed to accomplish the same mission, and that the mothership/parasite combination would be quite vulnerable to attack while attached.

In April of 1950, the GEBO II specification was changed to provide for a radius of 3500 to 4500 miles, with speeds as great as Mach 1.5. This changed specification was based on Convair's parasite proposal. By the fall of 1950, Convair had settled on a 100,000-pound aircraft powered by five engines. Three engines would be expendable, with one of the engines being mounted in the centerline pod itself and one expendable engine being mounted in a pod underneath each wing. The centerline pod would also contain fuel and a single nuclear weapon. The main module had a delta wing with two turbojets in partially buried wing nacelles. Once airborne, the B-36 would carry the parasite 2000 miles toward the target, release it and return to base. The parasite would then use its five engines to accelerate to a Mach 1.3 cruising speed. Over the target, the speed would increase to Mach 1.5. The parasite would then release the pod containing the nuclear warhead and return home at Mach 1.3. Once safely out of the combat zone, the aircraft would drop its two wing-mounted expendable engines and complete the trip back home at Mach 0.9. The aircraft would have no defensive armament, but it would be equipped with a full suite of electronic countermeasures equipment.

In the meantime, the Boeing Airplane Company, now freed up by the cancellation of the XB-55 project, began to study the possibility of a high-performance medium bomber. Performance objectives included a combat radius of 3000 miles at an altitude of 50,000 feet. The aircraft would be capable of supersonic speed of Mach 1.3 within 200 miles of the target. After looking at several different configurations, the Air Force selected the Boeing Model 484-405B as having the highest potential. The 484-405B was a fairly conventional design, with a low aspect ratio, high-mounted wing with a sweepback of 47 degrees. A bomb bay similar in size to that of the B-47 would be provided. Gross weight was 200,000 pounds. The aircraft was to be powered by four Pratt & Whitney J57-P-5 afterburning turbojet engines. The engines were to be mounted side-by-side, two in the inboard section of each wing. Because the wing was thin in order to make it possible to achieve supersonic performance, the fuel had to be housed entirely within the fuselage. The fuselage housed a pressurized cabin for a crew of three. A remotely-controlled tail turret was to be fitted.

By the end of 1950, the Bombardment Branch of the Air Materiel Command's Aircraft and Guided Missiles Section began to prepare a detailed military specification for both the Boeing and Convair proposals. Based on the AMC proposal, which was in turn based on input from both the Boeing and Convair design studies, requests were made for funds for beginning projects. The supersonic bomber was now officially a part of the Air Force's future plans.

On January 26, 1951, following the completion of the detailed study, Convair proposed that it develop a long range supersonic reconnaissance bomber. The project was given the number MX-1626 by the AMC under contract AF33(038)-21250. The contract called for partial Phase I development (initially without a mock-up) of a bomber/reconnaissance aircraft based on the GEBO II studies. However, the Convair proposal now had only three engines rather than five, with two being permanently mounted in the wing nacelles and one expendable engine being mounted in the droppable pod.

In February, the competing Boeing project was given a development contract by the AMC under the designation MX-1712 and contract AF33(038)-21388. Boeing's contract called for Phase I development of two bomber/reconnaissance aircraft through wind tunnel testing, engineering and mock-up. Initial flight dates for both designs were tentatively set for late 1954.

The MX-1626 was a two-seat delta-winged aircraft consisting of two main components -- a lower droppable bomb-pod and an upper main return component. The return component was a complete aircraft with a tricycle undercarriage to be used only for landing. The aircraft was to be launched from a trapeze which extended downward from its mother craft, but there is a Convair drawing showing the installation of a jettisonable landing gear that permitted takeoffs when the pod was fitted.  The return component featured a delta wing with an area of 1200 square feet and a total span of 47 feet. The fuselage was 82 feet long. At the rear of the fuselage was a triangular-shaped vertical tail. A pair of non-afterburning General Electric J53-GE-X25 turbojet engines were mounted in mid-wing pods. The long pointed pod was integrated into the bottom of the aircraft, and featured three large fins separated by 120 degrees. The free-falling bomb pod was to be roll-stabilized by the control surfaces in the two upper tails. A third J53 engine was to be mounted inside the pod and would be expended when the pod itself was dropped. A maximum speed over target of Mach 1.7 was anticipated, and a maximum total mission radius (carrier plus parasite) of 4000 nautical miles was anticipated.

Convair's parasite proposal turned out to be very short-lived. The parasite idea had arisen at a time when cost factors were particularly important, and it was thought that such an approach would provide a cost-effective answer to the problem of long-range strategic bombing. However, it soon became apparent that the two aircraft would require complex navigational equipment so that they could find each other on the return part of the mission. While joined, the two aircraft would be especially vulnerable to attack. In addition, the two-aircraft attack system would be much more expensive to build and maintain than a single bomber. Consequently, in December of 1951 the MX-1626 design was drastically revised. The parasite idea was abandoned in favor of a single aircraft that would be capable of being refueled in mid-air.

The third expendable engine in the bomb pod was eliminated, and afterburners were added to the aircraft's remaining two engines. A landing gear capable of supporting both landings and takeoffs would have to be provided. Gross weight rose to about 126,000 pounds and the number of crew members rose to three (pilot, navigator-bombardier and defense systems operator).

On February 1, 1952, the USAF issued General Operational Requirement SAB-51, where SAB stood for Supersonic Aircraft Bomber. It called for a multi-mission strategic reconnaissance bomber capable of carrying 10,000 pounds of bombs. It had to be capable of operating in all weather conditions, and had to be able to achieve a combat radius of 5000 miles with a single outbound in-flight refueling. It had to be capable of supersonic performance at altitudes of 50,000 feet or more and of high subsonic performance at lower altitudes. It was considered important that the aircraft be fairly small, since this would reduce the radar reflectivity and make the aircraft harder to detect. The Air Force wanted production to begin within five years.

On February 26, 1952, the SAB-51 GOR was revised in a document which came to be known as Directive Number 34. It was conceded that it was unrealistic to expect the rapid development of a high-altitude, long-range supersonic bomber that could also be suitable for low-altitude high speed missions. Consequently, the low-altitude performance requirement was dropped. Following discussions with the Air Council and representatives of the ARDC, SAC, the Rand Corporation and the Scientific Advisory Board, the Air Force endorsed this recommendation, and the revised SAB became formalized on September 1, 1952 as SAB-52-1. However, the Air Force still wanted the aircraft by 1957.

At the end of February 1952, General J. W. Sessums, ARDC Deputy for Development recommended that it would be better to forego the traditional industry-wide competition that would ordinarily be held for the supersonic bomber project. Time and money would be saved if contractors could be selected on the basis of the proposals already submitted. Although the AMC felt that the Boeing and Convair proposals offered the best hope for a supersonic bomber, the AMC had requested informal proposals from other manufacturers, including Douglas, Lockheed, Martin and North American. However, only two of the last four companies actually submitted proposals, and these were not very interesting. Shortly thereafter, the Wright Air Development Center endorsed this strategy and called for a competition between Boeing and Convair, the only two companies to have submitted proposals that were of any significant interest. The Air Force was now committed to the advanced bomber project, and placed heavy emphasis on the MX-1626 and MX-1712 programs. It requested that two parallel Phase I projects be initiated, thus engaging Boeing and Convair in an official competition. It was anticipated that contracts would be issued to both competitors in the fall of 1952 for detailed designs and mock-ups, followed by the selection of a winning design in February or March of 1953. The emphasis would continue to be on minimum size and maximum altitude and speed performance.

The financing of the Phase I development of two parallel projects was extremely difficult to support, especially during a period of financial austerity. The Boeing MX-1712 program had benefited somewhat from the XB-55 cancellation, which freed up some Boeing developmental funding for the new project, but Convair's MX-1626 was experiencing a severe funding problem. In late February, the MX-1626 program was almost cancelled due to the lack of funds, and the project remained in some danger until May 15, when enough additional funds were obtained to keep the project going.  In June of 1952, Convair proposed that they move the engines to nacelles placed underneath the wing. The engines were to be either two J75-P-1s, two J67-W-1s, four J57-P-7s, four J73s or two J77s.

Directive 34 had also dictated that the project use the weapons system concept, in which the equipment, weapons, electronics and components of the aircraft would be developed as an integrated whole to ensure that each component would be compatible with the others. By mid-1952, both Boeing and Convair had made considerable progress in bringing their projects into compliance with the weapons system philosophy. In the process of making their designs conform with the requirements of Directive 34, Convair's MX-1626 was now known as MX-1964 and Boeing's MX-1712 was now called MX-1965. The USAF designations B-58 and B-59 were tentatively assigned to the two competing projects, even though no production orders were yet forthcoming.

In the summer of 1952, the Wright Air Development Center concluded that a less costly alternative would be to select just one of the two competitors even before the design and mock-up stage was reached.

The small bomber concept was endorsed by the Air Force Council and by General Hoyt S. Vandenberg, who was Chief of Staff of the Air Force. However, General Curtis LeMay, head of the Strategic Air Command, generally favored larger bombers with longer ranges. SAC felt that high performance alone would not necessarily assure mission success, and that the small supersonic bomber's lack of range would prevent it from operating without mid-air refueling from most forward bases. Despite SAC's objections, the Wright Air Development Center recommended that the Boeing/Convair competition be stopped.

Even though the Air Force thought that Convair's estimates of the MX-1964's supersonic drag and gross weight were overly optimistic, the Air Force felt that the Convair design was superior to the Boeing proposal, since they felt that the Boeing design would offer insufficient supersonic capabilities, and on November 18, 1952,

General Vandenberg formally announced that Convair was the winner of the contest.  On December 2, 1952, it was announced that the designation of the new bomber would be B-58. The Deputy Chief of Staff for Development endorsed a production schedule based on the four-year procurement of 244 B/RB-58s. The first 30 would be used for testing, and they would be reworked on the production line as problems appeared and were solved. This plan was based on the "Cook-Cragie" philosophy, in which the prototype phase was skipped. This plan, named for General Laurence C. Cragie, Deputy Chief of Staff for Development, and Orval R. Cook, Deputy Chief of Staff for Materiel, was rather risky and was really applicable only when there is a fairly high degree of certainty that the aircraft is actually going to go into production. The F-102 interceptor had been designed according to this principle.

On February 12, 1953, the Air Force gave Convair a go-ahead to begin detailed Phase I work on the XB-58 and XRB-58. At this point, only the basic concept had been approved, not any detailed design. On March 20, the Air Force indicated its acceptance of a firm configuration with a 60-degree delta wing with the trailing edge swept forward by ten degrees. A small amount of leading edge camber was provided to reduce drag due to lift. The aircraft was to be powered by four General Electric J79 turbojet engines, with the two inboard units mounted on underwing pylons and the two outboard engines mounted on the wing upper surface.

Even though Convair had been selected over Boeing, many revisions of the MX-1964 design were to follow. At the same time, development problems with Convair's F-102 interceptor confirmed the Air Force's suspicion that the initial estimates of the aerodynamic drag of a delta-winged aircraft had been overly optimistic. NACA engineer Richard T. Whitcomb's ideas on the Area Rule had been verified in December of 1952, which suggested that the cure for excessive transonic drag was to equalize the cross-sectional area at all points along the fuselage, thus producing a narrow ("coke bottle") fuselage in the region of the wing. The crew was to be three --a pilot, a navigator/bombardier and a defensive system operator. The defense was to be one 30-mm gun mounted in a remotely-controlled tail position.

The first development engineering inspection took place on August 17/18, 1953. At this stage, the B-58 mock-up was known as Configuration II. The requirements matched fairly closely with the specifications issued by Convair in August of 1952 as well as with the USAF demands issued in the September 1952 GOR. At this stage in the design, the fuselage of the B-58 still consisted of an upper component and a lower pod that were integral with each other rather than being separated by a pylon. The return component had a flat fuselage undersurface once the disposable pod component had been jettisoned. In addition, nose gear requirements were complicated by the fact that both the pod and the return component required a nose gear. In the development engineering inspection of August of 1953, it became obvious that this pod would have to be completely redesigned. In October of 1953, the Air Force authorized Convair to shorten the pod to a length of 30 feet and to separate it from the fuselage by a pylon. In addition, the search radar was taken out of the pod and put in the nose of the upper compartment. The droppable nose gear was eliminated, and external fuel tanks were added to compensate for the fuel lost due to the shorter pod, and the positions of the navigator/bombardier and defensive systems operator were reversed.

The revised B-58 was known as Configuration III. External fuel tanks were added to the wing tips in the interest of longer range. Configuration III also omitted the jump seat which had been requested in the original military characteristics. However, there were still problems being uncovered by the wind tunnel testing at the Wright Air Development Center and at NACA. In 1953, the contractor and the Air Force had decided to mount the four turbojet engines inside two split-cell strut-mounted underwing nacelles, reminiscent of the inner two-engine pod of the B-47 Stratojet. It was thought that this configuration would save weight, ease engine maintenance, and facilitate retrofit of J57 engines with new J79s. However, wind tunnel testing indicated that the split-cell nacelles induced extra drag on the pod-carrying B-58.  These problems caused a postponement of the Configuration III mock-up from May to September 1954.

It was planned that the first 30 aircraft would be used for tests and evaluation. The first 18 of these would be powered by Pratt & Whitney J57 engines, while the remainder would be powered by four General Electric J79-GE-1 turbojets (known in the prototype form as the J73-X24A).

By August of 1954, what was to prove to be the final B-58 configuration was chosen. The engines were now mounted inside four individual underwing pylons, and all fuel was contained internally and in the podded lower component. The fuselage was aligned to the modified transonic area rule for supersonic speeds. The external wing tanks were eliminated, and the tail area was increased to 160 square feet.

In the meantime, the Strategic Air Command was still unhappy with the B-58. A mid-1954 staff study had actually excluded the B-58 from its projected 51-wing bomber force of 1958-1965. There were fears from even the B-58's most fervent supporters that even the latest configuration might not meet all the requirements of the military specification, but they still believed that the aircraft should be built even if the Air Force could not actually use it as originally intended. By this time, almost 200 million dollars had been spent. There was some thought to reorienting the program to a research and development effort, and even some thought to canceling the program altogether. In June of 1955, a decision was made to restrict the program to just 13 aircraft. However, on August 22, 1955, this decision was reversed again and the B-58 was once again authorized for production. A wing of B-58s would be ready for service by mid-1960. However, there was at this time no mention of which branch of the Air Force that this wing would actually belong to.

In December of 1955 a definitive contract was issued to Convair for 13 aircraft and 31 pods. A second Letter Contract, AF33(600)-32841, issued on May 25, 1956, provided additional money to maintain B-58 production at a minimum sustaining rate through October of 1956. In the fall of 1956, the Air Force would decide if it should buy more aircraft.

FUSELAGE

The fuselage of the B-58 was of semi-monocoque construction and featured the standard bulkhead, former and longeron construction. The area between bulkheads 1 and 5 carried the crew compartments. The volume aft of bulkhead 5 and all the way to bulkhead 19 was devoted exclusively to fuel except for the navigation system stable table area between bulkheads 8 and 9. The portion of the fuselage aft of bulkhead 19 contained the deceleration parachute, the tail armament and electronic equipment.

The crew consisted of a pilot, a navigator/bombardier and a defensive systems operator (DSO), all seated in tandem in three separate compartments. The pilot sat in front, the navigator/bombardier in the middle, and the DSO in the rear. A crawlway between the pilot's station and the second crew station on the right side of the fuselage could only be used for maintenance of electronic equipment, but a crawlway between the second and third crew stations could be used for passage during flight. The navigator/bombardier's panel was equipped with bomb and pod dropping instrumentation, bombing system indicators and monitors, plus the navigation equipment. The DSO's panel contained passive and active defense system monitors.

Structurally, the three crew compartments comprised a single pressurized cabin, but structural bulkheads and equipment created a compartmentalization effect. Each compartment had a separate canopy hinged at the rear for entry and exit. The compartmentalization prevented direct vision or physical contact between crewmembers during flight. The pilot had a windshield with six adjacent panels, plus one panel on each side of the canopy. This afforded excellent outside vision, and the pilot could see parts of the exterior of the aircraft as well as the engine nacelle inlets. The navigator/ bombardier and the DSO only had small side windows.

The crewmembers were seated on individual ejector seats which were catapulted out of the top of the aircraft by a rocket engine. Problems with the originally-fitted SAC-type ejector seats when they were called upon to be used for emergency exit in the supersonic regime led to the development of an encapsulated ejection system developed by Stanley Aviation of Denver, Colorado. The unit protected the pilot against supersonic wind blasts, supplied oxygen and pressurization during an ejection at high altitude, absorbed landing impact and had survival equipment installed. Each capsule was an independently operating unit which required no outside power source.  The second and third crew stations were identical and were both ejected on vertical rails. The pilot's capsule was similar, but included a flight control stick and was ejected on slightly leftward-canted rails. A three-piece telescoping clamshell door was pivoted on each side of the seat. It was stowed above the crew member's head during normal flight. It was actuated by raising the ejection handle, causing the doors to rotate downward to form a pressure-tight capsule. The doors could be closed in about a quarter second after actuation. Emergency oxygen and pressure were automatically actuated by door closure. After the doors were closed, each crew member manually ejected his own capsule by squeezing the trigger on the ejection handle, which jettisoned the canopy and fired the rocket catapult initiator. During high speed ejection, capsule stability was provided by the stabilization frame and stabilization parachute. The recovery parachute was automatically deployed at a preset altitude.

Landing impact was cushioned by crushable cylinders and stabilization fins. For water landings, flotation bags were provided. During an emergency, the aircraft could still be flown while the pilot was encapsulated, and a small window in the capsule clamshell door provided a view of the instrument panel, while the pilot's control stick permitted controlled flight.

ENGINES

The YJ79 was an axial-flow turbojet with a 17-stage compressor and a three-stage turbine. It was equipped with a variable exhaust nozzle, and air entering the compressor section was automatically controlled by variable positioning inlet guide vanes.

The variable exhaust nozzle system automatically controlled the exhaust area to provide for optimum thrust and specific fuel consumption under varying engine operating conditions. It also protected the engine from overheating. The system consisted of primary and secondary nozzle flaps, plus associated control units and actuators. Each primary nozzle control was mechanically interconnected with the engine throttle and synchronized so that throttle movement would automatically result in proper primary engine nozzle setting. The secondary nozzles were used to provide maximum thrust and reduced drag during cruise and military operating ranges.  These were opened during idle and afterburner operation, and were closed for operations in the cruise and military ranges.

Each engine nacelle was equipped with a variable-position spike which could be moved forward or aft in the inlet to maintain an efficient airflow to the engine throughout the speed range of the aircraft. During normal operation, control of the spike was completely automatic. The spike remained in the aft position until an airspeed of Mach 1.42 was reached. At that speed, a switch in the air data computer closed and activated the system. The transducer then moved the spike forward to the appropriate location for the particular airspeed. Engine cooling was provided by using two scoops located in the air inlet of each nacelle. Ram air was passed via these scoops through a series of bypass flaps into the hydraulic oil cooler, then aft between the engine and the nacelle walls to be expelled into the engine exhaust gases.

The fuel system was the most complex and sophisticated yet installed in an operational aircraft. The JP-4 fuel was stored in four main tanks, termed forward, aft, reservoir and balance units. Two more fuel tanks were installed in the under-fuselage pod (a total of 4172 US gallons in the MB or LA pod and 3885 US gallons in the TCP pod). The fuel system could operate with or without the pod being installed. The forward portion of both wings, as well as the fuselage volume between bulkheads 5 and 6 comprised the forward tank, which could accommodate up to 3202 US gallons of fuel. The aft portion of both wings and the fuselage volume between bulkheads 9 and 12 comprised the aft tank, which was capable of carrying up to 5893 US gallons of fuel. The fuselage section between bulkheads 6 and 8 comprised the 610-gallon reservoir tank, and the fuselage section between bulkheads 12 and 19 comprised the 1219-gallon balance tank. The reservoir tank acted as an accumulator tank by utilizing an autotransfer system which maintained a specified tank level until the other tank supplies had been depleted. The balance tank was not used for direct engine supply, but fuel could be transferred from the balance tank to the forward tank when needed. The center of gravity could be maintained either automatically or manually by transferring fuel between the forward, aft and balance tanks.

The B-58 was equipped with a mid-air refueling system, mounted in the upper portion of the nose radome some 45 inches ahead of the pilot's windshield. It consisted of a receptacle for a KC-135A flying boom probe. When not in use, the system was covered by a door which was normally flush with the contour of the radome. When the slipway door was opened, it formed a guide for the entry of the flying boom.

The B-58 was equipped with an emergency fuel dump system. The fuel could be dumped via a probe which extended two feet outward from the left side of the balance tank just aft of the wing trailing edge.

STRUCTURE/WINGS

The wings of the B-58 were of fully cantilever, modified delta type with cambered leading edges and outboard tips. They incorporated multi-spar construction with a sandwich panel covering that was secured with titanium screws. The leading edges were of sandwich-type skin construction preformed to shape without any internal bracing.

The internal structure of the wing consisted of multi-spar construction with bulkheads at the points of major load. The two inboard bulkheads located in the wheel well area were large channels that were flanged away from the wheel well to allow maximum clearance for main landing gear retraction. The spars were made so that contact with the wing skin was on a curved surface to allow the wing skin to more closely approach the contour of the desired airfoil. The wing sandwich panel was made up of aluminum sheet skins, adhesives, fiberglass and aluminum honeycomb core, and a machined aluminum grid.

The entire wing served as an integral fuel tank which was sealed from the outside during construction. Free-flow openings in other spars and bulkheads allowed fuel to flow between sections. Check valves prevented fuel flow between left and right wing sections when one wing was high. The wing panel fiberglass cores served to insulate the fuel in the wings from the external heat.

The vertical tail was a swept-back, truncated structure with spars and ribs providing the internal framework. The fin cap was fabricated of laminated fiberglass. The rudder had a full-depth aluminum core with an aluminum alloy skin bonded to it. It was hinged at 11 points along the front spar. The elevons were made of steel sandwich panels with a brazing alloy used as a bonding agent.

The control surfaces consisted of a rudder and two elevons. The pre-production B-58s also had two resolution surfaces, located inboard of the elevons. These were used to mask out the effects of mechanical backlash in the longitudinal control system and were completely automatic. However, they were found to be redundant to the primary trim system and were eventually eliminated from all production aircraft, and were removed from existing pre-production aircraft as well.

The flight control system incorporated three-axis damping, constant stick forces throughout the entire speed range, and continuous g protection, making it virtually impossible to manually overstress the airframe when in automatic flight mode. An artificial feel system was installed, and altitude and Mach number control was provided for station keeping, approach control, landing and flare-out.

The flight control system included an automatic trim system which had three modes of operation: takeoff and landing, manual and automatic. In the takeoff and landing mode, the trim system was locked at 3 degrees up elevator. In the automatic mode, the automatic trim system provided the elevon deflection required for 1-g flight.  The auto trim system was closely associated with the ratio changer or g-protection system.

There was an aileron-rudder interconnect system that served to cancel out the yawing that normally resulted from aileron deflection. Artificial pitch, roll and yaw damping were supplied to minimize variations in flight characteristics and to provide satisfactory damping. There was also a "wing heavy" control system which sensed lateral accelerations and provided corrective rudder through the rudder damper servo to prevent lateral fuel shift and subsequent wing heaviness.

The Eclipse Pioneer autopilot system provided a constant Mach number by automatic control of the elevators. The aircraft was maintained at the desired altitude and Mach number by automatically controlling the elevators and throttles. The heading was maintained by automatically steering the aircraft along a constant track or a computer ground track.

The main landing gear assemblies consisted of 8-wheel bogies, four wheels on each axle. Between each pair of tires there was a steel non-frangible wheel which was installed after mid-1961 in response to the continual problems that the B-58 had encountered with tire failure during landing and takeoff. The main gear was fitted with a set of multiple disk-type anti-skid brakes. The main gear bogies were attached to large legs which were in turn connected to the underwing attachment points by large 'U'-shaped connecting units. The main gear assemblies retracted into faired wing wells located inboard of the inboard engine pylons and nacelles. In order to provide sufficient space for the retracted wheel units within the thin wing, the wing wells projected significantly above the wing upper surface, being encapsulated under a wedge-shaped aerodynamic enclosure. The gear retraction mechanism was fairly complex, with the main unit retracting backwards into the wing recesses.  During retraction, the main leg folded at the point where it joined the 'U'-shaped connector in order that the whole unit could fit inside the bay. In spite of the complex retraction scheme, main landing gear retraction or extension failures were fairly rare. However, strut, bogie and axle assemblies did occasionally break, resulting in damage to the aircraft and at least one complete write-off.

The nose landing gear consisted of a pair of tires attached to a rather complex strut that retracted into a well covered by a pair of doors. The complexity of the strut was in part due to the fact that the gear had to clear the nose of the ventral pod during extension or retraction. The strut was hinged so that the main strut rotated up and back into the well while providing clearance for the MB-1 pod or TCP. The nose gear was fully steerable from the front cockpit.

The full sequence of landing gear retraction and extension took about 10 seconds.

The deceleration parachute system was housed underneath the rear fuselage just in front of the tail gun installation. It was a 28-foot ring slot parachute assembly housed inside a stowage compartment enclosed by dual clamshell doors. The pilot could jettison the parachute at his discretion.

ARMAMENT

The tail gun consisted of a General Electric T-171E-3 six-barrel 20-mm rotary cannon with a maximum firing rate of 4000 rounds per minute. The gun was mounted on the axis of an articulated cone which consisted of tapered, concentric aluminum rings which were spring-loaded against each other. The tail gun assembly was aerodynamically faired to conform to the rest of the aircraft.

The Emerson MD-7 radar for the tail gun was located in a bullet fairing above the tail cone. The MD-7 radar was a Ku-band unit. Data from the radar was fed to a computer mounted directly behind the gun and was then relayed electromechanically to the gun itself. Mach number and relative air density information were automatically supplied to the fire control system by the air data computer. The gun was aimed remotely by the fire control system in the tail, but there was a radar (automatic) fire control panel and a manual fire control panel located at the DSO's station, and the gun was actually fired by a button located there.

A total of 1200 rounds could be carried. Ammunition was drawn from a box in the fuselage just forward of the turret. The firing zone was any target within a 30-degree cone. During firing, spent ammunition shells were ejected through a ventral door.

The defensive electronic countermeasures system provided an early warning of the presence of enemy radar systems and could be used to deceive, confuse or jam them. The system consisted of AN/ALR-12 radar warning equipment and an AL/ALQ-16 radar track breaking equipment. An AN/ALE-16 chaff dispensing system was installed in each upper main gear fairing, with chaff being ejected through mechanically-actuated slots in the tops of each wing fairing. The AN/ALQ-16 radar track breaker was a repeater type jammer that generated deceptive radar jamming signals as a function of RF energy received from enemy tracking radars. When tracking radar signals were received, the track breaker generated and transmitted deceptive angle and range information, causing the tracking radar servo system to generate false antenna positioning information which in turn caused the tracking radar to compute false range information.

The aircraft was equipped with a Raytheon Ku-band (16-17 GHz) search radar. There was also a daytime-nighttime Lollsman Instruments KS-39 astro-tracker which automatically tracked a preset celestial body via a photocell mounted in a telescope and was so designed that it held the observed body in the center of the field of view.

The navigation/bombing system was a Sperry AN/ASQ-42 system. This system consisted of six major subsystems.

1...An aft-fuselage mounted AN/APN-113 Doppler radar which determined the ground velocity of the aircraft and also calculated airspeed and wind.

2...The heading subsystem including the astro-tracker and the remote compass transmitter which determined the true heading of the aircraft.

3...A navigation subsystem which continuously computed the latitude and longitude of the aircraft.

4...A sighting subsystem which consisted of the nose-mounted search radar and radio altimeter which established the position of the aircraft relative to a given point on the Earth.

5...An indicator subsystem which contained controls used by the navigator and pilot in the operation of the primary navigation system. In addition, it generated the pod release signal during the bombing run.

6...A malfunction system used to detect failures in the primary navigation system while also providing for alternative methods for completing the mission.

The primary navigation systems were provided by Bendix and Motorola.  It was basically a Doppler inertial system using the astro-tracker for a primary heading reference. While en route, the position and course were continually computed by a precise dead reckoning system. Periodic search radar sightings were used to check the accuracy of the dead reckoning, and corrections were made as necessary.

Midway through its operational career, the B-58 was fitted with a voice warning system with a pre-recorded female voice that would inform the crew when an emergency was taking place. Every major event from an engine fire to a hydraulic failure was included in the set of events, and a total of 20 emergencies could be programmed into the system from 50 inputs.

The Magnavox communication system provided a means of crew intercommunication, plus normal and emergency air-to-air and air-to-ground communication. An AN/APX-47 IFF system was installed. Also included were AN/ARN-69 TACAN, AN/ARN-50 VHF navigation equipment, as well as AN/APN-136 and AN/APN-135 beacons.

POD

The MB-1C pod was the standard free-falling unit that was carried underneath the B-58 fuselage on the centerline. It was basically a finned aerodynamic shell for a pair of fuel tanks plus a variable-yield thermonuclear bomb. The pod was 75 feet long with a diameter of about 5 feet. Empty weight was 2500 pounds without the fuel and the warhead, but when fully loaded with fuel and carrying the standard W39Y1-1 warhead it weighed 36,087 pounds. The pod was attached to the aircraft by three hooks. The pod had an equipment bay, a forward fuel tank, a bay for the thermonuclear weapon, an aft fuel tank, a tail cone and fins, plus an attachment pylon. The four fins were mounted at 45 degrees from the horizontal centerline and were slightly offset to give the pod a slow spin during free-fall. The warhead was fused by a set of barometric switches, set to trigger the weapon when the preset pressure was reached. Fuel and fuel pressurization disconnects released and closed instantly when the pod was released.

AN/ASH-15 Indirect Bomb Damage Assessment equipment was installed inside the aircraft to give continuous pod position data to the aircraft after pod release. This data was saved on an in-flight recorder inside the aircraft. At the time of warhead detonation, a photocell in the aircraft recorded the light intensity of the burst, and from the data recorded the yield, pressure altitude of the burst, the pod-to-aircraft range, and azimuth from aircraft could be determined.

Some MB-1C pods were modified to incorporate a Fairchild KA-56 camera in a forward compartment. When so modified, the pod was designated LA-1. The system consisted of the camera and magazine, a scanner and converter, a control panel, and associated air conditioning and electrical systems. The pod camera control panel was located at the navigator's position and replaced the weapons monitor and release panel when the photoreconnaissance pod was installed.

The MB-1C pod had a relatively short service life because there were persistent problems with fuel leakage into the weapon bay. Several years of fighting with this problem lead to the introduction of the two-component pod.

The Two-Component Pod (or TCP) had the same overall profile as the older MB-1 and used the same attachment points. However, it was built in two separate sections. The TCP was actually two pods, an upper BLU 2/B-1 bomb pod and a lower BLU 2/B-2 fuel pod. The warhead unit could be retained while discarding the lower fuel pod.

The 35-foot long upper component contained two fuel tanks, separated by a warhead cavity. Maximum diameter was 3.5 feet. A pylon and three fins were fitted. Two of the fins were mounted on the sides of the pod at 30 degrees above the horizontal, and the third (lower) fin was retracted within the upper pod while the lower pod was still attached. It deployed automatically upon lower pod release. The gross weight of the upper component when equipped with maximum fuel and a Mk. 53 nuclear warhead was 11,970 pounds.

The lower component was also divided into two tanks separated by a common bulkhead. It was supported underneath the upper pod by one forward and one aft releaser. There were no fins, but a pivot strut was mounted on the aft end of the pod to facilitate proper separation from the aircraft during release. The large lower pod was expendable and was released during flight when all the fuel in both the upper and lower components was consumed. The bomb pod remained with the aircraft for release during the delivery run. The empty weight was 1900 pounds, whereas the gross weight was 26,000 pounds.

Both the MB-1 and the TCP remained in the active inventory until the end of the B-58 program. Because of its several advantages, the TCP was the preferred pod, but the MB-1 pod accommodated a much larger warhead.

Midway throughout its career, the B-58 was reconfigured to carry four Mk. 43 thermonuclear weapons on external pylons underneath the wings between the fuselage and the main landing gear bays. Two weapons were carried on either side of the fuselage in tandem. The Mk. 43 weapon was about 12 feet long, 1.5 feet in diameter, weighed about a ton, and had a variable yield of up to 1 megaton.

The MA-1C pod was a proposed rocket-propelled version of the MB-1 free-fall pod, designed to give the B-58 a stand-off capability. The pod was to be powered by a Bell Aerospace LR81-BA-1 rocket engine, fueled by a combination of JP-4 and red fuming nitric acid. The maximum range was expected to be 160 miles. During the flight to target, a maximum altitude of 108,000 feet and a maximum speed of Mach 4 was to be obtained. A Sperry guidance system was to control the pod during its flight to the target. The MA-1C pod was cancelled before it could be deployed.

The MC-1 was a proposed dedicated photoreconnaissance pod. In 1953, the Fairchild Camera and Instrument Company was given a contract to develop a dedicated photo-reconnaissance pod for the B-58. The system could be installed within the standard MB-1C pod, and the only actual airframe modification would be the replacement of the package and bomb system equipment in the second crew station by photo-navigation equipment. The MC-1 was to have a multi-camera system consisting of three 36-inch format cameras installed in a stabilized mount.  A tri-camera system consisting of 3 6-inch cameras, one 3-inch forward oblique camera, a camera control system, a nose-mounted television view finder, a fan of five 3-inch cameras, a Melpar recording system, a Sperry navigation system, and a Raytheon search radar scope camera. However, in early July of 1955, the reconnaissance pod was cancelled due to funding limitations. The program was reinstated in September of 1955, when funding was again made available. However, the program was cancelled for good in early 1958, after only one pod had been completed. This pod was never actually flown under a B-58, although 55-0671 had been scheduled as the testbed aircraft.

The MD-1 was a proposed electronic reconnaissance pod, which used many of the shell components of the standard MB-1 free-fall pod. It was equipped with a wide range of electronic sensors to analyze and record all enemy radar signals that reached the aircraft. Only one example was actually completed, and it was never actually flight tested.

Convair B-58A Hustler Performance Information

Dimensions:

Wing span	56 ft. 10 in (17.32 m)

Length	96 ft. 9 in (29.49 m)

Height	31 ft. 5 in (9.58 m)

Wing Area	1,542 sq ft (470 sq m)

Weights:

Empty:	55,560 lb (25,201 kg)

Max T/O	163,000 lb (73,935 kg)

Performance:

Maximum Speed	1,321 mph (2,125 km/h) @ 63,150 ft (19,248 m)

Combat Ceiling	63,150 ft (19,248 m)

Record Ceiling	85,360 ft (26,017 m)

Combat Radius	1,750 miles (2,816 km) to, 5,577 miles (8,975 km)

with inflight refueling

Powerplant:

Four General Electric J79-GE-1 turbojet engines rated at 10,500 st. or 16,000 st. 69.4 Kn with afterburners.

Armament:

Single General Electric T-171E3 Vulcan 20 mm rotary cannon in the rear, plus nuclear or conventional bombs in the underwing pod.

SST Inertial Navigation System (INS) User Manual

IMPORTANT: This INS upgrade is supplied and used on condition of there being no technical support available for it we will be unable to assist.

# Overview

The inertial Navigation System (INS) is a reliable positioning system used on many civil and military aircraft both before and after the advent of GPS Satellite navigation.  The system modelled here is a representation of the instrument fitted in the Concorde aircraft. It has 25 pre-programmed waypoints for “Round Trip” Transatlantic crossing and can also be user programmed for different flight plans.

# Specifications

The instrument used here does not have the full functionality of the real Concorde instrument but it is does have the following features:

#1 	25 built in waypoints for London to New York and return to London.

#2	25 User defined waypoints.

#3	Save and load of a single custom flightplan.

#4	Auto waypoint increment.

#5 	Automatic selection of ILS frequency and approach Hold.

#6	4 display modes.

#7	Waypoint entry error checking.

#8	Audio Cues.

You can manually enter your own 25 Waypoints and save these for future use by entering the information via the INS keypad just like in the real thing. There is a more detailed section on waypoint entry later in the Manual

Use of the INS with the pre-programmed London-New York-New York-London route

It is recommended that you read all through this section on the INS before using this first flight.

To view the INS firstly reveal the centre console (shift+6 key) then left mouse click any where on any of the three INS keypad entry areas. This will show the full size INS instrument. (To minimise the instrument back down to the centre console, left mouse click on the bottom left minimise area of the maximised INS gauge. To remove the centre console press Shift+6 key again). You do not have to have the INS maximised for this pre-programmed flight to select the hold button, but you have to have the centre console showing. Please note this is not a SID (Standard Instrument Departure) or STAR (Standard Terminal Arrival) INS plan.

Take off from Heathrow on runway 27 left and as soon as the gear is up, activate the INS hold feature (press the INS “hold” button, maximised or minimised INS instrument). You will then fly the waypoints automatically selected by the INS in sequence until you reach New York JFK ILS intercept point (waypoint 13).  Here the INS will select the correct ILS frequency for a 13L approach, and then select the autopilot approach hold as it disconnects.  For the return flight simply select INS HOLD button immediately after takeoff for waypoint 14 and you will follow the track back to London Automatically. At the London Heathrow ILS Intercept point (waypoint 25), the INS will again select the correct frequency for the 09L ILS and switch the autopilot to approach hold before disengaging. Audible cues will be heard as the ILS switches between waypoints and on disengaging. You must fly the altitudes and speeds as instructed in this manual. The INS will only navigate the route. It does not control the altitude or speed.

### A look at the instrument

On the centre console (press the shift key plus the 6 key together to view) will see 3 INS control Panels. These are the normal locations for the Captains INS, Co-pilot INS and Flight Engineers INS. In Normal operation these 3 systems cross check each other for triple redundancy. In this normal view (Figure 1), the readouts are active and are displayed normally in “non maximised view mode” but the only active control is the HOLD button. This allows you to turn the INS hold operation on or off without having to maximise the INS.

## Figure 1

### A 	Concorde centre console and throttle quadrants

B	Captains INS panel

C	Co-pilots INS panel

#### D	Engineers INS panel

Please note these instruments are NOT fully functional when viewed in this non-maximised view mode. You will need to MAXIMISE the INS panel to program or use the different modes. Just click on the centre of the INS entry keypad to maximise.

The following section contains a diagram of the MAXIMISED Instrument showing all it’s features and functions in detail.

### The Maximised INS Panel

# A	Latitude readout

# B	Longitude readout

C	Autopilot hold button

D	Waypoint set mode Button

E	Current waypoint information button

F	Clear button

G	Distance to current waypoint button

H	Bearing to current waypoint button

I	INS programming keypad

J	Enter key

K	Waypoint increment button

L	(Inoperative in this version)

M	Mouse click area to minimise INS

N	(Inoperative in this version)
O	Autopilot hold indicator lamp
P	Current waypoint number indicator. This is the black rectangle in the middle of the INS display picture with the digits 01 in it.

# A	Latitude readout

# B	Longitude readout

The LED readouts shows different information depending on the MODE set by the user. When additional modes are set the display changes to that mode and holds for 6 seconds before returning to Standard Mode. These modes are as follows:

Normal mode- Actual current latitude and longitude shown. (Default display-no buttons pressed)

Current mode-Active waypoint latitude and longitude shown. Display this mode by pressing the CURRENT button (E)

Bearing mode -Current waypoint information shown by the bearing in latitude (left) display and estimated time of arrival (ETA) in longitude (right) display. Display this mode by pressing the BNG button (H)

Distance mode-Current waypoint information shown as distance in Nautical miles in the Latitude display and ground speed (in MPH) in the longitude Display. Display this mode by pressing the DIST button (G)

# C	Autopilot hold button

By pressing the HOLD Button the INS will command the aircraft autopilot into waypoint tracking mode. It uses the autopilot heading hold with bearing information fed to the autopilot via the INS System. On disengaging the HOLD feature the INS will sound a disconnect chime and the autopilot will remain in HDG HOLD mode until you deselect it.

D	Waypoint set mode button

Pressing the SET Button puts the INS into PROGRAM MODE. (See Programming guide later in the manual for full description.)

E	Current waypoint information button

See A+B Above

F	Clear button

Used to “Clear” an error in programming waypoints. (See programming guide later in the manual for full description.)

# G	Distance to current waypoint button See A+B Above

H	Bearing to current waypoint button

See A+B Above

I	INS programming keypad

Enter waypoint data via this keypad for your custom flight plans (See programming guide later in the manual for a full description.)

J	Enter key

Used to “Enter” the waypoint data for your custom flight plans (See programming guide later in the manual for a full description.)

K	Waypoint increment button

Pressing the INC Button steps the INS through the waypoint memory bank. If the INS is in hold mode the INS will send the “selected” waypoint data to the autopilot so you can skip waypoints. You will need to step through from waypoint #1 to waypoint#14 for example if Flying from JFK to London using the built in route information, as waypoints #1 - #13 are the London – JFK leg and #14 – 25 are for the return leg.

L	Inoperative

M	Minimise Area

When you have finished with the maximised Panel you can switch it back to the normal, non-maximised size by left mouse clicking in this area. The main INS panel will then minimise back to the centre console where you will still see the readouts and can activate the HOLD Button only.  To maximise the panel again, simply click anywhere on a “keypad area” on any of the 3 minimised INS panels

N	Inoperative

# O	Autopilot hold indicator lamp

This small green LED will illuminate when you have INS HOLD selected and the system is operating via the autopilot.

Note: In MAXIMISED MODE The INS Panel is fully featured as above but when MINIMISED only the CURRENT LAT LONG display is available as well as the HOLD BUTTON. No other features are available in Minimised mode.

INS Programming guide

If you wish to program your own flight plan routes the following section will explain the programming procedure of the INS System.

### To programme a waypoint

Press the SET BUTTON (D), this puts the INS in programme mode to allow you to enter the data.

Decide which waypoint you wish to re-program by selecting it using the INC BUTTON (K). You can programme any or all of the waypoints at any time. The LAT LON Display will blink with the currently programmed waypoint information.

You are now ready to enter the Waypoint co-ordinates.

### KEYPRESS SEQUENCE

It is important to enter the data in the correct sequence. For example to enter a waypoint at the location 51'17.28 N   3'09.50 W you must use the following sequence

[N] [0] [5] [1] [1] [7] [2] [8] [W] [0] [0] [3] [0] [9] [5] [0] [ENTER]

The INS numeric programming keypad (I) has the Letters N-S-E-W on the keys 2, 8, 6 and 4. Use these keys to enter the correct hemisphere information as part of the sequence

Please note the leading ZEROS as the INS expects a 3 figure “degrees” entry, so a figure of  22N  (22 degrees north) has to be entered as N022

If you make an error in the data entry you can press the CLR button (F) to “backspace” the entry. You can “backspace” as many times as required.  On pressing the ENTER key (J) The waypoint will be read into memory and become available for use. Repeat the above for up to a maximum of 25 waypoints.

SAVING AND LOADING YOUR CUSTOM FLIGHTPLAN

The INS doesn't keep your programmed waypoints between sessions unless you have saved it. (You can only save ONE flight plan at any one time above the included pre programmed London New York plan) However, if you programme your custom flightplan, you can save it and load back next time. Note: Saving another flight plan over writes the previously saved item.

To save your custom flightplan, press [SET][8][ENTER]. To load it back, press [SET][2][ENTER].

### RESETTING THE DEFAULT ROUTE

You can reset the default London Heathrow– John F Kennedy – London Heathrow flightplan at any time by pressing  [SET]  [ENTER]. This will load all original waypoints and reset the INS to waypoint #1

Important INS information

1) All manually programmed waypoint and the active HOLD waypoint are LOST when Flight Simulator is switched to full screen mode or back while in flight. The same happens when you select a different Concorde. If you have to switch, save your custom flightplan (if you programmed it) first, then change screen mode or aircraft then if it is a custom flightplan load it and use the WPT INC button to select the correct next waypoint and then engage the INS HOLD button. If you are using the built in London-New York-London flightplan, simply use the WPT INC button to go to the correct next waypoint and select the HOLD button. It is best to decide which mode to stay in, either normal or full screen mode and not to change during the flight. Select your mode choice before starting your flight and to use it until the end of your flight.

2) When you switch to another active application, and switch back to Flight Simulator, you may hear some of sounds the INS produces.

3) You can only save one extra flightplan above the pre-programmed item and it may only have maximum if 25 waypoints in it.

4) Do not use CTRL+F for the FMS system when using the INS. Either use the FMS of the INS do not mix them on the same flight.

5) You must have the Microsoft FS98 patch (fs98pat1.exe) loaded for the INS to function properly. If you suffer a lack of INS heading control then you do not have the patch loaded. To load the patch, uninstall Concorde as described in the manual and reinstall Concorde and select the Flight Shop converter and Patch install option this time around.

FS98 GPS gauge V2.5

This is a basic GPS gauge that displays heading (deg), distance (nm), and time (h:m) to a destination. A needle indicator at the top of the gauge displays the direction of the destination relative to the aircraft heading. The destination is set by clicking the small gray SET button at the bottom of the gauge. Longitude and latitude in the form of deg min sec are entered in the same way as radio frequencies, by clicking on the digits. N,S,E, and W are also changed by clicking on the letters. Once the destination is set,clicking the SET button again returns the display to normal operation.

The AP button turns on the autopilot and the heading hold function, and sets the autopilot heading to the heading of the destination. As long as the AP button is on (green), the autopilot heading will remain locked to the GPS heading, and the GPS display will change every 10 sec. between distance and time to the destination. Clicking the AP button off will disable the autopilot heading hold function and break the link to the autopilot, allowing it to be set manually. The AP button can not be changed from the setting display. If any changes are made to the GPSsettings while the AP button is on, the autopilot link will be broken when the display returns to normal, and the autopliot settings will remain unchanged.

The PWR button turns the power to the gauge on or off. Turning the power off when the AP button is on leaves the autopilot settings unchanged but breaks the link to the autopilot.

The heading displayed on the gauge defaults to true rather than magnetic heading, although the heading sent to the autopilot is always magnetic.

The GPS can be configured to display magnetic heading by clicking TRU onthe setting screen and toggling it to read MAG.

License

This software is released as freeware.  It may not be distributed by anymeans that requires users to pay for it without permission from the author.

Disclaimer

This software is released as is.  Installation is at your own risk. The author is not responsible for any damage resulting from the installation of this software.

Feedback/bug reports

Feedback and bug reports are welcomed and desired.  If you have any comments about this software, please feel free to drop me a line.

David Markovitch

davidm@terracom.net

NOTES