# Lancair Legacy 2000 for Flight Sim 2002 Pro

v 1.1

- Author: Robert Christopher rchris@flash.net

Copyright: February 2003

______________________________________________________________________________________

Thank you for downloading the Lancair Legacy 2000. This project has been a labor of love, and I hope you enjoy it as much as I have.

Features: Full compliment of moving aerodynamic surfaces, Fowler flaps, detailed retractable landing gear, steerable nose gear, rotating tires, flexing suspension, speed brakes, landing lights, detailed virtual cockpit interior with illuminated gauges, as well as reflective exterior surfaces with specular highlights.

- Flight dynamics: Engine portion - Bob Samuelson, aerodynamic portion - Robert Christopher. Rev’s to follow.

Installation:

1) UnZip Legacy.zip file to the Flight Sim 2002 main directory. Sub folders will be created as required, which include Gauge, Effect, and DLL folders.

2) Copy *.gau files to the main Gauge folder, and *.fx file to the Effects folder. The Gaugesound.dll must be placed in the root FS2002 directory.

3) Enjoy!

Data:

Manufacturer		Neico Aviation

Length 			22.3 feet

Wing Span		25.3 feet

Gross Weight		2200 lb.

Engine			6 cyl. Continental IO-550-N 310 BHP

Acceleration Limits	+4.4 and –2.2g at gross weight

# Airspeeds – IAS:

Never Exceed, Vne	274K

Maneuvering, Va		158K

Best Rate of Climb, Vx	135K

Best Angle, Vy		100K

Stall, Clean, Vs		68K

Stall, Dirty, Vso		56K

Flap Extension, Vf	111K

Gear Operation, Vge	140K

Operations Info:

1) Steering - Differential Brakes (a little power helps also) .

2) Take Off - Typically 1 notch of flaps - Trim (5) check the window above

the trim wheel in 2D - Rotate at 70K - should come off at 80-85K.

3) Climb Out - Retract gear - retract flap - gently lower the nose - trim

accordingly.

4) Cruise - Should be around 220 to 240K - depending on altitude, temp, etc.

5) Prep for Landing - Apply speed brake to slow - gear at first white arc,

flaps at second white arc

7) Pattern - Around 120-130K

8) Landing - With brake, gear and full flaps typical speed on final is around

100-110K, short final at 90K(power as necessary), and touch down around 80K.

- In the VC if you put the top of the left cowling cheek on the threshold lights

and hold 500 fpm decent, she comes in very nicely. Typical flare does

obscure the runway just before touchdown. Gently lower the nose and apply

even brakes.

- VC Lights - There are two small switches on the 2D panel next to the landing

gear light for panel (right) and gauge (left) lights at night. The landing light is

activated by the "Taxi Light" switch.

- Sorry, nothing opens such as canopy, etc. I did not want to do any further

cutting on the fuse trying to keep the smooth curves.

VC Gauges - To get the smoothest performance possible while in the VC,

gauges are mounted on individual planes. This in turn slows the update of

the gauges.

- There is also a Compass (Window2), 2D engine gauge (Window 3), and Coyote

Garmin GPS 195 (Window 4).

- Your comments would be appreciated. I can be contacted at rchris@flash.net.

- Updates will be released as available.

Legal Notice:

* This package is for freeware use only, and may not be included in any commercial product, or website without express written permission from the author. No portion of this package may be used without prior written permission from the author.

* I accept no liability for any effect this package may have on your system.