ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.1.0 GENERAL

The following sections provide expanded information

related to the normal procedures.

NOTE: Deviation from Norm Procedures will be published in

Airport Briefing Sheets and must be explicitely briefed

prior each flight.

The normal procedures consist of inspections,

preparations and normal checklists, All items are listed

in a sequence following a standardized scan of the

cockpit panels, except when required by the logic of

actions priority,   to ensure that all actions are

performed the meet efficient way.

The following assignations are used:

CM 1     refers to crew member one, left hand seat

CH 2     refers to crew member two, right hand seat

PF       	refers to pilot flying

PNF     	refers to pilot not flying

After completion of a given procedure, the respective

normal checklist is used. The normal checklist only

includes the items that may have a direct impact on

safety and efficiency in case of not being correctly

accomplished. All normal checks are initiated by PF

command. On ground by CM 1 command. If corresponding

checks are not requested in due time PNF while in flight,

CM2 on the ground, will offer the checks.

Some normal procedures which are non-routine are to be

found in FCOM chapter 2.02., Procedures and Techniques,

and 3.11., Special Operations.

All steps of the BEFORE FIRST TAKE OFF checklist must be

performed prior to each first flight of the day or

following a crew change or maintenance action. Transit

steps are the only ones to be completed after a transit

stop. They are grouped in the TURN AROUND checklist.

If there is any doubt whether the application of transit

procedures is covering all safety aspects, the complete

preparation has to be applied.

After a crew change during a short transit stop,TURN

AROUND checklist may be used, (refer 3.4.5 page 25)

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.2.0. FLIGHT PREPERATION

(Check also FOM Chapter 5.1.2)

03.2.1.  TECHNICAL CONDITION OF THE AIRCRAFT

The crew has to verify the technical status of the

aircraft ( HIL ) in respect to airworthiness, acceptabi-

lity of malfunctions ( MEL ) and possible influences to

the flight plan.

03.2.2. WEATHER BRIEFING

The crew receives weather briefing.

The briefing includes:

- The actual and expected weather conditions for the take

off and climb out including the runway conditions

- The enroute significant weather conditions, winds and

temperatures; the terminal forecasts for the destina-

tion and alternate airports

- The actual weather for the destination and alternates

and the recent past weather if available

- A survey of the meteorological conditions at the air-

ports along the planned route

03.2.3.  NAV/COM FACILITIES EN ROUTE

The crew studies the latest relevant NOTAMS and checks

that all required facilities at the departure, destina-

tion and alternate airports are operational and that they

fulfill the appropiate requirements.

03.2.4. FLIGHT PLAN AND OPERATIONAL REQUIREMENTS

The crew checks the company flight plan, in respect to

the routing, altitudes and flight time

The crew checks the estimated load figures and calculates

the max. allowed take off and landing weights.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.2.4.  cont.'d

The captain decides how much fuel will be necessary for

the safe conduct of the flight, taking into consideration

possible economic fuel tankering.

The captain checks the ATC flight plan and ensures that

it is filed according to the prescribed procedures.

03.3.0.  EXTERIOR INSPECTION

The exterior inspection is primarily a visual check to

ensure that the overall condition of the airplane and the

visible components and equipments are safe for the

flight.

It is performed by the captain according to the

manufacturers checklist before each orginating flight.

NOTE 1: During exterior walk around, observe that the flight

control surfaces and flaps are clear and memorize

position.

NOTE  2: On ground, the landing gear uplock box in closed

position leads to the red local UNLK alarm in the

cockpit. The uplock box can be opened by pulling the

landing gear emergency extension handle. Then, replace it

in its initial position.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.0. NORMAL CHECKLIST

03.4.1. CHECKLIST PHILOSOPHY:

The NORMAL CHECKLIST is a "Follow Up" checklist.

That means, all items are performed according to the task

sharing and a certain set-up prior to reading the respec-

tive checklist.

Therefore each check consists of two parts:

1.- a certain set up and/or specific items already done

according to the task sharing

2.- the appropriate checklist

Instrument readings, fuel quantity and position indica-

tions have to be confirmed aloud with the corresponding

value/position after the respective item in the respec-

tive checklist has been called out.

On ground all checks are requested by CM1 and performed

by CM2.

In flight all checks are requested by PF and performed by

PNF.

One open item per checklist (e.g. AFTER TAKE OFF

CHECKLIST) is permissable, unless checklist (e.g, ENGINE

START CHECKLIST) is divided by a line.

In this case the report will be: "ONE OPEN ITEM..."

(to be specified).

After completion of this open item, checklist will be

reported, "... CHECKLIST COMPLETED".

03.4.2.  CHECKLIST READING:

After the relevant checks have been performed, PNF (on

ground CM2) offers and reads the checklist. On the right

side of each checklist it is indicated who is performing

the certain item (not reading the checklist) CM1, CM2, PF

or PNF. "Both" indicate either both crewmembers are

simultaneously involved, although with different actions/

responsibilities (e.g. engine #2 start) or perform

duplicate actions (e.g. altimeters set/check). To reduce

the cockpit workload there are several "Silent" items

marked by the letter "S".

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.1.  cont'd

e.g. 4. Cockpit Lights.........................set.....................CM2 S

All other items are to be read aloud.

e.g. 6. Overboard valve......................auto ................... CM2

important items are printed in BOLD FACE letters and will

be checked according to the "Challenge Response"

principle requiring a specific answer after the challenge

and subsequent confirmation.

eg. 1. FUEL QUANTITY/FU ...........___/___.................... BOTH

CM2 (because on ground) :     "FUEL QUANTITY/FU"

CM1                               :     "CHECKED 2000 KG/ FU 0"

CM2                               :     "CONFIRMED"

03.4.3. BATTERY START

The following procedure is performed before the powering

of the airplane either by maintenance or the cockpit

crew, as appropriate, This is to ensure that there is no

danger to the airplane and personnel when powering the

systems.

Operation without external power is described in the

BATTERY START checklist. It assumes ENG 2 starting in

hotel mode and establishment of electrical and pneumatic

power. Thereafter continue with the BEFORE FIRST TAKE OFF

checklist.

Operation with a ground power unit is described in the

BEFORE FIRST TAKE OFF checklist.

NOTE: Due to the fact that a battery start is non-routine

within our company, BATTERY START checklist will be

used as "Do List".

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.3.  cont'd

Procedure:

CM1 announces: "BATTERY START CHECKLIST"

CM2 reads the checklist:

1. Battery switch.................on........................... CM1

- SET BAT toggle sw to ON

AT72 	* check MFC 1A and MFC 2A FAULT lights flash

* 3 sec after BAT toggle sw activation check:

MFC 1A and 2A FAULT light extinguish and MFC

1B and 2B FAULT light flash

* 6 sec after BAT toggle sw activation check:

MFC 1B and 2B FAULT light extinguish

NOTE: If cargo door control panel is open prior to set

battery toggle svitch to ON MFC Module 1A sad 2A

have been tested automatically. In this case test

sequence will start with MFC 1B and 2B.

- Check EMER BUS and ESS BUS EMERGENCY SUPPLY

INDICATION: arrows illuminated

- Check UNDV It extinguished

2. AT 72 MFC.................tested/on...................CM1

- PB norm and no FAULT LT indication

3. Circuit Breakers.......... checked................... CM1

- Check on lateral and overhead C/B panels that all

C/Bs are set

- reset as applicable

4. Parking Brake/Press......set/checked.............CM1

- Set handle to PARKING

- Check BRAKE ACCU pressure

- Use AUX HYD PUMP if necessary

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.3. cont'd

5. Gust Lock.......................... engaged............CM1

- Check gust lock engaged

6. Engine Levers.................... set GI/FSO........ CM1

- Check both PL on GI, both CL on FS0 and 	AT 42 ECU on

AT 72 EEC on

7. Landing gear......................down/3 green........CM1

- Check control lever down and three green Its illumi-

nate on the center panel

8. Engine Fire ...................... tested 2...............CM1

- Check ENC 2 fire handle IN, latched and safety wired

- Extinguish any white It

- Deprass SQUIB TEST pb and check both AGENT SQUIB It

illuminate

- Select TEST sw on FIRE and check:

* ENG FIRE red It illuminates in associated fire

handle

* CCAS is activated (CRC+MW It flashing red + ENG 2

FIRE red It on CAP)

- Open the CL to FTR and check the red It illuminates

in the CL

- Select TEST sw On FAULT and check:

- both LOOP A and B FAULT Its illuminate

NOTE: On BAT, only the LH loudspeaker will generate aural

alerts.

9. Prop Brake................... on.................... .... CM1

- Check PROP BRK blue lts ON on prop brake control

panel and memo panel

- If not, to engage: 	- push the AUX PUMP pb

- check READY It illuminates

- select PROP BRK sw to ON

- check UNLK It illuminates then

extinguishes

- check PROP BRK illuminates

both on prop brake control

panel and memo panel

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.3. cont'd

10. Fuel Pump.....................on................CM1

- Check RUN It illuminates, FEED LO PR It extinguishes

- Check LP VALVE in line

11. Engine 2 start In H-Mode .......perform......CM1

- For engine start refer to chapter 5 Procedures and

Techniques

CAUTION: Do not use ENG 2 in H-Mode without a qualified person

( flight crew or maintenance ) in the cockpit.

Do not use H-Mode when tailwind componant exeeds

AT 42 - 15 kts

AT 72 - 10 kts (gusts included)

NOTE1 : Without nacelle overheat detection installed the

tailwind component is limited to 10 kts on AT 42.

NOTE 2:  AT72 : On H-Mode,it is recommended to use only one

pack at a time to save engine life.

Procedure:

CM2 reports: "BATTERY START CHECKLIST COMPLETED"

03.4.4.  BEFORE FIRST TAKE OFF

Procedure:

CM1 enters the cockpit and starts procedure:

first of all he checks:  	- TechLog

- Gear Lever down

- Parking Brake set

- Use Aux Hyd pump to pressurize

Brake Accu

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4  cont´d

1. Battery switch

- Set BAT toggle sw to on

AT 72 	* check MFC 1A and MFC 2A FAULT lights flash

* 3 sec after BAT toggle sw activation check:

MFC 1A and 2A FAULT extinguish and MFC

1B and 2B FAULT light flash

6 sec after BAT toggle sw activation check:

MFC 1B and 2B FAULT light extinguish

NOTE  If cargodoor- control panel is open MFC 1A and 2A do

not test

Check EMER BUS and ESS BUS EMERGENCY SUPPLY

INDICATLON:  arrows illuminated

Check UNDV It extinguished

2.  AT 72 MFC

Check PB norm and no FAULT LT indication

3. External Power

- Check DC EXT PWR available and GREEN LIGHT ON

- Push EXT PWR pb ON

CM1 select NAV light on and checks together with

the CA the PA, INT and Capt. Call and switches COM 1

to ON,

CM1 announces: "BEFORE FIRST TAKE OFF CHECKLIST DOWN

TO THE LINE."

CM1 performs the exterior walk around check according

to the manufacturers exterior inspection checklist.

CM2 performs the cockpit checks and set up by memory as

described below. Thereafter he will confirm each item

according to the BEFORE FIRST TAKE OFF checklist.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

4. Circuit Breakers

- Check on lateral and overhead C/B Panels that all

C/Bs are set; reset as applicable

5. Cockpit Lights

- Check the Its are operative and set as required.

NOTE: Panel lts need ACW

6. Flt Data/CVRl

- Check the FDAU time base; adjust when necessary.

- Check the statuslts: SYST amber, FDAU off.

- Enter the flight number on the data entry counters.

- Depress RCDR pb, blue It comes on.(not applicable

when the A/C is on its own PWR supply)

- Depress the CVR TEST pb; the pointer moves to a

location between 8 and 10.

7. Trims

- Check ROLL and YAW TRIM operation

* ROLL : During 4 sec hold forward (/aft) aileron trim

control sw in left wing down position and then in

right wing down position whilst checking that the

indicator reading does not vary.

Hold the two aileron control switches in left wing

down (/right wing down) position whilst checking that

indicator reading follows selected movement.

Return to neutral position.

* YAW : During 4 sec, hold upper (/lower) rudder trim

control sw in nose RH then nose LH position whilst

checking that indicator reading does not vary,

Hold the two rudder trim control switches in nose LH

(/RH) position whilst checking that indicator reading

follows selected movement.

Return to neutral position.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

* PITCH: Operate normal elevator trim rocker switches

on pilot control column one after the other in both

directions, No trim function is noted

Operate rocker switches simultaneously in both

directions. Indicator shows correct trim deflection;

do the same operation for copilot control switches.

* STBY PITCH TRIM: Check stby pitch trim operation;

check sw guarded in OFF position.

* Adjust pitch trim

AT 42:  0°

AT 72:  1° up.

8. Engine Test

- For both engines:

turn the ATPCS to ARM,

the ARM green It illuminates,

turn the ATPCS to ENG position,

check the associated ENG UPTRIM It illuminates,

2.15 sec later check ARM It extinguished.

9. COM/ADF XPDR

- Check the COM 1 and 2 transmission and reception.

- Select ADF on ADF 1 and 2, check indication

- Select XPDR to STBY.

NOTE: VHF 2 operation maybe affected on ground due to

antenna location (shadow area).

10. Emerg Audio Cancel

- Check the sw guarded and safetied.

11. Idle Gate

- Check the It extinguished and amber band visible on

the lever.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

12. Engine Levers

- Both PL on GI

- Both CL on FSO

13. Parking Brake/Press

- Set handle to PARKING.

- Check BRAKE ACCU pressure 3000 Psi.

- Use AUX HYD PUMP if necessary.

14. Gust Lock

- Check the gust lock engaged.

15. Flaps

- check/set the flaps lever to 0° and check flaps

position.

16. EFIS/WX Radar

- Select EHSI ON - check composite mode.

- Select EADI ON - check normal display,

- Select BRGs as required

- Select WX DIM approx. 3 o'clock.

- Select WX RDR to STBY

17. TAS/TAT

- Check realistic values are displayed.

- Push the SAT pb to check SAT indication.

- Crosscheck the TAT/SAT value with ATIS

18. Crew Alerting Panel

Check that - AT 42: CCAS FAULT It

- AT 72: MFC FAULT It is extinguished.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

19. PWR Management

- Check rotary selector on T/O.

20. Engine Instruments

Check: Oil Press 0

OIL TEMP realistic indication

FUEL TEMP realistic indication

FF 0, FU 0.

NH 0

ITT realistic indication

NP 0

TQ 0,  target bug realistic indication

(crosscheck with ENG 2),

21. Engine Control

- Extinguish any white light

- Check the UPTRIM, LO PITCH lts are out

- ATPCS pb in.

22. Landing Gear

- Check the Lever is down

- Check for norrnal indication (3 green), crosscheck

with OVHD panel

- Check all ANTI SKID FAULT Its are extinguished.

23. Pressure Control

- Check all pb Its are extinguished

- Check MAN RATE knob NORM.

- Check the cabin press indication:

DIFF 0

RATE 0

ALT pressure altitude

24. LDG Elevation

- Set to destination field elevation, rounded up to

the next 100

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont´d

25. All Test P/B

- Press all TEST pb on pedestal and center panel:

T/O CONFIG, EFIS, PRESS, BRAKE (if installed), ANTI

SKID, LDG GEAR (if installed), FUEL and GPWS.

26. FD Switches

- Check both switches in FD BARS position.

27. NAVs

- Select both NAV on and check correct VOR/ILS

indication,

28. ANN Lights

- Set the ANN LT sw to TEST.

- Check all lt illuminate

-  Return to DIM or BRT as reguired.

29. Air Bleed/Temp Sel

- Push to extinguish any white It.

- Check the COMPT and DUCT ind. show realistic values

with COMPT SEL on FLT COMPT and CABIN,

- Set COMPT TEMP selector to 11 o'clock position and

thereafter according environmental condition

30. Avionic Vent

- Check in AUTO and guarded.

31. Main Oxygen

Check HP indicates between 1400 and 2025 PSI.

NOTE: If pressure is below 1400 PSI, check oxygen duration

chart in FCOM 2.01.05. to determine that the

quantity is sufficient for the scheduled flight.

- Select MAIN SUPPLY ON: check pb It extinguished.

- Check PAX SUPPLY off and guarded,

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

32. Fans

- Check in NORM position.

33. Engine Fire

- Check ENG 2 fire handle IN, Latched and safety wired.

- Extinguish any white It.

- Depress SQUIB TEST pb and check both AGENT SQUIB It

illuminate.

- Select TEST sw on FIRE and check:

* ENG FIRE red It illuminates into associated fire

handle.

* CCAS is activated (CRC+MW It flashing red + ENG 2

FIRE red It on CAP).

- Open the CL to FTR and check the red It illuminates

in the CL.

-Select TEST sw on FAULT and check:

both LOOP A and B FAULT Its illuminate and SC + MC

flashing amber , LOOP FAULT It on CAP.

- Same procedure for ENG 1.

34. Cabin signs

- Set both sw ON.

- Check ON indication on MEMO panel.

35. Ernerg Exit Lights

- Check toggle sw at DISARM position.

- Check all Its extinguished (except AIR BLEED LO

PRESS).

37. Probes/Windshield Heating

- Extinguish any white it, but leave STBY PROBE HTG

off.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont´d

38. ACW/Hydraulic

- Extinguish any white It.

- Check the BLUE and GREEN PUMP LO PR lt illuminated

and other It extinguished.

39. Emerg Loc Xmtr

- Check toggle sw AUTO and guarded.

40. External Lights

- cheok all lights off except NAV (+ LOGO at night).

41. Prop Brake

- Check PROP BRK blue its ON on prop brake control

panel and memo panel.

- If not, to engage: * push the AUX PUMP pb

* check READY It illuminates

* select PROP BRK sw to ON

* check UNLK illuminates then

extinguish

* check PROP BRK illuminates both

on prop brake control panel and

memo panel.

42. Eng Start Selector

- Check in OFF position,

43. Min cabin Light

Select ON.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4 cont´d

44. Fuel Panel

- Select the PUMPs ON and check:

RUN It illuminates

FEED LO PR It extinguishes

LP VALVE in line

X FEED valve X line.

Select the PUMPs OFF,

45. Doors Panel

- Depress SW TEST and check CAB OK and SVCE OK It

illuminated provided associated doors are open,

- Check DOORs It as required.

46. AT 72 TLU switch

- Check TLU sw AUTO and guarded,

- Check amber FAULT lt extinguished.

47. RH Maintenance Panel

- Check panel closed.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4 cont'd

Procedure:

CM2 reads the "Before First Take Off Checklist" down

to the line

1. Battery switch .................................. on.................CM1 S

2· AT 72 MFC....................................... tested on.......CM1 S

3. External Power..................................on..................CM1 S

4. Circuit Breakers.................................checked.........CM2 S

5. Cockpit Lights....................................set................CM2 S

6. Flt Data/CVR .................................... set/on...........CM2 S

7· Trims................................................ checked........CM2 S

8. Engine Test........................................performed......CM2 S

9. COM/ADF XPDR............................ ...on/stby..........CM2 S

10. Emerg Audio Cancel.........................guarded..........CM2 S

11.Idle Gate..........................................pulled............CM2 S

12. Engine Levers..................................set GI/FSO.....CM2 S

13. Parking Brake/Press........................set/checked....CM2 S

14. Gust Lock.......................................engaged..........CM2 S

15. Flaps .............................................checked 0.......CM2 S

16. EFIS/WX Radar................................on/stby...........CM2 S

17. TAS/TAT ....................................... checked..........CM2 S

18. Crew Alerting Panel ........................ checked.........CM2 S

19. PWR Management . ....................... set T/O...........CM2 S

20. Engine Instruments......................... checked..........CM2 S

21. Engine Control.................................set.................CM2 S

22. Landing Gear ................................. down/6 green...CM2 S

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4 cont'd

23. Pressure Control....................................checked/set........CM2 S

24. LDG Elevation........................................set.....................CM2 S

25. All Test P/B...........................................tested.................CM2 S

26. FD Switches..........................................on.......................CM2 S

27. NAVS....................................................on.......................CM2  S

28. ANN Lights.............................................tested.................CM2  S

29. Air Bleed/Temp Sel.................................set......................CM2 S

30. Avionic Vent...........................................auto....................CM2 S

31. Main Oxygen..........................................on/checked........ CM2 S

32. Fans......................................................normal................CM2  S

33. Engine Fire.............................................tested.................CM2 S

34. Cabin Signs ...........................................on.......................CM2 S

35. Emerg Exit Lights...................................disarmed.............CM2 S

36. Anti/De Icing...........................................set......................CM2 S

37. Probes/Windshield Heating ......................set.....................CM2  S

38. ACW/Hydraulic .......................................set.....................CM2 S

39. Emerg Loc Xmtr.......................................auto/guarded ......CM2 S

40. External Lights.........................................set.....................CM2 S

41. Prop Brake..............................................on.......................CM2 S

42. Engine Start Selector...............................off........................CM2 S

43. Min CabinLight.........................................on.......................CM2 S

44. Fuel   Panel ............................................tested.................CM2  S

45. Doors  Panel ..........................................checked..............CM2  S

46. AT 72 TLU switch.................................... auto/guarded .......CM2 S

47. RH Maintenance Panel............................ closed ................CM2 S

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

Procedure:

CM1 performs the exterior walk around check acc. to

the manufacturers exterior inspection checklist.

When CM1 is back from the exterior inspection he

announces: "PREFLIGHT INSPECTION COMPLETED"

CM2 announces: "BEFORE FIRST TAKE OFF CHECKLIST DOWN TO

THE LINE COMPLETED"

CM1 and CM2 are checking together

- that all manuals and documents for the intended

flight are on board:

AFM, FCOM 1+2, MEL, official Documents, FOM, FSM,

FOD, Normal and A+E Checklists, NL Speed Booklet,

Limiting Speed Chart,  Blanc Flight Logs, Load

Sheets, Aircraft and Technical LOG,

- the serviceability of the A/C (acc, HIL and MEL if

applicable) .

CM1 and CM2 are checking together the emergency equip-

ment:

- Exit hatch closed, handle locked and lockwired.

- Escape rope stowed,

- Oxygen masks (observer,CM1,CM2) stowed.

- Flashlights operating and stowed,

- Smoke goggles stowed,

- Portable fire extinguisher lockwired, and pressure in

the green area.

- Axe stowed.

- Life jackets stowed,

- Gear pins on board and stowed.

- Protective gloves.

- L/G emergency extension handle stowed and cover

closed.

- CA cabin emergency equipment check completed and

signed.

CM1 announces:"STICK PUGHER TEST"

- CM1 performs the test,

- Disengage gust lock.

- Set rotary selector to:

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont´d

AT42 - STICK PUSHER YES:

Stall cricket and shaker ON.

After 10 sec. CHAN Its 1+2 ON.

Stick pusher activated and both green STICK

PUSHER Its ON.

STICK PUSHER FAULT:

Stick pusher no more active,

After 4 sec: MC and FLT CONTROL on CAP, STICK

PUSHER FAULT amber lt ON.

STICK PUSHER NO:

FAULT alarm extinguishes.

Stall cricket and shaker activated

only when ICING AOA It illuminated.

AT72 -STICK PUSHER YES :

Stall cricket and shaker ON.

After 10 sec, CHAN Its 1+2 ON,

Stick pusher activated and both green STICK

PUSHER Its ON.

STICK PUSHER TEST 1 :

Stick pusher is not activated.

LH stick shaker only is activated.

Stall cricket,

CHAN 1+2 illuminated,

MC and FLT CONTROL on.

CAP, STICK PUSHER FAULT amber It ON.

STICK PUSHER TEST 2:

Stick pusher is not activated.

RH stick shaker only is activated,

Stall cricket.

CHAN 1+2 illuminated.

MC and FLT CONTROL on.

CAP, STICK PUSHER FAULT amber it ON.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.41 cont'd

STICK PUSHER TEST 3:

Stick shaker is activated.

CHAN 1+2 It illuminated.

Stall cricket.

FAULT It extinguished,

Stick pusher is activated only if

ICING AOA It is illuminated.

CM2 observes the test

NOTE: CM2 snail not hold the control column during the test

as this may lead to a pitch disconnect

When STICK PUSHER TEST is completed.

CM1 announces: "STICK PUSHER TEST COMPLETED,

ROTARY SELECTOR NORM FLIGHT"

CM1 and CM2 are checking their lateral consoles.

CMI checks NW STEERING sw guarded in ON position,

CM1 checks the GPWS

- sw guarded in NORM position.

- GPWS FAULT is not illuminated on CAP.

- Depress the GPWS/GS it, observe the GS and GPWS

warning Its and audio according to the test sequence

(FCOM 1.15.40).

CM1 and CM2 check the OXY Masks:

- Set audio control panel INT/RAD selector to INT, open

volume.

- Push down and hold RESET/TEST pb and squeeze side

levers to inflate the harness.

- Observe blinker momentarily yellow,

- Check oxygen flow sounds through the loudspeaker.

- Release side levers and RESET/TEST pb.

- Check oxy LO PR It is not lit.

- Set N/100% rocker lever to 100 %.

CM2 checks together with CM1 AUDIO SEL, the CAPT/FO

SWITCHING and the FLIGHT INSTRUMENTs.

- Check for proper function on both sides.

- Check override function on CM1 side.

- Check ATT/HDG,VOR/ILS and SGU It extinguished.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.4. cont'd

- Crosscheck RMI/EHSI heading information and absence

of flags.

- Check VSI no flag and pointer shows zero.

- Check EADI no flags and cautions, horizon normal.

- Check ALTIMETER no flag.

- Check ASI no flag, pointer indicates 0 andVMO pointer

indicates 250 kt.

CM1 announces:"BEFORE FIRST TAKE OFF CHECKLIST

TO THE END".

CM2 reads the checklist.

48. DOCUMENTS...........................CHECKED/ON BOARD.........BOTH

49. PREFLIGHT INSTECTION.........COMPLETED.......................CM1

50. EMERGENCY EQUIPMENT.......CHECKED.............................BOTH

51. GEAR PINS..............................STOWED.............................BOTH

52.STICK PUSHER TEST...............PERFORMED........................CM1

53. LH MAINTENANCE PANEL.......NORMAL FLIGHT..................CM1

54. NOSEWHEEL STEERING..........ON/GUARDED.......................CM1

55. OXYQEN MASKS.....................CHECKED/SET 100................BOTH

56. CAPT/FO SWITCHING..............PERFORMED/NORMAL.........BOTH

57. GPWS SWITCH........................NORMAL/GUARDED.............CM1

58. AUDIO SELECT.........................NORMAL..............................BOTH

59. CLOCKS...................................SET......................................BOTH

60. FLIGHT INSTRUMENTS..............CHECKED............................BOTH

Procedure:

CM2 reports:"BEFORE FIRST TAKE OFF CHECKLIST COMPLETED".

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.5.     TURN AROUND

Items 1 - 4 to be performed on intermediate stops only if

the cockpit crew has changed.

Item 5 to be performed only if electrical power was

switched off,

During a normal turn around after completing the PARKING

checklist, continue the TURN AROUND checklist with item

6.

Procedure:

CM1 announces:"TURN AROUND CHECKS"

CM1 takes care of the refuelling and makes a turn around

inspection.

When starting the procedure with items 1 - 4, perform

these items like described in the BEFORE FIRST TAKE OFF

CHECK-LIST.

If Item 5 has to be performed, then use SECURING

CHECKLIST from bottom to top with reverse action.

NOTE : H-Mode can be started only after completion of the

turn-around checklist.

CM2 performs the turn around checks,

Circuit Breakers

- Check on IateraI and overhead C/B panels that all CBs

are in, reset as applicable,

Parking Brake/Press

- Set handle to PARKING,

- Check BRAKE ACCU Pressure 3000 psi.

- Use AUX HYD PUMP if necessary.

Engine Levers

- Check both PL on GI, and both CL on FSO.

Gust Lock

- Check gust lock engaged.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.5. cont'd

Engine Test

- For both engines:

* Turn the ATPCS to ARM,

* When the ARM green It illuminates turn the ATPCS

to ENG position.

* Check the associated ENG UPTRIM It illuminates.

* 2.15  sec later check ARM it extinguished.

TRIMS

- Check YAW and ROLL -trims to 0.

- Check the STBY PITCH TRIM operation,set the sw in

guarded OFF position.

Set PITCH TRIM to AT 42: 0

AT 72: 1

Landing Elevation

- Set to destination field elevation, rounded up to the

next 100 ft.

Anti Skid

- Press TEST, 4 amber F lit and check extinguished

after test sequence.

Engine Control

- Extinguish any white light.

- Check the UPTRIM, LO PITCH Its are out.

- ATPCS pb in,

Engine Fire

- Check ENG 2 fire handle IN, latched and safety wired,

- Extinguish any white It.

- Depress SQUIB TEST pb and check both AGENT SQUIB lt

illuminate,

- Select TEST sw on FIRE and check:

* ENG FIRE red It illuminates into associated fire

handle.

" CCAS is activated (CRC+MW it flashing red + ENG 2

FIRE red It on CAP).

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.5. cont'd

- Open the CL to FTR and check the red It comes on the

CL.

- Select TEST sw on FAULT and check:

both LOOP A and B FAULT lts illuminate and SC+MC

flashing amber , LOOP FAULT It on CAP.

- Same procedure for ENG I.

Prop Brake

- Check PROP BRK blue Its ON on prop brake control

panel and memo panel.

- If not, to engage: * Push the AUX PUMP pb.

* Check READY lt illuminates.

* Select PROP BRK sw to ON,

* Check UNLK It illuminates then

extinguish.

* Check PROP BRK illuminates both

on prop brake control and memo panel.

Cabin Signs

- Set both sw ON,

- Check ON indication on MEMO panel,

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.5. cont'd

CM2 reads TURN AROUND CHECKLIST

1. PREFLIGHT INSPECTION............COMPLETED............CM1

2. DOCUMENTS..............................ON BOARD...............BOTH

3. EMERGENCY EQUIPMENT..........CHECKED.................BOTH

4. GEAR PINS.................................STOWED..................BOTH

5. Securing Checks..........................reverse.....................CM2 S

-------------------------------------------------------------------------------------------------

6. Circuit Breakers............................checked...................CM2 s

7. Parking Brake/Press.....................set/checked..............CM2 S

8. Engine Levers...............................set GI/FSO................CM2 S

9. Gust Lock....................................engaged.....................CM2 S

10. Engine Test................................performed...................CM2 S

11.Trims..........................................checked/set................CM2 S

12. Landing Elevation........................set.............................CM2 S

13. Anti Skid....................................tested.........................CM2 S

14. Engine control............................set..............................CM2 S

15. Engine Fire................................tested..........................CM2 S

16. Prop Brake.................................on..............................CM2  S

17. Cabin Signs................................on..............................CM2  S

CM2 announces : "TURN AROUND CHECKLIST COMPLETED".

NOTE: If wind condition permits, ENG2 will be started in H-

Mode shortly prior passenger arriving at the A/C. If H-

Mode is running for more then 10 min take a FF of 80 kg/h

into consideration.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.6. ENGINE START

Procedure:

CM1 notes the amount of fuel planned for departure into

the A/C LOG crosschecking FUEL QTY IND and FUEL SLIP,

FUEL QUANTITY/FU

He checks that both fuel tanks are refuelled symmetri-

cally and the total fuel amount corresponds to the

planned tankage.

CM2 sets FU indicators to zero if not operating in H-

Mode.

CM2 copies the latest ATIS.

ALTIMETERS

CM2 is setting his ALT and STBY ALT to actual QNH.

CM1 is setting his ALT to actual QNH.

CM1 determines the max ALL.WEIGHT for T/O and confirms

the TORQUE BUG setting (acc. FOD or FCOM/AFM),

TORQUE BUGS

CM2 sets the manual bugs to computed T/O value.

COM/NAV/ADF

PF for the next flight performs the departure briefing

and confirms the NAV/ADF set up. (For standard NAV SET

UP see FTM chapter 5 Procedures and Techniques)

On the first flight of the day for the crew, the PF will

perform the expanded EMERG BRIEFING. (See FTM chapter 5

Procedures and Techniques)

Both crew members are checking their instruments.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.6. cont´d

FLIGHT INSTRUMENTS

- Crosscheck RMI/EHSI heading information and absence

of flags,

- Check VSI no flag and pointer shows zero.

- Check EADI no flags and cautions, horizon normal.

- Check ALTIMETER no flag.

- Check ASI no flag, pointer indicates 0 and VMO

pointer indicates 250 kt.

At least 15 min prior scheduled departure, CM1 reports

"READY FOR BOARDING" to the ground handling.

After receiving the loadsheet CM1 checks and signs the

load sheet.

TRIMS

CM1 sets PITCH TRIM according to load sheet for T/O.

SPEED MARKS/BUG

Both CM1 and CM2 set their speed MARKS and BUGS acc. to

the actual T/O weight.

- Set T/O speeds according to the actual T/O weight.

- Set the speed marks to V1, Vmlb0 NORM and Vmlb 0

ICING,

- Set the speed bug to V2.

When PAXs on board, CA reports number of PAX to CM1. If

that number corresponds to the loadsheet figures, CM1

will agree with the closure of the cabin door.

DOORS

- Check that all Its are extinguished on the doors

panel,

When cabin door is closed:

- CM2 requests start up on command of CM1.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.6.    cont'd

CM1 announces:"ENGINE START CHECKLIST DOWN TO THE LINE",

CM2 reads the ENGINE START CHECKLIST down to the line

1. FUEL QUANTITY/FU ...............___/___........................BOTH

2. TORQUE BUGS .....................___ % OBJ TORQUE .....BOTH

3. COM/NAV/ADF......................SET FOR DEP................BOTH

4. ALTIMETERS.........................___hpa/___ft..................BOTH

5. FLIGHT INSTRUMENTS.......... CHECKED/___°.............BOTH

6· TRIMS ...................................SET/___°.......................BOTH

7. SPEED MARKS / BUG............SET/ ___t......................BOTH

8. DOORS..................................CLOSED.......................BOTH

----------------------------------------------------------------------------------------------

and reports: "ENGINE START CHECKLIST DOWN TO THE LINE

COMPLETED".

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.6.    cont'd

On first flight of the day or after an extended parking

period, both engines have to be started in order to get

a sufficient oil temperature.

This Procedure has also to be executed on contaminated

apron / taxiways or if a sharp turn out of parking

position is required.

In these cases start ENG1 at first, prior to releasing

PROP BRK,

Check engine start- and/or prop brake release procedure

in FTM chapter 5 Procedures and Techniques.

Procedure:

When start up clearance obtained and clear signal re-

ceived from the ground staff CM1 switches on the FUEL

PUMPS

Fuel Pumps

- Select the PUMPs ON and check:

* RUN it illuminates.

* FEED LO PR It extinguishes.

* LP VALVE in line,

* X FEED valve X line.

Beacon/Wing Light

- CM1 sets BEACON and WING LT to on.

CM1 starts ENG 2 by releasing the prop brake or ENG 1+2

as described above.

Engine 2

For Engine start procedure check FTM chapter 5

Procedures and Techniques.

CM checks engine readings normal end advances CL to

MAX RPM.

When ENG 2 started CM1 selects FLAPS 15.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.6 cont'd

Flaps

- Select flaps to 15 for T/O and checks flaps position

on the flaps position indicator,

NOTE: For flaps setting during winter operation refer to

chapter 6 ADVERSE WEATHER OPERATION.

Gen 2/Ext PWR PB

CM1 checks voltage of DC GEN and deselects EXT PWR PB

and observes DC load decreasing below 400 A.

Emerg Exit Lights

CM1 sets EMERG EXT LT to ARM,

CM1 gives the sign to remove GPU/Chocks and

GPU/CHOCKS

checks all clear sign received from the ground staff.

AT42 - CA reports:Cabin ready for taxi"

AT72 - CA reports:"Cabin ready for taxi, tail prop

on board"

CABIN REPORT FOR TAXI

AT 72 TAIL PROP

- Check that CA reported CABIN READY FOR TAXI.

- AT72: Check that CA reported CABIN READY FOR TAXI and

TAIL PROP ON BOARD,

CM1 and CM2 confirm: "CHECKED"

When GPU and chocks are removed CM1 announces:"ENGINE

START CHECKLIST TO THE END"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.6. cont'd

White Lights

CM2 extinguishes any white light and reads the check-

list.

9. Fuel Pumps.......................................on..........................CM1 S

10. Beacon/Wing Light...........................on...........................CM1 S

11. Engine 2..........................................started.....................BOTH

12. Flaps...............................................set/indicating 15...... CM1 S

13. Gen 2/Ext PWR PB..........................checked/off..............CM1 S

14. Emerg Exit Lights.............................armed......................CM1 S

15. GPU/CHOCKS..................................REMOVED..............BOTH

16. White Lights......................................extinguished...........CM2 S

17. CABIN REPORT FOR TAXI.................RECEIVED.............BOTH

AT 72 TAIL PROP.............................ON BOARD.............BOTH

CM2 announces:"ENGINE START CHECKLIST COMPLETED"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.7.    TAXI

Procedure:

CM1 announces:"REQUEST TAXI"

CM2 requests taxi clearance from ATC.

When taxi clearance received:

Taxi Light

CM1 select taxi lt on and taxi to the assigned RWY.

CM1 anncunces:"BRAKES CHECK"

BRAKES

CM1 checks his brakes.

Thereafter CM1 announces: "CHECK YOUR BRAKES"

CM2 checks his brakes.

ENGINES

If not already done for reasons described before:

CM1 anncunces:"START ENGINE NUMBER 1"

CM2 starts Eng 1 and anuounces: "ITT" when light up

occurs and:"ENGINE NUMBER 1 STARTED" when start

procedure completed,

CM2 checks both engine readings normal and both CL

on MAX RPM.

ANTI/DEICE/WX RDR

At the first flight of the day or when requested by

CM 1, CM 2 performs the ANTI/DEICE/WX RDR TEST and

announces: "ANTI/DEICE WX RDR TEST"

- Switch on level 2 and 3 (mode selectors norm).

- Check no FAULT during 70 sec.

- Switch level 2 and 3 back to NORMAL.

- Press PTT pb of ICE DET and check MC and ICING It on

steady.

- Deselect AOA, press PTT pb of ICE DET and check MC

and ICING It flashing,

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.7  cont'd

- Switch WX RDR to TEST and check:

* Range is automatically selected to 100 NM.

* Three arcs appear on EHSI, with green, yellow

and red colours, plus an are predominately

green between 85 and 95 NM,

TEST appears on the top left of EHSI.

When completed CM2 announces:"ANTI/ DEICE/WX RDR TEST

COMPLETED" :

CM1 commands: "TO CONFIG TEST"

CM2 confirms: "TO CONFIG TEST"

T/O CONFIG

CM2 depresses the TO CONFIG TEST pb and checks that

no alert appears.

CM2 reports: "TO CONFIG TEST COMPLETED"

CM1 announces: "T/O BRIEFING"

PF performs the abbreviated T/O Briefing,(see FTM

Chapter 5 Procedures and Techniques) and confirms

ADU setting.

CABIN REPORT FOR T/O

CA reports:"CABIN OK".

CM1 and CM2 confirm:"CHECKED"

CM1 announces:"TAXI CHECKs":

Overboard Valve

CM2 switches OVBD VALVE to AUTO and guarded.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.7.     cont'd

CM2 offers "TAXI CHECKLIST",

CM1 accepts the TAXI CHECKLIST when ready: "GO AHEAD"

CM2 reads the TAXI CHECKLIST.

1. Taxi Light..................................on.................................CM1 S

2· BRAKES ..................................CHECKED.....................BOTH

3· ENGINES .................................STARTED.....................BOTH

4. (as required)

ANTI/DEICE/WX RDR..................TESTED.......................CM2

5. T/O CONFIG...............................TESTED.......................CM2

6. Overboard Valve...........................auto..............................CM2

7. T/O BRIEFING/ADU..................... PERFORMED/SET........PF

8. CABIN REPORT FOR T/O .............RECEIVED..................BOTH

CM2 announces:"TAXI CHECKLIST COMPLETED"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.8.  BEFORE TAKE OFF

Procedure:

CM1 announces:"READY FOR DEPARTURE"

CM2 advises ATC

When T/O clearance received and before entering the RWY:

CM1 or CM2, depending on whose side the approach sector

is located checks the approach sector clear and

announces:"APP sector clear"

CM1 announces:"BEFORE TAKE OFF CHECKS'

During line up CM1 checks his rudder and centers the

RUDDER CAM

CM2 starts the T/O SET UP:

XPRD

- checks that the assigned code is selectad and turns

the sw to ALT.

FLIGHT CONTROLS

- Checks GUST LOCK released,

- Checks the full travel and freedom of movement in

PITCH, ROLL (incl. SPOILER It).

- CM1 checks YAW and centers the rudder releasable

centering unit,

COND LEVERS

- Checks that CL at MAX RPM.

CCAS

- Pushes RCL and checks no FAULT lt.

- Pushes TO INHI and checks TO INHI It ON.

External Lights

- Checks all EXT LT ON

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.8.  cont´d

Engine Start selector

- Sets CONT RELIGHT ON for takeoff.

ANTI/DEICE

- Switches ON if required according to T/O Briefing

(see Chapter 6 Adverse Weather Operation

AT42 BLEED VALVES

- Checks BLEEDS OFF for T/O.

AT 72 BLEED VALVES

- Checks BLEEDS ON/OFF according T/O Briefing (FOD/AFM)

FLIGHT INSTRUMENTS

- Checks ATTITUDE and HEADING information

- Checks that no flags are visible.

- Compares HDG read out to RWY direction.

- Checks lateral FD Bar centered.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.8.    cont'd

CM2 offers: "BEFORE TAKE OFF CHECKLIST"

CM1 accepts BEFORE TAKE OFF CHECKLIST: "GO AHEAD".

CM2 reads the BEFORE TAKE OFF checklist

1.  XPRD................................ALT........................... CM2 S

2. FLIGHT CONTROLS ...........CHECKED...................BOTH

3. COND LEVERS ...................MAX...........................CM2

4. CCAS..................................recall/ T/O inhi.............CM2 S

5. External Lights.....................on...............................CM2 S

6. Engine Start Selector............cont relight...................CM2 S

7. ANTI/DEICE.........................AS REQUIRED.............CM2

8. AT 42   BLEED VALVES.......OFF.............................CM2

AT 72   BLEED VALVES.......AS REQUIRED..............CM2

9. FLIGHT INSTRUMENTS........CHECKED, HDG__°.......BOTH

CM2 announces:"BEFORE TAKE OFF CHECKLIST COMPLETED"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.9. TAKE OFF

Procedure:

When T/O clearance received

CM1 announces:"MY STEERING, YOUR COLUMN"

CM1 and CM2 start TIMING,

CM1 holds NOSE WHEEL TILLER and FL,

CM2 holds control column into the wind andapplies

slight forward pressure and confirms: "MY COLUMN"

CM1 announces: "TAKE OFF"

AT 42 CM1 advances both PL to the WHITE BAND

AT 72 CM1 advances both PL to the NOTCH.

When ATPCS ARM LT illuminated.

CM2 reports: "ATPCS ARMED",

AT 42 CM1 requests: "ADJUST TORQUE",

CM2 adjusts the torque end reports:

"TORQUE ADJUSTED"

AT 72 CM1 announces: "IN THE NOTCH CHECK TORQUE"

CM1 checks the torque rnatching the objective T/O

Torque and calls out any deviation,

When all indications are normal

CM2 reports: "TORQUE CHECKED"

From now on, both pilots have their hand on the PLs

until v1.

The A/C accelerating and at 70 kts IAS

CM2 reports: "SEVENTY"'

CM1 crosschecks with his ASI and checks that the speed

differs not more than 10 kts, and acknowledges: "CHECKED"

PF CM1:   CM1 commands: "MY CONTROL" and takes over the

control column and CM2 confirms: "YOUR CONTROL"

PF CM2:   CM1 commands: "YOUR CONTROLS" CM2 takes over

the control column and rudder and

confirms: "MY CONTROLS'

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

0.4.9.  cont´d

From this moment on CM1 does not steer the aircraft

with the nose wheel steering anymor,

The PF now is using the rudder smoothly for directional

control.

The PNF reports: "V1" , "ROTATE" , "V2" at the respective

speeds.

The PF is checking silent on his ASI for the correct

value, and rotates the A/C at VR smoothly to

- AT 42: 12° nose up

- AT 72:   8° nose up until positive climb occurs, then

12°- 15°

to keep V2 + 5 kts up to acceleration ALT.

When positive ROC is established, seen by the PNF on

BARO ALT and RADAR ALT, PNF reports: "POSITIVE CLIMB"

PF checks on BARO ALT and RADAR ALT the positive ROC

and coIllmands: "GEAR UP"

PNF confirms "GEAR UP"

Landing Gear

PNF - selects the gear lever up

- puts the YAW Damper ON and

- switches the TAXI LT to OFF

- checks all GEAR LTs extinguished and

PNF reports: "GEAR LIGHTS OUT",

At 400 ft RA the PNF announces: "400 FT"

PF checks on his RA and confirms: "CHECKED"

PNF contacts ATC.

At ACC. ALT (BARO ALT) the PNF announces: "ACCELERATION

ALTITUDE"

PF checks his ALT for the correct value and requests:

"SPEED INCREASE"

PNF adjusts standard climb speed on ADU with PTW

- AT 42: 160 kts

- AT 72: 170 kts

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.9. cont'd

Passing VmLB 0 aocording normal or icing conditions

the PNF reports: "FLAP RETRACTION SPEED"

PF checks on his ASI for the correct speed and

commands: "FLAPS ZERO"

PNF confirms: "FLAPS ZERO"

Flaps

PNF - moves the flap lever to 0.

- observes the fIap moving on the indicator and

PNF reports: "FLAPS INDICATING ZERO"

PF checks silent the flap position and commands:

"CLIMB SEQUENCE"

AT 42: PF retards the PLs 5° PLA.

AT 72: PF checks/retards PL into the NOTCH.

PNF confirms: "CLIMB SEQUENCE"

Thereafter PNF performs the CLIMB SEQUENCE

PWR Management

PNF Sets PWR MGT to CLB.

- Sets manual torque bugs to max. climb torque.

- Sets CLs to 86 % NP,

Bleed Valves

PNF sets/checks BLEED VALVES to ON

PNF asks "CONT RELIGHT ?"

PF decides "ON" or "OFF" acoording the environmental

condition (see Adverse weather Operation chapter 6).

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.9.  cont'd

Engine Start Selector

PNF sets the engine start selector to OFF when required

(see Adverse weather Operation chapter 6)

PF / PNF set their internal speed bug to

AT 42: 160 kts

AT 72: 170 kts

AT 42: PF adjusts PLs to match FDAU target bugs.

AT 72: PF checks torque matches FDAU target bugs.

PNF reports: "CLIMB SEQUENCE COMPLETED"

NOTE: AP may be used from this point on

Passing transition altitude PNP reports: '"TRANSITION

ALTITUDE"

PF checks on his ALT the correct value and requests:

"AFTER TAKE OFF CHECKS",

CM1 decides whether NO SMOK SIGNs ON or OFF.

(CABIN SIGNS are Captain switches!) "NO SMOKING SIGN ?"

According environmental conditions PF (CM1) requests:

"OFF" or "ON"

No Smoking Signs

PNF sets No smoking signs according to CM1 decision.

PF requests "STANDARD QNH"

ALTIMETERS

PF sets 1013,25 hpa on his ALT

PNF sets 1013,25 hpa on SBY ALT and his ALT

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

0.4.9. cont'd

PNF offers "AFTER TAKE OFF CHECKLIST"

PF accepts the After Take Off Checklist when SID

and or communication no longer require attention

of both pilots: "GO AHEAD".

PNF reads the AFTER TAKEOFF CHECKLIST:

1. Landing Gear..........................up.........................PNF S

2. Flaps.....................................set/indicating 0.......PNF S

3. PWR Management..................CLB......................PNF S

4. Bleed Valves...........................on........................PNF S

5. Engine Start Selector...............set.......................PNF S

6. No Smoking Signs...................set.......................PNF S

7. ALTIMETERS .........................___hpa/ ___ft........BOTH

PNF announces: "AFTER TAKE OFF CHECKLIST COMPLETED"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.10. CLIMB

Procedure:

when passing FL 100,  PNF reports: "FL 100"

PF requests "EXTERNAL LIGHTS OFF" and "FASTEN SEAT

SELT SIGN OFF/ON"

PNF selects LDG, WING DEICE and LOGO LIGHTs off. If there

is no further need switch the FASTEN SEAT BELT SIGN off,

(This action must be confirmed by CM1). If the CABIN

SIGNS have to stay on, CA has to be notified accordingly.

During climb passing FL100 FD/AP IAS mode may be

changed to VS or BASIC PITCH Mode, however minimum

standard climb speed is

-   AT 42 = 160 kts IAS

-   AT 72 = 170 kts IAS

For climb in icing conditions see chapter 6 Adverse

Weather Operation. In order to maintain visual control

of the airspace when flying VMC, paperwork has to be

restricted to the absolutely neceesary minimum,

especially below FL 100.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.11. CRUISE

Procedure:

After reaching cruising level and Cruise TAS,

PF commands: "CRUISE TORQUE"

PNF acknowledges: "CRUISE TORQUE"

PNF selects PWR MGT selector to CRZ.

When Torque is below: 	- AT 42 = 80 %

- AT 72 = 92 %

and condition permitting

PF requests: "NP 77%"

PNF selects condition lever to 77% NP

NOTE: use of NP 77% is prohibited in icing conditions.

AT 42: PF adjusts the torque.

AT 72: PF checks that the torque matches the FDAU targets

PNF reports: "CRUISE TORQUE SET"

PF / PNF set their internal speed bug to CRZ IAS.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.12    DESCENT

Procedure:

when receiving clearance for descent, PF calls for

"DESCENT CHECKS"

Seat Belt Sign

PNF - switches the FASTEN SEAT BELT SIGN to ON, however

this action should not be delayed belew FL 100,

- checks that SEAT BELT illuminated on MEMO panel

NOTE: This action has to be confirmed by CM1.

CCAS

PNF - depress the RCL p/b and

- checks if any system failure is indicated on CAP

Landing Elevation

PNF checks/set the LANDING ELEVATION.

Both, PF and PNF, check now the FU to confirm the

correct landing weight,

SPEED MARKS

Both PF and PNF:

- Set the speed MARKS to 	1. VmHB LDG CONFIG

2. VmLB 0 NORM COND           		                                                           						3. VmLB 0 ICING COND

- Set the speed BUG to VMO - 10 kts

PNF offers: "DESCENT CHECKLIST"

PF accepts DESCENT CHECKLIST: "GO AHEAD"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.12. cont'd

PNF reads the DESCENT CHECKLIST.

1. Seat Belt Sign.........................on.................PNF S

2. CCAS ....................................recall.............PNF S

3. Landing Elevation.....................set................PNF S

4. SPEED MARKS......................SET/ ___t......BOTH

PNF reports: "DESCENT CHECKLIST COMPLETED"

During descent PNF checks and notes the latest MET

report from ALT and DEST AIRPORT.

When approaching FL 100, PNF reports: "FL 100"

PF requests "EXTERNAL LIGHTS ON",

PNF selects LDG, WING DEICE and LOGO LTS ON.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.13. APPROACH

Procedure:

When cleared down to QNH Altitude, PF sets his ALT to

the local QNH and PNF sets the SBY ALT. The ALT of PNF

remains on standard QNH until passing the TRANSITION

LEVEL.

APPROACH BRIEFING

PF performs the APP.BRIEFING ( See FTM Chapter 5

Procedures and Techniques) as soon as possible after

the completion of the descent checks

Whon Passing TRANSITION LEVEL, PNF reports: "TRANSITION

LEVEL" and PF requests: "APPROACH CHECKS"

ALTIMETERS

PNF sets his ALT to local QNH

SPEED BUG

Both, PF / PNF, 	- set internal SPEED BUG to VApp

- Confirm SPEEDS acc NORM or ICING

conditions

TORQUE BUGS

PNF sets manual BUGS to computed GA TORQUE value

Cabin Altitude

PNF 	- checks that CABIN ALTITUDE and differential

pressure are decreasing on schedule

- confirms LANDING ELEVATION

Cabin Signs

PNF - switches the NO SMOKsignto ON, however this

action should not be delayed unneccesarily in

order to allow cabin the required checks in an

orderly manner.

- checks SEAT BELT sign ON

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.13 cont'd

ANTI-/DEICE

PNF checks ON if required (See FTM Chapter 6 Adverse

Weather Operation)

PNF offers: "APPROACH CHECKLIST"

PF accepts APPROACH CHECKLIST: "GO AHEAD"

PNF reads the APPROACH CHECKLIST

1. APPROACH BRIEFING.........PERFORMED................PF

2. ALTIMETERS.......................__hpa/___ft...................BOTH

3. SPEED BUG........................SET/____kts...................BOTH

4. TOQUE BUGS.....................__ % GA TORQUE..........PNF

5. Cabin Altitude......................checked..........................PNF S

6. cabin Signs ........................both on............................PNF S

7. ANTI-/ DEICE......................AS REQUIRED.................PNF

PNF reports: " APPROACH CHECKLIST COMPLETED "

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.14. FINAL

Procedure:

For different approach procedures see FTM Chapter

Procedures and Techniques.

CABIN REPORT FOR LANDING

CA reports: "CABIIN OK""

PF and PNF confirm: "CHECKED"

Passing deceleration altitude, PF retard both PLs to FI

and reduces airspeed.

NOTE: Prior reducing PLs to FI Cont. Relight has to be

switched ON

When speed VFE PF commands : "FLAPS 15"

PNF acknowledges: "FLAPS 15"

FLAPS

PNF 	- checks the speed < VFE and

- selects flap lever to 15

- observes on indicator the moving and when indica-

ting 15 check EXT flag disappeared

- reports: "FLAPS INDICATING 15"

PF checks the indication and when reaching VLO he

commands: "GEAR DOWN"

PNF acknowledges: "GEAR DOWN"

LANDING GEAR

PNF - checks the speed < VLO

- selects the gear lever down

PWR Management

PNF sets the PWR MGT selector to T/O

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.14. cont'd

External LTs

PNF 	- switches the TAXI LT on

- checks/sets all External Lts ON

Engine start Selector

PNF - checks/sets Engine Start Selector on CONT RELIGHT

When the gear is down 6 green Its on PNF reports:

"3 GREENS"

When reaching VFE30

PF commands: "FLAPS 30"

PNF acknowledges: "FLAPS 30 "

FLAPS

PNF - checks the speed < VFE

- selects flap lever to 30

- observes on indicator the moving and when

indicating 30 check EXT flag disappeared

- reports: "FLAPS INDICATING 30 "

PF checks the indication and when approaching VApp

AT 72 TLU

PF checks green LOW SPEED LT illuminated on center

panel

CONDITION LEVERS

AT 42 - PF requests : "MAX RPM"

AT 72 - PF reguests : "NP 86 %"

or when required = "MAX RPM"

(See Procedures and Techniques Chapter 5)

PNF advances CL smoothly to

AT 42  -   MAX RPM

AT 72  -   86 % or MAX RPM

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.14. cont'd

PNF offers : "FINAL CHECKLIST"

PF accepts the FINAL CHECKLIST: "GO AHEAD"

PNF reads the FINAL CHECKLIST

1. CABIN REFORT FOR LANDING.........RECEIVED.....................BOTH

2. LANDING GEAR...............................DOWN/ 3 GREEN............BOTH

3. PWR Management............................T/O.................................PNF S

4. External Lights..................................set..................................PNF S

5. Engine Selector.................................Cont. ReIight...................PNF S

6. FLAPS..............................................SET/INDICATING  30......BOTH

7. AT 72 TLU .........................................LOW SPEED.................BOTH

8. AT 42 CONDITION LEVERS ................MAX RPM......................BOTH

9. AT 72 CONDITION LEVERS..................AS REQUIRED...............BOTH

Procedure:

PNF announces: "FINAL CHECKLIST COMPLETED"

NOTE: At 500 ft above field elevation the aircraft should be

fully established on final approach speed and final

checklist should be completed.

When approaching 500 ft BARO PNF reports: "500 ft"

PF - checks on his ALT and RA

- confirms "CHECKED"

When reaching 100 ft before published minimum PNF

reports: "APPROACHING MINIMUM"

PF checks on his ALT and confirms: "CHECKED"

At minimum, AUTO CALL or PNF reports: "MINIMUM"

PF decides and reports: "CONTINUE " or "GO AROUND"

CM1 confirms the decision "CONTINUE" or "GO AROUND"

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.14. cont'd

GO AROUND procedure: see FTM Chapter 5 Procedures and

Techniques

CONTINUE procedure;

When reaching 100 ft BARO PNF reports: "100 FT"

PF checks on his instruments and confirms: "CHECKED"

PNF reports the RAs	"50"

"40"

"30"

"20"

"10"

At 20 ft RA the PF reduces the PL to FI and lands the

A/C.

At 70 kts the PNF reports: "SEVENTY"

If CM 1 is PNF:

CM1 commands: '"MY CONTROL, YOUR COLUMN"

CM2 confirms: "YOUR CONTROL, MY COLUMN"

If CM 1 is PF:

CM1 commands: "YOUR COLUMN"

CM2 confirmes: "MY COLUMN"

CM 1 takes over the PLs and controls the nose wheel

steering and the brakes as required.

CM 2 takes over the control column, applies slight

forward pressure and banks into the wind.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.15    AFTER LANDING

Procedure:

When leaving the RUNWAY, CM 1 requests: "AFTER LANDING

CHECKS"

CM2 performs the AFTER LANDING SET UP:

GUST LOCK

CM2 engages gust lock.

XPRD

CM2 sets XPDR switch to STBY.

FLAPS

CM2 - Sets flap lever to 0.

- Checks flaps indicating 0 on FLAP POS IND,

RA Altitude

CM2 checks/sets RA to 0.

WX Radar

CM2 checks/sets WX Radar to STBY.

TRIMS

CM2 AT 42 Checks/sets all trims to 0.

AT 72 - Checks/sets pitch trim to 1° uose up,

External LTs

CM2 - Sets landing and strobe lights OFF,

- At daylight sets logo lights OFF,

Engine Start selector

CM2 sets Engine start selector to OFF/START ABORT.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.15. cont´d

ANTI-/DE ICE

CM2 checks/sets ANTI-/DE ICE to OFF.

STBY Probe heating

CM2 sets STBY probe heating to OFF.

On the last flight of the day CM1 commands: "TAXI LIGHT

OFF, ENGINE TEST"

CM2 confirms "TAXI LT OFF, ENGINE TEST"'

CM2 	- performs the ENG TEST:

- turns ATPCS selector to ARM

* ARM It illuminates green

* Torques increase

* NH and NP decrease on both ENG

-  turns ATPCS selector to ENG

* selected ENG Torque decreases below 21%

Opposite ENG:

* Torque doesn't change

* UPTRIM LT illuminates

* NP and NH increase slightly

2.15 sec later:

* ARM LT extinguish

- checks Auto feathering of the affected PROP

CAUTION: 	- Do not perform ENG TEST while taxing if DC HYD

pump is not operating.

- During ENG TEST, ACW is temporarily lost and

consequently BOTH MAIN HYD PUMPS are tempora-

rily lost as well.

- If braking is required during test, it will be

performed using the EMER handle as required,

NOTE: If test must be repeated, wait 10 minutes before

setting ATPCS selector to ENG position in order not

to damage feathering pump (winding heating).

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.15. cont'd

CM2 reports: "ENGINE TEST COMPLETED"

CM1 requests: "TAXI LIGHT ON"

CM2 switches the TAXI LT on.

CM1 orders: "FEATHER ENGINE #1 "

CM2 feathers the engine and starts timing.

CM2 offers "AFTER LANDING CHECKLIST"

CM1 accepts AFTER LANDING CHECKLIST: "GO AHEAD"

CM2 reads the AFTER LANDING CHECKLIST

1. GUSTLOCK.......................engaged..................CM2 S

2. XPDR................................STBY......................CM2 S

3. FLAPS.............................set/indicating 0.........CM2 S

4. Radar Altitude...................set 0........................CM2 S

5. WXRadar .........................stby.........................CM2 S

6. Trims................................reset........................CM2 S

7. External Lights..................set...........................CM2 S

8. Engine Start Selector.........off............................CM2 S

9. Anti / De ICE ....................off............................CM2 S

10.Stby Probe Heating...........off............................CM2 S

11.( Last Flight of The Day )

ENGINE TEST.................PERFORMED............CM2

CM2 announces: " AFTER LANDING CHECKLIST COMPLETED "

1 min after feathering engine # 1

CM1 requests: "SHUT OFF ENGINE # 1"

CM2 shuts off ENG #1.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.16    PARKING

Arriving at parking position, CM1 requests: "TAXI LIGHT

OFF" to avoid blinding ground staff.

Parking Brake

On parking position CM1 sets parking brake handle to

PARKING and checks brake pressure

On parking position CM1 requests: "FEATHER ENGINE #2"

CM2 feathers the engine and starts timing,

H-Mode

After 15 sec CM1 checke the green READY lt on and

switches the PROP PRK sw to ON. (see Chapter 5

Procedures and Techniques)

When the prop has stopped rotation

Beacon / Wing Light

CM1 switches off BEACON and WING LT.

Seat Belt Sign

CM1 switches SEAT BELT sign to OFF.

Emerg Exit Light

CM1 sets EMERG EXT LT switch to DISARM.

CM1 checks the AVAIL It of GPU and the voltage on

the LH maintenance panel prior selecting on,

External Power

CM1 switches EXT PWR PB on.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.16. cont'd

H-Mode / Fuel Pumps

CM1  - sets CL fso

- selects both fuel pumps off

- checks white lights on

CM1 requests "PARKING CHECKS"

CM2 - notes fuel used

- resets to 0

Thereafter CM2 offers: "PARKING CHECKLIST "

CM1 accepts PARKING CHECKLIST: "GO AHEAD"

CM2 reads the PARKING CHECKLIST

1. Parking Brake.......................set..................CM1 S

2. H Mode................................set..................CM1 S

3. Beacon / Wing Light..............off...................CM1  S

4. Seat Selt Sign......................off....................CM1 S

5. Emerg Exit Light...................disarmed..........CM1 S

6. External Power......................checked/on......CM1 S

7. H -Mode/ Fuel Pumps............fso/off...............CM1 S

8. Fuel Used.............................set 0.................CM2 S

CM2 announces: "PARKING CHECKLIST COMPLETED"

When the A/C continues for another flight start again

with the TURN AROUND procedure.

When the cockpit crew changes the A/C may be left under

power only if the next crew directly takes over the A/C.

At the  last flight of the day or when  the A/C stops a

longer period the A/C has to be switched down according

to the SECURING procedure.

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.17.   SECURING

Procedure:

CM1 announces :"SECURING CHECKS "

CM2 switches down the A/C in the sequence as listed in

the SECURING CHECKLIST,

Oxygen Main Supply

CM2 - selects the main supply off

- checks that the white OFF light comes on

Probe / Windshield Heating

CM2 - selects the CAPT and F/O probes heating off

- selects the L and R windshield heating off

- checks that all white OFF lights come on

NAV/EFIS/RADAR

CM2 - switches of the respective NAV power knobs to

OFF

- switches off first both HSIs

- checks composite mode,

- switches off the EADIs

COM 2 / ADF / XPRD

CM2 switches the respective power knobs off

Min Cabin Lights

CM2 switches off MIN CAB light

Cabin Signs

CM2 switches off both cabin signs

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.17. cont´d

Fuel Used

CM2 - notes fuel burned

- sets counter to O

CM2 announces "A/C READY FOR SHUT DOWN"

If not already done

H-Mode / Fuel Pumps

CM1 	-switches off H-Mode by selecting CL to fso

- switches fuel pumps off

- checks that white lights come on

COM 1

CM1 switches off power knob

External Lights

CM1 switches off all exterior lights

External power PB

CM1 deselects external power PB

Battery Switch

CM1 - waits until INV 2 amber fault light illuminates

- then switches battery off

ATR 42/72 Flight Training Manual

NORMAL PROCEDURES AT42/72

03.4.17. cont'd

CM1 requests: "SECURING CHECKLIST"

CM2 reads the SECURING CHECKLIST silently.

1. Oxygen Main Supply ...........off.......................... CM2 S

2. Probe/Windshield Heating.....off...........................CM2 S

3. NAV / EFIS / RADAR...........off............................CM2 S

4. COM 2 / ADF / XPDR...........off............................CM2 S

5. Min Cabin Lights..................off............................CM2 S

6. Cabin Signs.........................off............................CM2 S

7. Fuel Used............................set 0........................CM2 S

8. H-Mcde / Fuel Pumps...........fso/off.......................CM1 S

9. COM 1.................................off............................CM1 S

10. External Lights....................off............................CM1 S

11. External Power PB..............off............................CM1  S

12. Battery Switch ...................off.............................CM1 S

CM2 announces:" SECURING CHECKLIST COMPLETED "