Arbs777 Panel

Original version 2.0 by D.Durst and M.Vidal modified by Trevisi Roberto

27 February 2001 rtrevisi@tiscalinet.it

Contents of Arbs777.zip

The file contains:

_ Arbs777.doc       (this document)

_ Arbs777.txt         (this document in text format)

_*.jpg , *.bmp        (images of the panel and aircraft enclosed)

_ panel.arbs777_informations.jpg  (instruments and switches position )

_ file_id.diz

_ panel.arbs777.zip   (panel files)

_ gauges.zip              (gauge files)

_ sound.zip                (sound file)

_ b777-200aairlines.zip  (aircraft files)

_ acsgps17.zip                 (acsgps98)

Introduction

This is a modification of the D.Durst/M.Vidal's dc-9-51 panel for fs98 (version 2.0) ; it can be used with 2 engines, jets and big jets.I mainly use it with Boeing 777 (I enclosed an American Airlines B777 already configured to work with it) but it can be used with Boeing 757,767 and Airbus 320.(I tested on these)

Installation instructions

_Unzip gauges.zip in "Gauges" dir under "Flight Simulator" dir.

_Unzip panel.arbs777.zip in a temporary folder and call it "panel.arbs777" , then copy this folder under "FSFSConv"dir. The result must be the follow:

Flight Simulator>Aircraft>FSFSConv> panel.arbs777>_ arbs777.bmp  _panel.cfg  _spoiler.bmp

_Unzip b777-200aairlines.zip in a temporary folder, call it "b777-200aa" and copy it under "Aircraft" dir.

The result must be the follow:

Flight Simulator>Aircraft> b777-200aa> _772aa101.AIR   _aircraft.cfg

Flight Simulator>Aircraft> b777-200aa>Model> _772aa101.MDL  _model.cfg

Flight Simulator>Aircraft> b777-200aa>Panel>panel.cfg

Flight Simulator>Aircraft> b777-200aa>Sound>sound.cfg

Flight Simulator>Aircraft> b777-200aa>Texture>*.*af

_Unzip sound.zip and put click.wav in "sound" dir under "Flight Simulator" dir

_Unzip and install AcsGps17.zip

then start fs and select the aircraft.

_You can find informations about instruments and switches locations in the panel.arbs777_informations.jpg

To use the panel with other airplanes go in   "your airplane">"Panel">   folder, make a copy of your panel.cfg and rename it, for example like panel_cfg.old , open panel.cfg with notepad and modify it as follow:

[fltsim]

alias=FSFSCONV\panel.arbs777

To come back to your old panel simply restore the initial situation.

Notes

I used for this panel some gauges that I list with informations and the name of  their authors (some are reported in the copyright notes) :

_FF10K-1.gau, FF10K-2.gau          John J. Valley          jjvalley@richmond.infi.net

_Autobrake_V2.GAU,  Abrakedis_V2.GAU   Laurent Crenier     Laurent.Crenier@village.uunet.be

AUTOBRAKE  INSTRUCTIONS FOR USE

To use the gauge, simply move the autobrake selector with left mouse clicks. According to the mouse cursor position,

the selector turns clockwise or anticlockwise.The autobrake status appears in blue. During the automatic braking, the readout changes to red.

The selector positions are (clockwise) :

RTO ...........	If selected, provide maximum braking pressure automatically if the throttles are moved to idle after

reached 85 knots during takeoff roll  (It is the Rejected Takeoff situation). RTO selection is not

permitted in flight.

OFF ...........	Nothing happens ! Autobrake system is OFF. If you switch to OFF when autobrake is active, braking stops

immediately.

DISARM ........	The selector goes automatically to this position when the autobrake system disarms itself. When selected

manually, is similar to OFF.

Level 1 to 4 ..	Provides increasing deceleration rate during a normal landing (see Technical Data). Levels 1 & 2 are

usually selected for landings on dry runways and levels 3 & 4 for landings on wet runways.

MAX AUTO ...... Provides maximal braking pressure during the rollout, e.g. for an landing on a short runway.

AUTOBRAKE LOGIC

The autobrake computer which pilots the system is active when the selector is on any position except OFF and DISARM.

It detects automatically the ground, takeoff and landing phases of the flight. The autobrake actions are determined

accordingly. The TAKEOFF phase begins when 85 knots are reached on the runway and ends when the plane leaves the ground.

The LANDING phase begins at touchdown and ends when the ground speed is inferior to 10 knots.

The autobrake system will automatically apply maximal brake pressure during the TAKEOFF phase (that is when speed is

greater than 85 knots) if RTO is selected AND throttles are moved to IDLE (Rejected Takeoff situation).

The system will automatically disarm when the target ground speed is reached (<10 knots) OR if throttles are moved

forward. Manual braking don't disarm autobrake during RTO automatic braking.

The autobrake system will automatically apply appropriate brake pressure during the LANDING phase if levels 1 to 4 or

MAX AUTO are selected (see Technical data). The system will automatically disarms when the target ground speed is

reached (<10 knots) OR if throttles are moved forward (Go/Around situation) OR in case of manual braking. Nevertheless,

manual braking don't disarm autobrake during MAX AUTO automatic braking.

The autobrake system will also automatically disarm  in the following situations :

a. On the ground, during the TAKEOFF phase, when RTO is selected and brakes are applied manually.

b. During normal takeoff with RTO selected, autobrake switches to OFF just after the liftoff.

c. If throttles are advanced beyond idle after touchdown, with levels 1 to 4 or MAX AUTO selected (Go/Around situation).

d. When the target speed (<10 knots) is reached by the automatic braking.

TECHNICAL DATA

This gauge has been tested during "standardized" landings (ILS approach),  with the FS98 Boeing 737-400 at the Brussels

International Airport (EBBR) on the runway 25L, without winds.

You find here the length (in seconds) to decelerate from touchdown to less than 10 knots (the moment when the autobrake

switches to disarm) with each autobrake status. Speed at touchdown was 140 knots, the spoilers were not extended and the

thrust reversers not used.

Autobrake Status			Time to disarm

______________________________________________________

MAX AUTO .............................	14 seconds

LEVEL 4 ..............................	17 seconds

LEVEL 3 ..............................	35 seconds

LEVEL 2 ..............................	47 seconds

LEVEL 1 ..............................	65 seconds

During automatic braking, the red "BRAKES" indicator of the Flight Simulator sometimes flickers, especially when

levels 1 to 3 are selected. It's normal !

The reason is that it is currently impossible (due to the gauge development interface) to force the brakes to apply

a constant, non maximal pressure.  However, the autobrake computer bypass this "software" problem and applies short,

discontinuous raisings in brake pressure to reach the selected deceleration pattern. This can be seen as an antiskid

simulation !

Copyright

I report the original copyright in the readme file:

YOU MAY NOT MODIFY/USE any part of this Panel without the express consent of either

Mike Vidal or David Durst. If you would like to use any part or the gauges, please

contact one of us for the permission to use, and supply at least a Picture so we may

see the changes that have been made.

Additional copyright notes on gauges made by HGHB Virtual Instruments, Hof , Germany

The gauges made by HGHB and included in this panel are freeware, as long as distributed

with this panel itself. If anyone wants to include them in a panel that is to be

distributed in any manner, writtern permission of the author(s) is necessary.

The permission will only be granted when the panel itself has been verified by the

authors of these gauges. If anyone wants to include these gauges into a panel that

is to be distributed as payware in any way, a small fee has to be paid. Contact the

Authors by E-Mail:

H.GEIER@hof.baynet.de and helmut.busch@hof.baynet.de

Original panel by David Durst/Mike Vidal modified by Trevisi Roberto.

27 February 2001 rtrevisi@tiscalinet.it