/* excerpt from Ralph Brown's PORTS.LST */

#if 0

PORT 03F0-03F7 - FDC 1	(1st Floppy Disk Controller)	second FDC at 0370

Note:	floppy disk controller is usually an 8272, 8272A, NEC765 (or

compatible), or an 82072 or 82077AA for perpendicular recording at

2.88M

SeeAlso: PORT 0370h-0377h

03F2   W  diskette controller DOR (Digital Output Register) (see #P151)

03F4  R	  diskette controller main status register (see #P152)

Note:	in non-DMA mode, all data transfers occur through

PORT 03F5h and the status registers (bit 5 here

indicates data read/write rather than than

command/status read/write)

03F4   W  diskette controller data rate select register (see #P153)

03F5  R	  diskette command/data register 0 (ST0) (see #P154)

status register 1 (ST1) (see #P155)

status register 2 (ST2) (see #P156)

status register 3 (ST3) (see #P157)

03F5   W  diskette command register.  The commands summarized here are

mostly multibyte commands. This is for brief recognition only.

(see #P160)

03F6  --  reserved on FDC

03F6  rW  FIXED disk controller data register (see #P158)

03F7  RW  harddisk controller (see #P159)

03F7  R	  diskette controller DIR (Digital Input Register, PC/AT mode)

bit 7 = 1 diskette change

bit 6-0   tri-state on FDC

03F7  R	  diskette controller DIR (Digital Input Register, PS/2 mode)

bit 7	 = 1 diskette change

bit 6-3 = 1

bit 2	     datarate select1

bit 1	     datarate select0

bit 0	 = 0 high density select (500Kb/s, 1Mb/s)

conflict bit 0	   FIXED DISK drive 0 select

03F7  R	  diskette controller DIR (Digital Input Register, PS/2 model 30)

bit 7	 = 0 diskette change

bit 6-4 = 0

bit 3	     -DMA gate (value from DOR register)

bit 2	     NOPREC (value from CCR register)

bit 1	     datarate select1

bit 0	     datarate select0

conflict bit 0	   FIXED DISK drive 0 select

03F7   W  configuration control register (PC/AT, PS/2)

bit 7-2       reserved, tri-state

bit 1-0 = 00  500 Kb/S mode (MFM)

= 01  300 Kb/S mode (MFM)

= 10  250 Kb/S mode (MFM)

= 11  1   Mb/S mode (MFM) (on 82072/82077AA)

conflict bit 0	   FIXED DISK drive 0 select

03F7   W  configuration control register (PS/2 model 30)

bit 7-3       reserved, tri-state

bit 2	       NOPREC (has no function. set to 0 by hardreset)

bit 1-0 = 00  500 Kb/S mode (MFM)

= 01  300 Kb/S mode (MFM)

= 10  250 Kb/S mode (MFM)

= 11  1   Mb/S mode (MFM) (on 82072/82077AA)

conflict bit 0	   FIXED DISK drive 0 select

Bitfields for diskette controller Digital Output Register (DOR):

Bit(s)	Description	(Table P151)

7-6	reserved on PS/2

7	drive 3 motor enable

6	drive 2 motor enable

5	drive 1 motor enable

4	drive 0 motor enable

3	diskette DMA enable (reserved PS/2)

2	=1  FDC enable	(controller reset)

=0  hold FDC at reset

1-0	drive select (0=A 1=B ..)

SeeAlso: #P146,#P152,#P153

Bitfields for diskette controller main status register:

Bit(s)	Description	(Table P152)

7	=1  RQM	 data register is ready

=0  no access is permitted

6	=1  transfer is from controller to system

=0  transfer is from system to controller

5	non-DMA mode

4	diskette controller is busy

3	drive 3 busy (reserved on PS/2)

2	drive 2 busy (reserved on PS/2)

1	drive 1 busy (= drive is in seek mode)

0	drive 0 busy (= drive is in seek mode)

SeeAlso: #P151

Bitfields for diskette controller data rate select register:

Bit(s)	Description	(Table P153)

7-2	reserved on 8272

7	software reset (self clearing)	82072/82077AA

6	power down			82072/82077AA

5	(8272/82077AA) reserved (0)

(82072) PLL select bit

4-2	write precompensation value, 000 default

1-0	data rate select

=00  500 Kb/S MFM	250 Kb/S FM

=01  300 Kb/S MFM	150 Kb/S FM

=10  250 Kb/S MFM	125 Kb/S FM

=11  1Mb/S	MFM	illegal	 FM on 8207x

SeeAlso: #P151

Bitfields for diskette command/data register 0 (ST0):

Bit(s)	Description	(Table P154)

7-6	last command status

00  command terminated successfully

01  command terminated abnormally

10  invalid command

11  terminated abnormally by change in ready signal

5	seek completed

4	equipment check occurred after error

3	not ready

2	head number at interrupt

1-0	unit select (0=A 1=B .. ) (on PS/2: 01=A  10=B)

SeeAlso: #P155,#P156,#P157

Bitfields for diskette status register 1 (ST1):

Bit(s)	Description	(Table P155)

7	end of cylinder; sector# greater then sectors/track

6	=0

5	CRC error in ID or data field

4	overrun

3	=0

2	sector ID not found

1	write protect detected during write

0	ID address mark not found

SeeAlso: #P154,#P156,#P157

Bitfields for diskette status register 2 (ST2):

Bit(s)	Description	(Table P156)

7	=0

6	deleted Data Address Mark detected

5	CRC error in data

4	wrong cylinder detected

3	scan command equal condition satisfied

2	scan command failed, sector not found

1	bad cylinder, ID not found

0	missing Data Address Mark

SeeAlso: #P154,#P155,#P157

Bitfields for diskette status register 3 (ST3):

Bit(s)	Description	(Table P157)

7	fault status signal

6	write protect status

5	ready status

4	track zero status

3	two sided status signal

2	side select (head select)

1-0	unit select (0=A 1=B .. )

SeeAlso: #P154,#P155,#P156

#endif /* end of excerpt from Ralph Brown's PORTS.LST file */