PRRIETRK V3.0

Simple Antenna Steering program for the Prairie Digital Interface

by

WB1BQE

DESCRIPTION:

PRRIETRK  is an antenna steering program designed for use with the Prairie Digital hardware interface published in the 12/94 amsat Journal.  PRRIETRK also supports the use of a “Sun Sensor”  which can be connected to any of the unused A/D converter channels on the Prairie Digital board.

In its most basic form,  PRRIETRK allows on-screen manual antenna control. .   It can also interface with FT7361 - an FT736r control program by the same author, or WINORBIT (by K8CG),  or WISP (by G7UPN)  to provide integrated windows based tracking and tuning functions.  (Use of Prrietrk with Wisp requires the Windows-95 version of Wisp.)

PRRIETRK was originally written for use with the ORBIT-360 televisiom rotator (though a homebrew computer interface), but should work with  most rotators which can be interfaced to the Prairie Digital interface board.

LICENSING:

This program is copyrighted (C) shareware,  all rights reserved.

Permission to use and distribute this progaram is granted, subject to the following terms:

- Any user may use the program for up to 30 days.  After 30 days,  please send a donation to the AMSAT organization of your choice,  and mail me a QSL card at my current callbook address (WB1BQE) telling me that you have done so.  (This is entirely on the honor system,  but I'm curious to know who finds this useful, and will attempt to contact you if bugfixes/updates become available etc. I will  NOT give your name out to anyone else...)

- This program may be freely re-distributed, by non-commercial entities, as long as all the original contents are intact  and this notice is included.   (Additions to files marked  as 'user modifiable' are  permitted.)

- Commercial use or commercial re-distribution is prohibited.

- No warranty of any sort  is provided.

- I disclaim any and all liability for any direct, indirect,  incidental, consequential (including loss of profits) or punitive damages whether based on contract, tort, or any other legal theory which somebody might cook up...

SUPPORT:

I will support this program on a Best Effort basis,  in my spare time.  Written questions and comments may be sent to me at “WB1BQE@AMSAT.ORG”,  or to my Callbook addresss.   Please avoid calling me on the telephone at home...

For those who are experiencing difficulty getting the Prairie Digital Board connected up properly,  I also have a simple utility called PRRIEGEN which reads all 8 ADC channels, and is useful for debugging etc, which is available for the asking.

GETTING STARTED:

PRRIETRK should be relatively self explanatory.  All the files should be placed in the same directory,  and a Windows Program Item (or Windows 95 Shortcut) created  to point to the executable (PRRIETRK.EXE)

Upon running the program for the first time,  the user should run  through all the setup options,  then select the "Save Current Settings" function from the OPTIONS menu, and restart the program.  A more detailed explanation of the various setup options follows:

IT IS VERY IMPORTANT THAT PRIOR TO USE, THAT YOU VISUALLY VERIFY THAT YOUR ANTENNAS ARE MOVING IN THE DIRECTIONS THAT YOU EXPECT.   DO NOT LEAVE THE PROGRAM RUNNING UNATTENDED IN AUTOMATIC MODE UNTIL YOU ARE SURE THAT IT IS OPERATING CORRECTLY.

SETUP OPTIONS:

PRAIRIE BOARD

Enter  the hardware base address where the board is installed,  followed by the signal placement information.  (For convenience,  I allow the Azimuth and Elevation signals to be swapped from the Amsat Journal article.  The Sun Sensor (if installed)  can go on any of the other free channels (2-7.)  If no Sun Sensor is present, enter -1.

AZIMUTH ROTATOR

Enter the direction of  rotation (from 0v),  and the range of position voltages used by your rotator.  (A GR5400 uses 0-5 volts with a clockwise rotation.  An Orbit 360 rotator uses 0-4.3v with a counterclockwise  rotation.)

ELEVATION ROTATOR

Enter the voltage range used by your rotator as prompted.   The program will determine the appropriate rotation direction based on whether the “level” or 180 degree  voltage is larger than the other.  (This is necessary, since “rotation direction”  may be dependent on  which “sideways” mounting scheme is used for TV rotators.)   After setting up the program,  briefly activate the “up” (or down)  control and visually verify that your rotator is indeed turning in the proper direction.  If not, it may be necessary to reverse the “UP” and “DOWN” command signals going from the Prairie board to the rotator.

You may also enter values for the Maximum and Minimum Elevation.  If your tracking program attempts to drive the rotators above or below these values,  the “desired elevation”  window will turn  grey,  and the Maximum (or Minimum) elevation values will be used instead of  the tracking program data.  (This is useful for people with obstructed horizons,  or mechanical positioner limitations.)

STALL DETECTION

This option allows you to either enable or Disable the Stall Detection portion of this program.  It is recommended that you do not disable Stall Detection.

OPERATION:

Program operation is simple.  While "at rest", or under manual control,   the current antenna position will be read and the screen  updated every second.  Manual antenna  movements may be  performed by enabling the “Controls” from the menu bar,  and then simply clicking on any of the 4 "left/right/up/down" buttons on the screen.  The antenna will begin to move, and will stay in motion until the stop button has (in the  center) is pressed.  If the rotator is stalled,  the motion button and associated position displays will turn red,  and your command will be ignored.  Stalls may be cleared via the MAINT  menu.

TRACKING MODES:

PRRIETRK has no internal orbit calculation  ability.  It gets its data from any of  3  external sources:  -   FT7361,  WINORBIT,  and     WISP.  (Wisp mode requires WISP-95 and will run under windows 95 only.)

FT7361 mode creates a DDE link to the FT7361 application (Which must already be running) , and moves the antennas to the  positions specified by the FT7361 program.

WINORBIT mode creates a DDE Link to the WINORBIT program (available from the AMSAT FTP Site)  and uses the tracking data generated by WINORBIT to determine where to point the antennas.  (Antenna movement will only take place when the satellite elevation is above the minimum elevation specified in PRRIETRK’s Setup options.

WISP mode creates a DDE link to WISP (GSC) and uses the tracking data to determine where to point the antennas.  Data from wisp is obtained every 5 seconds,  which should be adequate for all antenna control applications.   (NOTE:  The WISP DDE interface only supports one client DDE connection at a time. .  If you are also using FT7361 with WISP,  you should set PRRIETRK to get its data from FT7361,  rather than directly from WISP.)

During any automatic tracking mode,   PRRIETRK will only position the antennas at values which fall between the Maximum and Minimum elevations specified in the Setup menu.  Commands which would position the antenna outside this range are ignored,  and the appropriate window (Desired Azimuth or Desired Elevation) will turn  light grey.   Likewise,  PRRIETRK maintains a 4 degree “guard band” on either side of the azimuth rotator stop where it will not permit  the antenna to be positioned in any automatic tracking mode.  (This was done due to the tendency of rotators to change calibration with temperature changes which caused frequent stalls.)   The rotators may be set to any position (without limitation) via the manual controls.

It is important to note that during automatic tracking,  manual intervention via the screen is possible,  but likely to be immediately overridden by the next tracking command.

AUTOMATIC STARTUP

Prrietrk can be made to enter any of its 3 operating modes {FT7361, WINORBIT,WISP} automatically at program start time.  This is useful for applications where a computer may be set to boot automatically to work an unattended satellite pass etc.

To enable automatic startup,  create a file called startcfg.dat  in the program directory.  This file should contain a single keyword (in capital letters) specifying the desired startup mode.

A file called STARTCFG.TXT is shipped as a sample.  (It is named startcfg.TXT to prevent automatic startup when you first load the program onto your machine.)

MAINT MODE  (Menu Bar)

The MAINT menu bar allows you to clear the Stall flag for either of the rotators,  as well as enable a maintenance display which shows the actual voltages returned from the rotators,  and any Sun Sensor information.    (Note:  The Sun Sensor signal should be a 0-5v signal with “5v”  corresponding to darkness, and 0v corresponding to bright sunlight.  This reverse range was selected to allow ease of constuction using a standard Photocell (which decreases in resistance with an increase in light) as part of a voltage divider network.   The  “Sun Volts”  display gives the absolute voltage,  while the Bar Graphs give a more intuitive reading of  light intensity.  (Higher Graph readings correspond to brighter light.)

QUIRKS, HACKS,  BUGS and other Unpleasentries:

- This version will work with Windows V3.1, and Windows 95.  It  will not  work with Windows NT,  but since I run NT at home,  this is high on my priority list.

- This version of  PRRIETRK  has only a crude means  of Stall Detection.  When the rotators are set in motion,  a timer is started.  The interval is 2 minutes for azimuth movements,  and 1 minute for elevation movements.  If at the end of this interval,   the antennas are still “in motion”,  a stall will be declared, and all further attempts at motion will be ignored until the stall is cleared.  While a particular rotator is stalled,  motion on the other rotator will be permitted,  unless it is also stalled.   Saving the current settings while a rotator is stalled will result in  the stall information being written to the configuration file.  (This was done to allow for extended operation on one rotator, after the other has failed, as is frequently necessary in winter climates.)

- It is important to note that when stall detection is enabled, the  1 (Elevation) or 2 minute (Azimuth) stall detection intervals are used regardless of the amount of movement desired.  Thus, it is possible for the rotator to be in the stalled condition with power applied for up to 2 minutes.   This hasn't  proved to be a problem when used with the ORBIT360 rotators, which are pretty tolerant of Stalls,  but   please don't connect it to anything expensive and walk away until you are sure that everything is running correctly and reliably.

- If the computer hangs while the rotators are moving,  or the user exits to DOS mode, the antennas  will  continue to execute their last  command until the hardware is reset. (The program’s Stall detection mechanism offers no protection against this type of failure.)  Beware!  Please don't run this unattended until you are 100%  certain that it's working reliably in your environment.  External limit switches are always a good idea.

KIT CONTENTS:  (This part is subject to change)

PRRIETRK.doc       This documentation (MS/Word Source)

PRRIETRK.exe       The executable

PRRIETRK.cfg		Configuration file (User Modifiable text)

startcfg.txt       Sample Automatic start file

IPORT.vbx	      \

\

THREED.vbx           --	Visual Basic Support files

GAUGE.vbx          /

*VBRUN300.dll     /

* VBRUN300.DLL is required,  but available

separately to cut down on size of this zip.