10 January 2002.

The DELTA IM Module for SRE2001

Abstract:

SRE2002's DELTA "instance manipulation" module provides support for

delta encoding.

--------------------------------------------

Contents:

1) Introduction

2) Installing and Configuring the DELTA IM module

3) Technical Notes

--------------------------------------------

1) Introduction

One of the most significant advances of the http/1.1 standard is

increased support for caching.  Caching is the act of using a

a locally available version of a resource instead of re-obtaining

the resource from its originating server.  To the extent that caching

can be enhanced, overall internet traffic will be reduced, with concomittant

increase in delivery speeds for what remains.

Unfortunately, the improved (but still relatively simple) caching schemes

supported by http/1.1 are not well suited to dynamic content.  Since a large

(and probably growing) share of web resources are dynamic (that is, they

change from day to day), this weakness may seriouslly undermine the

potential advantages of caching.

One strategy of dealing with this problem is through the use of "deltas".

Current cache schemes require a server to instruct a client to either use a

cached item as is, or not at all.  Delta caching schemes represent a

compromise -- the server can tell a client to use its cached version

as a "base", and send a list of "differences". This list of differences,

which we also refer to as "deltas", may often be much smaller then

the full contents of the resource.

Technically speaking ...

If the server and client can ascertain that both have the same copy of

a prior instance of a request-URI, then the server can compute

the difference between this "commonly owned" prior instance and

the current instance.  One can think of this difference as being

an "encoding" of the current instance, in much the same way that

GZIP compression is an encoding.  Upon reciept of this "difference",

the client can create a duplicate of the new version by combining

the "difference" with his "prior version" of the instance.

An http standard that  specifies the use of deltas is now in draft

stage (see RFC3229.TXT).SRE2002's DELTA IM Module supports almost all

of this standard.

--------------------------------------------

2) Installing and Configuring the DELTA IM module

The DELTA IM module  (DELTA.RXX) is packaged in DELTA.ZIP. DELTA.ZIP

also which includes several support programs (and this documentation).

The INSTALL.CMD program included in DELTA.ZIP should be used to install DELTA.

Before installing DELTA, you need the following:

1) an HFPS drive for storing old instances (using long filenames).

This should have at least  50M free.

Ideally, this will be the same drive as SRE2002 is installed on.

If not, you'll need to  modify a few parameters in DELTA.RXX

2) The EMX (ver 0.9d) "runtime environment".

EMX ver 0.9d can be found at  hobbes -- try

http://hobbes.nmsu.edu/pub/os2/dev/emx/v0.9d/emxrt.zip

or, you can get it from:

http://www.srehttp.org/pubfiles/emxrt_09d_04.zip

Installing EMX 0.9d is fairly easy:

a) copy the .ZIP file to the root  of your boot drive (say, C:\)

b) Unzip this file -- make sure you use an UNZIP that will automatically

create sub directories.

* An EMX directory (with several subdirectories and about 50 files)

will be created.

* There should be nothing copied to the root directory.

c) Modify the following in  CONFIG.SYS:

i)  Add C:\EMX\DLL to your LIBPATH= line.

ii) Add C:\EMX\BIN to your PATH= line.

(of course, the c:\emx should be changed to reflect where

you installed EMX).

d) Reboot your computer.

EMXREV is a short program (that comes with EMX) that will report on

what version of  EMX you've installed -- you can run it (after

rebooting) to check that the installation was successful.

For example:

c:>emxrev

should return something like:

EMX : revision = 61

EMXIO : revision = 60

EMXLIBC : revision = 63

EMXLIBCM : revision = 64

EMXLIBCS : revision = 64

EMXWRAP : revision = 60

Given these prerequisites, you can install DELTA:

1) Unzip DELTA.ZIP to an empty temporary directory

2) If SRE2002 is running, shut it down.

3) From an OS/2 command prompt, go to this empty temporary

directory and run INSTALL.

4) You will be asked to provide the name of the SRE2002

"working" directory (where SRE2002 is installed).

You will also be asked if you want DELTA to be the default IM.

5) The installation program will create a few subdirectories,

copy a few files, and will modify the SRE2002.CFG configuration file.

6) Restart SRE2002 -- DELTA is now installed!

Notes:

* DELTA uses several GNU programs -- see the COPYING.GNU file in

x:\sre2002\BIN\IM\DELTA for the use license (where x:\sre2002

is where you've installed SRE2002).

You can get the full version of these programs at:

http://archiv.leo.org/pub/comp/os/os2/leo/gnu/systools/

(look for the GNUED and GNUDIFF entries).

* DELTA also uses GDIFF.DLL. For further details on GDIFF, see

http://www.srehttp.org/apps/gdiff

* DoGET.CMD, that comes with the DELTA module, has a "delta encoding"

option enabled. This instructs DOGET to include the necessary headers

(such as A-IM and If-None-Match) in all requests. More importantly,

DOGET will store responses (instances) to a "deltacache" subdirectory.

These cached instances will then be used to automatically undo instance

manipulations, and thereby regenerate the current "undifferenced" instance.

Note that DOGET's "instance manipulation" awareness is good for any rfc3229

compliant server, not just SRE2002.

* CLEANUP.CMD is installed to your x:\sre2002\BIN\IM\DELTA subdirectory.

This is a standalone utility that will cleanup the delta-cache directory,

removing least-recently used files.

To run CLEANUP.CMD:

i) From an OS/2 prompt, CD to x:\sre2002\BIN\IM\DELTA, where

x:\sre2002 is the SRE2002 directory.

ii) run CLEANUP.CMD

There are no run-time parameters to set. However, there are a few parameters

that you can modify in the user-configurable parameters section of

CLEANUP.CMD.

--------------------------------------------

3) Technical Notes

i) The delta-encoding standard: RFC3229

ii) Description of IM Modules: IM_USE.HTM

iii) The 226 IM Used response code

iv) GDIFF and DIFF-E are the currently supported differencing algorithims

v) Partial support for multiple range requests

vi) Cases where DELTA will not be attempted

vii) A-IM conditions that are ignored

viii) Using RETAIN to specify how long to retain an instance

ix) Delta not used if it's longer then the original

x) Costs to using delta, and when it may not be worth it

xi) Dynamic resources and creating/storing seperate instances for all requests

xii) Issues in specifying etags for dynamic resources that change infrequently

xiii) Using LIFESPAN, MAX_BYTES, and MAX_MIN_FREE to specify retention time

xiv) Delta not attempted if no etag specified

xv) What to do if the base and current content-codings differ

xvi) The delta storage directory

xvii) Structure of instance files stored in the delta storage directory

----------------------------------------------

i) RFC3329.TXT (which is installed to x:\sre2002\bin\im\delta)

is the Delta-Encoding draft standard. Of course we recommend reading it!

II) IM_USE.HTM (in x:\sre2002\docs) contains more details on installing

IM Modules. IM_USE.HTM also discusses the concept of "instance

manipulation", a concept that is key to the operation of delta encoding.

iii) Note that if delta-encoding is done, then the response code is:

HTTP/1.1 226 IM Used

In addition, IM and Delta-Base headers are added; and a retain and im

tokens may be added to a Cache-Control header.

iv) Currently, the DELTA IM module supports the GDIFF and DIFF-E

"delta encoding types".

DIFF software is readily available, and free; a free OS/2 version of

GDIFF can be found at http://www.srehttp.org/apps/gdiff.  DIFF

does NOT work with binary files, but tends to be better then GDIFF

for non-binary (text) files.

The delta encoding spec mentions VCDIFF. If OS/2 implementations

of these (or other common) differencing algorithims become available,

we will be happy to add support (code donations are gratefully accepted!)

v)  At this writing, delta is not fully supported for multiple/byte ranges.

More precisely, multiple differences (over multiple byte ranges)

are not attempted; but multiple byte ranges over a single difference

is supported.

Technically speaking, this is NOT supported  (where xdiff is either

DIFFE or GDIFF):

Range: bytes=1000-2000,3000-4000

A-IM: range,xdiff

but this IS supported

Range: bytes=1000-2000,3000-4000

A-IM: xdiff,range

vi)  There are several cases where DELTA will not be attempted, even

though an A-IM and If-None-Match request headers are provided.

1) If A-IM does not contain a delta-token

(the spec insists on this)

2) In A-IM, if a GZIP appears before a delta-token

(it's fruitless to apply differencing to gzip'ped files)

3) If the If-None-Match does not list a currently available

instance.  As a subset of this, if If-None-Match is empty,

DELTA is not attempted.

4) there is no   no Host: request hedaer (or a host is not

specified in the request-line)

Note that the "delta tokens" are GDIFF and DIFFE (case insensitive)

vii)  Certain semi-pathological A-IM  conditions are "liberally" ignored:

1) If A-IM contains multiple delta-tokens, they must be adjacent.

If not (if one or more non-delta token sit between delta-tokens),

the later delta-tokens are ignored.

2) Only one RANGE token can be in A-IM. Later RANGE tokens are ignored.

3) Only one GZIP token can be in A-IM. Later GZIP tokens are ignored

viii)  You can specify how long SRE2002 should retain this instance

(for use in future delta request) by using one of:

> Retain for nseconds seconds

astat=SRE_COMMAND('IMFINFO ','Retain: nseconds')

> Do NOT retain this instance

astat=SRE_COMMAND('IMFINFO ','Retain: 0')

> Retain this instance indefinitely (given the LIFESPAN and other

parameters).

astat=SRE_COMMAND('IMFINFO ','Retain: ')

This is the default.

ix) If the best delta is longer then the current instance, a

delta response will NOT be sent.

x)  Delta encoding is a grand idea, but it comes at some cost to your

server -- the time required to compute differences, and the

space (and time) required to store "prior instances".

In many cases, it's not worth it -- such as for resource that are

rarely re-requested, for resources that rarely change, or for small

resources that contain many possible changes.

Thus, on many sites it may be sensible to NOT choose DELTA to be

your "default IM". If this is so, then you'll need some other mean s

of enabling delta encoding.

Basically, this is done by using the IM option of SRE_COMMAND('FILE ...')

and SRE_COMMAND('VAR ...').

For example:

astat=sre_command('FILE type text/plain im delta name d:\www\plan9.txt')

See SRE2PRC.HTM for the details.

Note that the SREhttp/2 "filter" will offer configuration options that

allow you to easily enable/disable DELTA on a request specific basis.

xi)  Keep in mind that for dynamic resources (say, a web page that

contains a hit counter, or a clock), every single request

results in a different instance, so that each request results in

the server retaining a copy of the response.

Future versions of the DELTA IM module will support DCLUSTER and

DTEMPLATE extensions to the delta-encoding standard.  This

extension (that is still being written) allows a server to specify

instances that will be used for a variety of request-URIS, thereby

simplifying the task of maintaining a useful cache of base instances.

xii)  Some dynamic resources are generated on the fly, but have contents

that change infrequently. For example, HTML documents with server-side

includes may be rebuilt on every request, but may have characters

that change infrequently (say, the only server side include is a date

field).

For such resources, the default SRE2002 "auto-etag" option will

create a new etag for each request. This is unfortunate, since the

server must now store seperate instances for each request, even though

the contents of these instances are identical.

To get around this problem, you should:

i) use the ETAG_AUTO2 option,in SRE_COMMAND('FILE and SRE_COMMAND('VAR.

ii) Set the ENTITY_HEADERS_IGNORE to include LAST-MODIFIED and EXPIRES

Note that sreLite2, using the default values of ENTITY_HEADERS_IGNORE,

uses these two options.

xiii) The LIFESPAN, MAX_BYTES, and MAX_MIN_FREE parameters (in

DELTA.RXX) control the retention (and deletion) of old-instances.

Basically, a LRU algorithim is used to remove instances that

have not been used recently; with this removal dictated by

age and by how full your disk is.

xiv) Delta encoding will NOT be attempted if the response does not include

an ETAG response header (as provided by the filter), or if there is

Note that all of the SRE2002 filters (simple, sreLite2, and

SREhttp/2) include ETAGS in all "normal" requests (requests for

static files).

xv)  If an otherwise useable base-instance has a different content-encoding

then the current instance, it will not be used.

There is one exception to this rule:

> if the only difference, between a base instance content-encoding

and the current instance's content-encoding, is a trailing GZIP

in the base instance,

> then the base-instance will be unGZIPped,

> and this unGZIPped variant will be used as the base-instance when

performing instance-manipulations

For example:

Current instance Content-Encoding: x,y,z

Base instance Content-Encoding: x,y,z,GZIP

then use an unGZIPped version of the base instance.

xvi)  DELTA uses a "delta_dir" to store old instances. By default, this will

be the x:\sre2002\TEMP\DELTA directory (where x:\sre2002 is where

you installed SRE2002).

At any time you can delete the contents of thie directory with no

deleterious effect -- other then suppressing delta encoding against

these instances. Note that this does not effect the storage (and use)

of future instances.

xvii) Files in the "delta_dir" have the following structure:

a1_a2.b.c

where

and

c : a hash of the servername

b : a hash of the request string

a2 : _a2 is used only part of the time. It is a hash of the contents

of the instance

a1 : a hash combining entity headers and date & size information

The hashs are derived from crc of the respecttive items, and consist

of between 6 and 9 upper case alphanumerics.